3GPP TSG-RAN WG3 Meeting #115-e	R3-22xxxx

21 February – 3 March 2022

Agenda item:	11.3

Source: 	Ericsson, Qualcomm Incorporated, Huawei

Title: 		(TP for NGAP BL CR on RedCap) Proposed updates to the NGAP BL CR

Document for:	Other

# 1. Introduction

This contribution provides proposals for minor semi-editorial changes in the NGAP baseline CR for RedCap [1] mirroring some of the changes proposed in [2].

# 2. Change proposals

There is an editor’s note in section 9.3.1.xx of the NG-AP BL CR “Editor’s Note: confirm the encoding of NR Paging Time Window IE.”

As explained in [2], we need to rename the variable as TeDRX, CN for better clarity, as it aligns with TS 38.304 terminology.

Also, there are apparent issues in the encoding of the eDRX paging cycles and the PTW for NR in the NG-AP 9.3.1.xx IE compared to how they are specified in TS 24.008 (clause 10.5.5.32 – copied in the Appendix below). Specifically:

- There are no eDRX cycle values differing from a power of 2,

- There are no PTW values larger than (16 x 1.28s).

Therefore, it is needed to align NG-AP with TS 24.008 since the UE cannot be configured with other values.

Proposal 1: Delete editor’s note in section 9.3.1.xx of the NG-AP BL CR “Editor’s Note: confirm the encoding of NR Paging Time Window IE.”

Proposal 2: Align the NR Paging eDRX Information IE with the codepoints specified in TS 24.008

We propose to address these misalignments in the TP to the NG-AP BL CR proposed in the annex below.

# 4. References

[1]		R3-221529, Support for Redcap UEs, NG-AP BL CR, Nokia et al., RAN3#115-e, Feb-March 2022

[2]		R3-22xxxx, (TP for XnAP BL CR on RedCap) Proposed updates to the XnAP BL CR, Qualcomm Incorporated et al, RAN3#115-e, Feb-March 2022.

# 5. Appendix

Extract from TS 24.008:

WB-S1 mode WB-N1 mode and NR connected to 5GCN The field contains the PTW value in seconds for WB-S1 mode, WB-N1 mode and NR connected to 5GCN. The PTW value is used as specified in 3GPP TS 23.682 [133a] and 3GPP TS 23.501 [166]. The PTW value is derived as follows: bit. 8; 7; 6; 5; Paging Time Window length. 0; 0; 0; 0; 1,28 seconds. 0; 0; 0; 1; 2,56 seconds. 0; 0; 1; 0; 3,84 seconds. 0; 0; 1; 1; 5,12 seconds. 0; 1; 0; 0; 6,4 seconds. 0; 1; 0; 1; 7,68 seconds. 0; 1; 1; 0; 8,96 seconds. 0; 1; 1; 1; 10,24 seconds. 1; 0; 0; 0; 11,52 seconds. 1; 0; 0; 1; 12,8 seconds. 1; 0; 1; 0; 14,08 seconds. 1; 0; 1; 1; 15,36 seconds. 1; 1; 0; 0; 16,64 seconds. 1; 1; 0; 1; 17,92 seconds. 1; 1; 1; 0; 19,20 seconds. 1; 1; 1; 1; 20,48 seconds. […] NR connected to 5GCN The field contains the eDRX value for NR connected to 5GCN. The eDRX cycle length duration value and the eDRX cycle parameter 'TeDRX' as defined in 3GPP TS 38.304 [183] are derived from the eDRX value as follows:. bit. 4; 3; 2; 1; eDRX cycle length duration; eDRX cycle parameter 'TeDRX'. 0; 0; 0; 0; 2,56 seconds; NOTE x. 0; 0; 0; 1; 5,12 seconds; NOTE x. 0; 0; 1; 0; 10,24 seconds; 20. 0; 0; 1; 1; 20,48 seconds; 21. 0; 1; 0; 0; 40,96 seconds; 22. 0; 1; 0; 1; 81,92 seconds; 23. 0; 1; 1; 0; 163,84 seconds; 24. 0; 1; 1; 1; 327,68 seconds; 25. 1; 0; 0; 0; 655,36 seconds; 26. 1; 0; 0; 1; 1310,72 seconds; 27. 1; 0; 1; 0; 2621,44 seconds; 28. 1; 0; 1; 1; 5242,88 seconds; 29. 1; 1; 0; 0; 10485,76 seconds; 210.

# 6. Text Proposal

>>>> START OF CHANGES <<<<

#### 9.3.1.xx	NR  Paging eDRX Information

This IE indicates the NR Paging eDRX parameters as defined in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX, CN defined in TS 38.304 [12]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …) | PTW defined in TS 38.304 [12]. Unit: [1.28 seconds] |

Next change

NR-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsingNB)) OF NR-CGI

NR-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF NR-CGI

NR-PagingeDRXInformation ::= SEQUENCE {

nR-paging-eDRX-Cycle		NR-Paging-eDRX-Cycle,

nR-paging-Time-Window		NR-Paging-Time-Window					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NR-PagingeDRXInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Paging-eDRX-Cycle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4,

hf8, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NR-Paging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...

}

>>>> END OF CHANGES <<<<