3GPP TSG-RAN WG3 Meeting #115-e	R3-22xxxx

# Online, 21th January – 3rd March 2022

Agenda Item:	22.2.6

Source:	Ericsson

Title:	Discussion on MBS service areas

Document for:	Discussions & Approval

# 1	Introduction

The chairman notes [1] from RAN3#114bis-e contains the agreement and working assumption below.

- Discuss whether MBS service area identity (SAI) can be used as MBS service area information.

- WA: A list of IDs (e.g. SAI) may be used for identifying MBS service area of a broadcast session, pending to RAN2 agreement.

Further, the baseline CR for TS38.413 contains an FFS for the NR CGI in section 9.3.1.ccc.

#### 9.3.1.ccc	MBS Service Area information

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI [FFS] | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is FFS. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is FFS. |

Figure 1: MBS Service Area information TS38.413 TP

In this document we discuss the [FFS] with the NR CGI and in particular the use case when it is applicable. We also discuss explain when the NR CGI is difficult to use and propose an alternative solution for this case.

# 2	Discussion

The NR CGI is used to globally identify an NR cell and contains a PLMN and a NR Cell Identity. The NR Cell Identity is 36 bits long where the leftmost bits correspond to the gNB ID and has a variable length from 22 to 32. From previous discussions the purpose of the NR CGI list in the scope of 5G-MBS is to provide an alternative when TAI is not sufficiently granular nor flexible. The problem with exposing the NR-CGI to the core network is that the identifier is designed for other use hence within the scope of 5G-MBS we try to use it for additional functionality it was not originally intended for. This introduces dependencies which impact is difficult to foresee and costly to manage. In network supporting energy saving and cell shaping mechanisms it is not unlikely that cells are only present sometimes, and possibly in certain combinations, making it impossible for the core network to have a sufficiently accurate picture of the RAN cell configuration. As a solution we propose introduce a new identifier decoupled from the cell. Our understanding based on the feedback we have received during the SA2 discussions our understanding is that the discussion can be seen for two different types of use cases.

- The core network has configured information about the NG-RAN.

- The core network initiates a session based on received measurement reports via an application layer.

Observation: The appropriate content of the MBS Service Area information depends on the use case.

## Use case 1

Use case description (Location dependent MBS service): The core network provides different session content in different geographical areas and the granularity based on TAIs is not sufficient.

From TS23.247 [1]: MBS service area: The area within which data of one Multicast or Broadcast MBS session may be sent. For location dependent MBS, for each MBS service area, an Area Session ID, which is unique per MBS Session ID, is allocated and the same location dependent content data for an MBS session is delivered to the UE(s) within an MBS service area.

The receiving NG-RAN node compares the MBS Service Area Information with the cell configuration and selects the matching cells. It then starts the service in the matching cells to the extent that it compliant to the existing cell configuration in the node. For example, if the node has two cell configurations where one is a three-sector configuration for capacity and an omni cell configuration for energy saving it becomes unclear how the receiver shall interpret and behave with the received information. It is however sufficient for the core network to understand in which geographical area to provide the service. In addition to identify the cells with their TAI it is  in addition useful to define a new area concept (MBSSAI). Each cell is then configured (or autonomously selected by NG-RAN) to belong to zero or more SAIs. The SAI is added in the MBS Service Area information according to Figure 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI [FFS] | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| Supported MBS SAI List |  | 0..<maxnoofMBSSAIs> |  | FFS: PLMN / NID dependancy of MBS SAI. also whether the Broadcast PLMN Identity Info List needs that input. |
| >MBS Service Area Identity | M |  | OCTET STRING(2) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is FFS. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is FFS. |
| maxnoofSAIforMBS | Maximum no. of SAI allowed within one MBS Service Area . Value is FFS. |

Figure 2: Addition of MBS Service Area Identity to the MBS Service Area information.

Proposal 1: Add the MBSSAI to the MBS Service Area Information according to Annex A.

Figure 3 shows a simple network configuration with just a few nodes. Each DU contains a set of SAIs marked in blue which maps to a geographical radio coverage area. To provide a session in the complete network in the example the core network would start the session in the SAIs {A,D,E} while a session located to only DU3 and part of DU 4 could be started by addressing SAI {C,D}. The mechanism provides a large flexibility and is independent of the constraints also set by the TAI since a describes the connectivity to the core network (AMFs and UPFs).

Figure 3: SAI configuration example

When a SAI based configuration is used in the network this information is provided from the DU to the CU-CP. When the CU-CP receives Broadcast Session Setup Request message it compares the MBS Service Area information to its known cells and then starts the service in those cells. Continuing with the sector- and omni switch example above we can think of the sector configuration to consist of three cells and the omni configuration of one cell. Here, all four cells would belong to the same SAI and the NG-RAN can switch between two configurations without any need to inform the core network which keeps the complexity local. The switch between cell configuration can be frequent while the SAI configuration is not.

Observation: By introducing a new service area concept instead of the MBS Service Area Cell list the complexity for the core network to have an up-to-date model of the cell states in RAN is significantly reduced.

Proposal 2: The 3GPP standard does not support 5G-MBS use cases where the core network needs to store cell configuration information.

Proposal 3: RAN3/3GPP to add a new area concept (MBSSAI).

If the SAI is added to the NG-Setup message and NG-Configuration update messages a change of RAN configuration, e.g. adding a cell to a new MBSSAI, could trigger the core network to provide any information necessary for this new MBSSAI.

Proposal 4: Add the MBSSAI to the NG Setup and NG RAN Configuration update procedures.

Depending on the relation between the SAI discussed here and the SAI defined by RAN2 additions may be needed also in TS38.473. However, the MBS SAI List IE is already included in the F1 SETUP REQUEST message in the base line TS38.473 and it is FFS if this IE can be re-used or if a new list of SAIs using another definition is needed.

## Use case 2

Use case description: The UE reports cell measurements for cells where it needs broadcast to be activated to an application server in the core network (not visible to RAN3). The core network request broadcast start on cell basis using this information.

In this scenario the assumption is that the UE can indicate to the application server when it is on a cell where it wants to receive the broadcast MBS session if not broadcast. The problem with coordination of cells and cell states between the core network and NG-RAN is eliminated since only measured cells are requested. If the cell would disappear (cell configuration change) before the service is started, the UE enters a new cell where the broadcast is already enabled or it could request the service once again.

Proposal 5: MBS service activation requests where the area is indicated using cell identifiers is limited to cases when up to date cell information in NG-RAN is provided using UE measurements.

A remaining issue for use case 2 is that the flexible gNB ID length implies that the gNB Identifier cannot be derived from the UE cell measurement. It is therefore not possible for the application server to understand to which gNB the cell belongs which causes signalling to incorrect nodes. This topic is ongoing under agenda item 32 and we refer the discussion there where also this MBS use case needs to be taken into account.

Observation: To support use case 2 the application server needs to be able to decode the gNB ID from the measured cell identifier which is not possible without it knowing the gNB ID length which is currently not available to the UE.

Proposal 6: Discuss the problem with variable gNB ID length for MBS discussions about the flexible gNB ID length under agenda item 32.

# 3	Conclusion and Proposals

Proposal 1: Add the MBSSAI to the MBS Service Area Information according to Annex A.

Proposal 2: The 3GPP standard does not support 5G-MBS use cases where the core network needs to store cell specific configuration information.

Proposal 3: RAN3 to add a new area concept (SAI) if the flexibility and or granularity of the TAI is not sufficient since cell configuration in the core network is not supported.

If the SAI is added to the NG-Setup message and NG-Configuration update messages a change of RAN configuration, e.g. adding a cell to a new SAI, could trigger the core network to provide any information necessary for this new SAI.

Proposal 4: Add the SAI to the NG Setup and NG RAN Configuration update procedures according to Annex A.

Proposal 5: Activation of MBS services based on cell identifiers is limited to the case when up to date information is provided by UE measurements.

Proposal 6: Consider the issue with variable gNB ID length for MBS discussions about the flexible gNB ID length under agenda item 32 and tdoc [2].

Proposal 7: Send LS to SA2 according to Annex B informing about the addition of MBSSAI as a new MBS Service Area.

# 4	References

[1]	TS23.247v17.1.0; Architectural enhancements for 5G multicast-broadcast services; Stage 2

[2]	R3-22xxxx; Ericsson, Bell Mobility, Telus, Verizon Wireless, China Telecom; Discussion and way forward on Flexible gNB-ID solutions

# Annex A - Text Proposal for BL CR against TS 38.413

This is a text proposal against the latest BL CR for TS 38.413, changes are highlighted in cyan.

<<<<<<<<<<<<<<<<<<<< Begin of changes for BL CR for TS 38.413 >>>>>>>>>>>>>>>>>>>>>

### 9.2.6	Interface Management Messages

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Supported MBS SAI List |  | 0..<maxnoofMBSSAIs> |  |  | YES | ignore |
| >MBS Service Area Identity | M |  | OCTET STRING(2) | FFS: PLMN / NID dependancy of MBS SAI. also whether the Broadcast PLMN Identity Info List needs that input. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofSAIs | Maximum no. of MBSSAIs. Value is FFS. |

--------------------------------Start of the Next Change-----------------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Supported MBS SAI List |  | 0..<maxnoofMBSSAIs> |  |  | YES | ignore |
| >MBS Service Area Identity | M |  | OCTET STRING(2) | FFS: PLMN / NID dependancy of MBS SAI. also whether the Broadcast PLMN Identity Info List needs that input. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |
| maxnoofSAIs | Maximum no. of MBSSAIs. Value is FFS. |

--------------------------------Start of the Next Change-----------------------------

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI [FFS] | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| Supported MBS SAI List |  | 0..<maxnoofMBSSAIs> |  | FFS: PLMN / NID dependancy of MBS SAI. also whether the Broadcast PLMN Identity Info List needs that input. |
| >MBS Service Area Identity | M |  | OCTET STRING(2) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is FFS. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is FFS. |
| maxnoofSAIforMBS | Maximum no. of SAI allowed within one MBS Service Area . Value is FFS. |

# Annex B – LS out proposal

3GPP TSG-RAN WG3 Meeting #115-e	R3-22xxxx

# Online, 21st February 3rd March 2022

Title:	[DRAFT] LS on new MBS Service Area

Response to:

Release:	REL-17

Work Item:	NR_MBS-Core

Source:	Ericsson (will be RAN3)

To:	SA2

Cc:

Contact Person:	Alexander Vesely

Tel. Number:

E-mail Address:	alexander dot vesely at ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 has in release-17 introduced a new identity to describe an MBS service area. The new MBS Service Area Identity defines a geographical area where the NG-RAN provides the MBS Session. The NG application protocol is designed to receive an optional list of MBS Service Area Identities.

2. Actions:

To: SA2 group.

ACTION: 	RAN3 asks SA2 group to update their specifications with the new MBS Service Area Identity.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting#116-e	16th - 27th May 2022

TSG-RAN WG3 Meeting#117	22nd - 26th August 2022