3GPP TSG-RAN WG3 #116-e									R3-223038

9th – 19th May 2022 Online

Agenda Item:	9.1.12.1

Source:	China Telecom, ZTE, CATT,Huawei

Title: 	Correction of UDC in CP-UP Split architecture

Document for:	Discussion

Introduction

In RAN3#115e meeting, UDC functionality was agreed to be supported in both NR CP/UP split scenario and LTE CP/UP split scenario. And corresponding CRs were approved. In this contribution, we discuss some issues on UDC in CP-UP Split architecture.

Discussion

In baseline CR R3-222613, the CU-CP could configure UDC for one DRB via Bearer Context Setup procedure. And the following procedure texts were captured in TS38.463:

If the UDC parameters IE is included in the PDCP Configuration IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take these parameters into account to perform appropriate uplink data compression for the concerned DRB.

Since the PDU Session Resource To Setup List IE or DRB To Setup Modification List E-UTRAN IE in Bearer Context Modification Request message can be used to setup new DRB(s), thus we propose to add the following texts in Bearer Context Modification Request message.

If the UDC parameters IE is included in the PDCP Configuration IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take these parameters into account to perform appropriate uplink data compression for the concerned DRB.

Proposal1: The UDC configuration for DRB(s) can be configured via Bearer Context Modification procedure. And the corresponding texts for the CU-UP behaviour shall be added in in Bearer Context Modification Request message.

The corresponding CR can be seen in [3].

Proposal 2: to approve the corresponding CR in [3].

Proposal

Based on the above analysis, we have the following proposals:

Proposal1: The UDC configuration for DRB(s) can be configured via Bearer Context Modification procedure. And the corresponding texts for the CU-UP behaviour shall be added in in Bearer Context Modification Request message.

Proposal 2: to approve the corresponding CR in [3].

Reference

- TS38.331 v16.7.0

- TS38.463 v16.9.0

- R3-22xxxx, Correction of UDC in CP-UP Split architecture