3GPP TSG-RAN WG3 Meeting #116-e	R3-223070

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0201 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RACH based SDT and CG based SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom, China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Issue 1: small errors in RACH based SDT Section 8.18.1 Proposal 1: Section 8.18.1, Step 3, editorial udpate by removing the “with” before “including”. Note 2: Clarify which gNB-CU-CP is the “other gNB-CU-CP” in Note2. Issue 2: section 8.20.2, The UE’s state in step 0 is incompleted. - Issue description: It is possible that UE is in RRC_INACTIVE mode rather than in RRC_CONNECTED when gNB-CU-CP decides to release UE with CG-SDT configurations, e.g., at the end of an RA-SDT procedure. Hence in Figure 8.20.2-1, the step 0 “UE in RRC_CONNECTED mode” should be removed or updated to “UE in RRC_CONNECTED or RRC_INACTIVE mode”. Correspondingly in step 1, the “change” should be updated to “move” since UE may remain in RRC_INACTIVE and its mode is not changed. proposal 2: Update step 0 in Figure 8.20.2-1 to “UE in RRC_CONNECTED or RRC_INACTIVE mode”, and update the “change” in step 1 to “move”. Issue 3: Whether gNB-DU buffers the UL SDT data and/or UL SDT signalling in CG-SDT. - Issue description: Despite of having the F1 tunnel information in CG-SDT, the gNB-DU should still buffer the UL SDT data, not sending it to the gNB-CU-UP, until gNB-CU-CP verifies UE successfully. First, the data transmission over F1-U is always performed after the successful UE verification in current systems, and it is better to follow the legacy way. Besides, it is safer and more reliable since we can avoid delivering the data of unverified UEs in the network. Similarly, UL SDT signalling should also be buffered at gNB-DU rather than be sent to gNB-CU-CP directly. When it is gNB-DU to buffer the UL SDT data and/or UL SDT signalling, gNB-CU-CP should send the UE CONTEXT MODIFICATION REQUEST message to gNB-DU. Then, gNB-DU can send the buffered UL SDT data and/or UL SDT signalling to gNB-CU-UP and gNB-CU-CP, respectively. These steps should be captured in Figure 8.20.2-1. proposa 3: gNB-DU should buffer the UL SDT data and/or UL SDT signalling until receiving the UE CONTEXT MODIFICATION REQUEST message from gNB-CU-CP. These steps should be added in Figure8.20.2-1. Isseu 4: need to move 8.20.2 to be a subsection of 8.18 - Issue description: The section 8.18.1 RACH based SDT and section 8.20.2 CG based SDT should be introduced as two subsections of the same section, but currently they are located in wrong places. Proposal 4: change 8.20.2 to 8.18.2, and void 8.20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 8.18.1, Step 3, editorial udpate by removing the “with” before “including”. Note 2: Clarify which gNB-CU-CP is the “other gNB-CU-CP” in Note2. - Update step 0 and step 1 in Figure 8.20.2-1. - Add the following steps to Figure 8.20.2-1: - The gNB-DU buffers the UL SDT data and/or UL SDT signalling. - The gNB-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU. - The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU-CP. - The gNB-DU sends the UL SDT data, if any, to the gNB-CU-UP, and sends the UL signalling, if any, to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF. - Change 8.20.2 to 8.18.2, and void 8.20 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The possibility that UE may in RRC_INACTIVE mode when gNB-CU-CP decides to release UE and configure CG-SDT resources is excluded. - It is not clear which node to buffer the UL SDT data and/or UL SDT signalling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18, 8.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

## 8.18	Overall procedure for Small Data Transmission during RRC Inactive

### 8.18.1	RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.18.1-1.

Figure 8.18.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL SDT data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1:	In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP first establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retreived before step 4. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT DRBs.

NOTE 2:	In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. In addition, the UL SDT data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP for which the partial context is retrieved via the XnAP RRC TRANSFER message.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT DRBs. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-CP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.

-------------------------------------------Change 2-------------------------------------------

## 8.20	void

-------------------------------------------Change 3-------------------------------------------

### 8.18.2	CG based SDT

The procedure for CG based small data transmission in RRC Inactive is shown in Figure 8.18.2-1.

Figure 8.18.2-1: CG based Small Data Transmission in RRC Inactive state.

1.	The gNB-CU decides to move UE into RRC_INACTIVE state.

2.	The gNB-CU-CP decides to configure CG-SDT, it sends UE CONTEXT MODIFICATION REQUEST message including a query indication for CG-SDT related resource configuration associated with the information of SDT Radio Bearer(s).

3.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message including the CG-SDT related resource configurations for the requested SDT Radio Bearer(s) within the DU to CU RRC Information IE.

4.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST towards the gNB-CU-UP, with the suspend indication.

5.	The gNB-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE towards the gNB-CU-CP.

6.	The gNB-CU-CP sends the UE CONTEXT RELEASE COMMAND message to the gNB-DU including an RRCRelease message to the UE with the CG-SDT information within suspend configuration. The gNB-CU notifies the gNB-DU to keep the SDT RLC config, F1-U tunnels, F1AP UE association, and store the CG resource for SDT when the UE is entering RRC_INACTIVE with an explicit CG-SDT kept indicator.

7.	The gNB-DU sends RRCRelease message to UE.

8.	The gNB-DU sends UE CONTEXT RELEASE COMPLETE message. The gNB-DU keeps the SDT RLC config, F1-U tunnels, F1AP UE association, and stores the CG resource for SDT when the UE entering RRC_INACTIVE. The gNB-DU also stores the C-RNTI, CS-RNTI, and which bearers are CG-SDT bearers.

After a period of time of the UE being in RRC_INACTIVE mode

9.	The UE decides to perform CG based SDT procedure, it sends the RRCResumeRequest message together with UL SDT data/UL NAS PDU.

10. The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message including the RRCResumeRequest message.

12/13.	The gNB-CU-CP initiates the BEARER CONTEXT MODIFICATION procedure to resume SDT DRBs.

14. The gNB-CU-CP sends the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU.

15. The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU-CP.

16/16a. The gNB-DU sends the UL SDT data, if any, to the gNB-CU-UP, and/or sends the UL signalling, if any, to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

-------------------------------------------End of the Changes-------------------------------------------