3GPP TSG-RAN3 Meeting #116	R3- 223220

# E-meeting, 9 May – 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0208 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.401 on inter-donor redundancy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In procedure for inter-donor redundancy of the boundary node, IP addresses are requested/responded during IAB transport migration management procedure. The problem is that the non-F1-terminating CU cannot determine whether the IP address request is initiated for boundary node or descendant nodes, thus extra spec impact is foreseen. - In RAN3#115e meeting, it is agreed to introduce a new Class 1 non-F1-terminating CU initiated procedure to support traffic release or modification for inter-CU topology redundancy scenario. The new messages associated with the non-F1-terminating CU initiated traffic release and modification have been introduced for XnAP, and there are corresponding texts for non-F1-terminating CU initiated traffic modification procedure. But the texts for non-F1-terminating CU initiated release procedure are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The F1-terminating donor uses XnAP procedures for dual connectivity to request IP addresses for the boundary node in non-F1-terminating topology. - The texts for non-F1-terminating CU initiated traffic release procedure are added for inter-CU topology redundancy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The non-F1-terminating CU can not determine whether the IP address request is initiated for boundary node or descendant nodes. - Release procedure of transport migration which is initiated by non-F1-terminating CU is not supported in inter-CU topology redundancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////////////////Start of change///////////////////////////////////////////////////////////////////

8.17.2		IAB Inter-CU Topology Redundancy

8.17.2.1	IAB Inter-CU topological redundancy procedure

The inter-CU topological redundancy procedure enables the establishment, modification and release of redundant paths in IAB-topologies underneath different IAB-donor-CUs. Since topological redundancy uses NR-DC for the IAB-MT, it is only supported for IAB-nodes operating in the SA mode.

Figure 8.17.2.1-1 shows an example of the inter-CU topological redundancy procedure, where a second path for the F1 connection of the dual-connecting IAB-node with the F1-terminating IAB-donor-CU is established in the topology of the non-F1-terminating IAB-donor-CU. By retaining the F1-connection of the dual-connecting IAB-node’s IAB-DU with the F1-terminating IAB-donor-CU and having a second RRC connection for the dual-connecting IAB-node’s IAB-MT with the non-F1-terminating IAB-donor-CU, this procedure renders the dual-connecting IAB-node as a boundary IAB-node.

Figure 8.17.2.1-1 IAB inter-CU topology redundancy procedure

1. 	The NR-DC establishment procedure is performed for the IAB-MT of the dual-connecting IAB-node as described in TS 37.340 [12], clause 10.2. Or NR-DC modification procedure is performed for the IAB-MT of the dual-connecting IAB-node as described in TS 37.340 [12], clause 10.3. The F1-terminating IAB-donor-CU may include the dual-connecting IAB-node’s TNL address information in the RRC container of the XnAP message for NR-DC to the non-F1-terminating IAB-donor-CU. The non-F1-terminating IAB-donor-CU provides an RRCReconfiguration message which is included as part of the XnAP response message to the F1-terminating node, or the non-F1-terminating IAB-donor-CU deliveres the RRCReconfiguration message to the dual-connecting IAB-node. The RRC configuration includes a BAP address for the dual-connecting IAB-node in the non-F1-terminating IAB-donor-CU’s topology. The RRC configuration may include new TNL address(es) anchored at the second-path IAB-donor-DU for the dual-connecting IAB-node. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the new TNL address is outer IP address.

2. 	The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to provide the context of the traffic to be offloaded.

3. 	The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the dual-connecting IAB-node and the second-path IAB-donor-DU as well as DL mappings on the second-path IAB-donor-DU for the dual-connecting IAB-node’s second path. The DL mappings may be based on the TNL address(es) allocated to the dual-connecting IAB-node in step 3. These configurations may support the transport of UP and non-UP traffic on the second path.

4. 	The non-F1-terminating IAB-donor-CU responds with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the F1-terminating IAB-donor-CU to provide the mapping information for the traffic to be offloaded as indicated in step 2. The message includes the L2 info necessary to configure the dual-connecting IAB-node with the UL mappings for this traffic. The message includes the DSCP/IPv6 Flow Label values to be used for the DL traffic to be offloaded.

NOTE:	The non-F1-terminating IAB-donor-CU should select the same IAB-donor-DU in its topology for all to-be-offloaded traffic, whose UL BH mappings received from the F1-terminating IAB-donor-CU in step 2 share the same BAP address.

5. 	The F1-terminating IAB-donor-CU updates the boundary node with the UL BH information received form the non-F1-terminating IAB-donor-CU in Step 4. This step may also update UL FTEID and DL FTEID associated with individual GTP-tunnel(s). The affected GTP tunnel(s) will be switched to use the dual-connecting IAB-node’s new TNL address(es). This step may use non-UE associated signaling in E1 and/or F1 interface to provide updated UP configuration for F1-U tunnels of multiple connected UEs or child IAB-MTs. Implementation must ensure the avoidance of potential race conditions, i.e., no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

The F1-terminating IAB-donor-CU may also provide UL BH information associated with non-UP traffic. New TNL addresses for F1-C traffic configured in step 1, if any, can be added to the dual-connecting IAB-DU’s F1-C association(s) with the F1-terminating IAB-donor-CU.

6.	The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to modify the context of the dual-connecting IAB-node’s offloaded traffic. The message may include the DL TNL address information used for the offloaded traffic and reported by the boundary node in step 5. The non-F1-terminating IAB-donor-CU may use this information to configure or modify DL mappings on the second-path IAB-donor-DU.

7. The non-F1-terminating IAB-donor-CU responds with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the F1-terminating IAB-donor-CU.

8. Repetition of steps above, as needed, where the F1-terminating IAB-donor-CU can request addition, modification, or release of the offloaded traffic. The non-F1-terminating IAB-donor-CU can fully or partially reject the addition or modification requests by the F1-terminating IAB-donor-CU.

The non-F1-terminating IAB-donor-CU may request the modification of the L2 transport for the offloaded traffic in the non-F1-terminating IAB-donor-CU’s topology using the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message. The F1-terminating IAB-donor-CU reconfigures UL BH mappings accordingly and acknowledges the modification via the IAB TRANSPORT MIGRATION MODIFICATION RESPONSE message. The non-F1-terminating IAB-donor-CU may further reconfigure the TNL addresses of the dual-connecting IAB-node via RRC.

The traffic offload for descendant nodes follows the same procedure as defined for the partial migration in clause 8.17.3.2.

The F1-terminating IAB-donor-CU may request full or partial release of the offloaded traffic from the non-F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Management procedure towards the non-F1-terminating IAB-donor-CU (e.g., for the purpose of revoking or in case UE bearers are released). The non-F1-terminating IAB-donor-CU may request full or partial release of the offloaded traffic by initiating the IAB Transport Migration Modification procedure towards the F1-terminating IAB-donor-CU (e.g., for the purpose of revoking).

The traffic offloaded through the inter-CU topological redundancy procedure described in steps 1 to 8 (including the offload of traffic pertaining to the dual-connecting boundary IAB-node and its descendant IAB-nodes and their served UEs) can be partially or fully revoked, resulting in the return of the offloaded traffic back to the F1-terminating IAB-donor-CU’s topology. Full or partial traffic revoking can be initiated by the F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Management procedure towards the non-F1-terminating IAB-donor-CU. The non-F1-terminating IAB-donor-CU can request partial or full traffic revoking from the F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Modification procedure towards the F1-terminating IAB-donor-CU.

//////////////////////////////////////////////////////////////////////////End of change///////////////////////////////////////////////////////////////////