3GPP TSG-RAN WG3 #116-e									R3-223320

9th – 19th May 2022 Online

Agenda Item:	9.3.6

Source:	China Telecom

Title: 	Support of an EN-DC X2 connection between SA gNB and LTE eNB

Document for:	Discussion

Introduction

The PRACH coordination between LTE and NR was discussed in TEI-17. Two possible solutions, namely Solution 1 and Solution 3, were down selected in RAN3#115e meeting. The essence of Solution 1 is to introduce an EN-DC X2 interface for NR standalone site while Solution 3 is to enhance S1/NGAP message. The following progress on this topic were achieved [1]:

WA: Solution 1 is agreed to support resource coordination between LTE and NR SA sites.

Whether there is any spec impact on RAN3 and other WG will be discussed at next meeting.

Standard impact, to be continued...

The intention of this paper is to analyse the standard impact of solution 1 and provide some proposals.

Discussion

RAN3 impact

For Solution 1, an additional EN-DC X2 interface needs to be established between LTE and SA gNB to enable the coordination of resources between co-channel sharing LTE and NR cells. In order to avoid affecting the existing NG-RAN architecture, the NR site shall also support an additional “en-gNB" role, which implies the NR site has to configure both SA and NSA dual protocol stack simultaneously. Since the NSA comprises of a large number of features and all the NSA sites in China have been upgraded to SA-only mode [2], it is not a good idea to reconfigure a full NSA protocol stack in SA gNB to support the coordination of resources between co-channel sharing LTE and NR cells.

The contribution [3] proposed to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation. In our view, that is not enough for Solution 1. In order to maintain an X2 interface between LTE and NR, the following global procedures shall be supported:

- EN-DC X2 Setup

- EN-DC Configuration Update

- EN-DC X2 Removal

- E-UTRA - NR Cell Resource Coordination

Per TS36.423, one NR cell can indicate the following mode to LTE eNB during EN-DC X2 setup procedure:

- NSA and SA dual mode: Configured TAC (2 byte) and 5GS-TAC(3byte)

- NAS mode: Configured TAC (2 byte)

- SA mode: 5GS-TAC(3byte)

Where the configured TAC refers to the TAC configured in the en-gNB, different from the 5GS TAC broadcast in the NR cell and enables application of Roaming and Access Restrictions for EN-DC as specified in TS 37.340. The Configured TAC and 5GS-TAC could not be used to indicate the mode “Only Resource Coordination”. Thus, it is natural to introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE. Upon received this new type indication, the LTE eNB understands the sender node is a special en-gNB which only supports global EN-DC procedures and E-UTRA-NR Cell Resource Coordination.

Proposal 1: to introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE in EN-DC X2 Setup/Configuration Update.

For TS37.340, the “only resource coordination” is also a new functionality for en-gNB. Therefore, the corresponding stage 2 texts on this new functionality is also needed.

Proposal 2: the corresponding stage 2 texts on “Only Resource Coordination” is also needed in TS37.340.

In addition, the existing NCR from a source E-UTRA cell to a target NR cell means that eNB controlling the source cell knows the NCGI and PCI of the target cell. If an NCR from a source E-UTRA cell to a target E-UTRA cell exists, the eNB controlling the source cell has information whether the target E-UTRA cell has an existing NCR to a target NR cell for performing EN-DC. Therefore, it is need to add this new attribute “Only Resource Coordination” in stage 2 description on ANR function in TS36.300. The attributes have the following definitions:

------------------------------------------------------------start change of TS36.300------------------------------------------------------

- 22.3.4a	Automatic Neighbour Relation Function towards NR

-------------------------------------------------------------------skip unchanged-----------------------------------------------------

An X2 link may be set up between eNB and en-gNB. The NoRemove, the NoHO and the NoX2 attributes apply when the en-gNB parents the target cell. Each NCR has the following additional attribute:

-	No EN-DC: If checked, the Neighbour Cell Relation shall not be used by the eNB for EN-DC.

-	Only Resource Coordination Indication: If checked, the neighbour relation shall use an X2 interface only in order to coordinate resources between source and the target cell

Each E-UTRA cell contains an Inter Frequency Search list. This list contains all frequencies that shall be searched.

The PCI is defined by the frequency of the SSB associated with SIB1, and NR-PCI.

--------------------------------------------------------------------end change of TS36.300-------------------------------------------

Proposal 3: to introduce a new attribute” Only Resource Coordination” in ANR function of TS36.300.

SA5 impact

Per TS28.541, the NRCellRelation comprises of the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nRTCI | O | T | T | F | T |
| cellIndividualOffset | M | T | T | F | T |
| isRemoveAllowed | CM | T | T | F | T |
| isHOAllowed | CM | T | T | F | T |
| isESCoveredBy | CM | T | T | F | T |
| isENDCAllowed | CM | T | T | F | T |
| isMLBAllowed | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFreqRelationRef | M | T | T | F | T |
| adjacentNRCellRef | M | T | T | F | T |

Where the attribute “isENDCAllowed” refers to “Multi-Radio Dual Connectivity with the EPC is supported.” Clearly, it is need to introduce a new attribute attributes to indicate” Only Resource Coordination”. SA5 related specification shall be updated to support the new attribute “Only Resource Coordination”.

Proposal 4: it is need to send a LS to SA5 on support of the new attribute “Only Resource Coordination” in EN-DC.

Proposal 5: to approve the corresponding (DRAFT)CRs [4][5][6].

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to introduce a new indication “Only Resource Coordination” in Served NR Cell Information IE and NR Neighbour Information IE in EN-DC X2 Setup/Configuration Update.

Proposal 2: the corresponding stage 2 texts on “Only Resource Coordination” is also needed in TS37.340.

Proposal 3: to introduce a new attribute” Only Resource Coordination” in ANR function of TS36.300.

Proposal 4: it is need to send a LS to SA5 on support of the new attribute “Only Resource Coordination” in EN-DC.

Proposal 5: to approve the corresponding (DRAFT)CRs [4][5][6].

Reference

- 3GPP TSG-RAN WG3 #115-e chairman notes

- R3-222048 Comparison of Potential Solutions for PRACH Coordination, China Telecom, ZTE, Huawei, CATT

- R3-222094, LTE-NR Resource Coordination, Ericsson

- R3-223329, TS36.423 CR on support of the coordination of resources between co-channel sharing LTE and NR cells

- R3-223507, TS36.300 CR on introduction of new attributes “Only Resource Coordination” in ANR

- R3-223519 37.340 CR on introduction of only resource coordination in EN-DC