3GPP TSG-RAN WG3 Meeting #116-e	R3-223406

E-meeting, 09 May – 19 May 2022

Agenda Item:	9.3.6

Source:	Ericsson

Title:	Conclusions on PRACH Coordination Between LTE and NR

Document for:	Discussion, Decision

# 1. Introduction

During RAN3-115-e, the following agreements and open points were captured regarding the topic of PRACH Coordination Between LTE and NR:

WA: Solution 1 is agreed to support resource coordination between LTE and NR SA sites.

Whether there is any spec impact on RAN3 and other WG will be discussed at next meeting.

It is useful to mention that during RAN3-114e the following agreement was also captured:

Reuse Protected E-UTRA Resource Indication (from LTE to NR) + E-UTRA-NR cell resource coordination information to support resource coordinate between SA NR and LTE. Whether to exchange PRACH Configuration as resource coordination information is FFS.

The agreed solution 1 is described below:

Solution 1: The assumption in Solution 1 is that an SA gNB can also support an EN-DC X2 connection. Namely an SA gNB can also act as an en-gNB. Therefore, the solution proposes to reuse the 	E-UTRA – NR Cell Resource Coordination function available over the EN-DC X2 interface to enable the coordination of resources between co-channel sharing LTE and NR cells.

This paper draws conclusions on this topic and proposes a way forward for Rel17.

# 2. Discussion

The agreements taken during RAN3-115e and RAN3-114e sealed the following principles:

- An SA gNB can also act as an en-gNB (see definition of Solution 1)

- The E-UTRA – NR Cell Resource Coordination procedure defined over X2 can be reused for PRACH Coordination Between LTE and NR

In light of the above principles, derived from the agreements taken, it is clear that there is no need to modify the specifications. This is because the specifications already allow a physical node hosting a gNB to also host an en-gNB. For that, the physical node is able to establish an EN-DC X2 interface with any neighbour eNB and to run the E-UTRA – NR Cell Resource Coordination procedure over it.

Conclusion1: No change to the current standard is needed to adopt Solution 1 for PRACH Coordination Between LTE and NR

With the above in mind, we also acknowledge one concern from operators on establishing EN-DC X2 interfaces only for the purpose of resource coordination. It was explained at RAN3-114e that there is no mandatory functionality that shall be supported over the EN-DC X2 interface. The functionalities that need to be supported are decided between network and vendor and are related to the business model followed. For that, this discussion is out of scope of 3GPP.

Nevertheless, we could foresee a possible improvement to the standard consisting of configuring the EN-DC X2 interface for resource-coordination-only procedures.

As an example, this could be achieved by modifying TS36.300 as follows:

Start of change for TS36.300

### 22.3.2a	Automatic Neighbour Relation Function

The purpose of the Automatic Neighbour Relation (ANR) function is to relieve the operator from the burden of manually managing Neighbour Cell Relations (NCRs). Figure 22.3.2a-1 shows ANR and its environment:

Figure 22.3.2a-1: Interaction between eNB and O&M due to ANR

The ANR function resides in the eNB and manages the conceptual Neighbour Cell Relation Table (NCRT). Located within ANR, the Neighbour Detection Function finds new neighbours and adds them to the NCRT. ANR also contains the Neighbour Removal Function which removes outdated NCRs. The Neighbour Detection Function and the Neighbour Removal Function are implementation specific.

A Neighbour Cell Relation (NCR) in the context of ANR is defined as follows:

An existing Neighbour Relation from a source cell to a target cell means that eNB controlling the source cell:

a)	Knows the ECGI/CGI and PCI of the target cell.

b)	Has an entry in the Neighbour Cell Relation Table for the source cell identifying the target cell.

c)	Has the attributes in this Neighbour Cell Relation Table entry defined, either by O&M or set to default values.

For each cell that the eNB has, the eNB keeps a NCRT, see Figure 22.3.2a-1. For each NCR, the NCRT contains the Target Cell Identifier (TCI), which identifies the target cell. For E-UTRAN, the TCI corresponds to the E-UTAN Cell Global Identifier (ECGI) and Physical Cell Identifier (PCI) of the target cell. Furthermore, each NCR has some attributes. The attributes have the following definitions:

-	No Remove: If checked, the eNB shall not remove the Neighbour Cell Relation from the NRT.

-	No HO: If checked, the Neighbour Cell Relation shall not be used by the eNB for handover reasons.

-	No X2: If checked, the Neighbour Relation shall not use an X2 interface in order to initiate procedures towards the eNB parenting the target cell.

-  Only Resource Coordination: if checked, the neighbour relation shall use an X2 interface only in order to coordinate resources between source and the target cell.

End of change for TS36.300

The highlighted yellow parts in the example above show that the EN-DC X2 interface could be configured to carry out resource coordination only. The latter will solve the problem seen by some operators to deploy the EN-DC X2 interface with additional functionalities that are not needed.

Conclusion 2: An enhancement to the standard may consist of allowing EN-DC X2 utilisation configurations that enable the usage of the interface only for the purpose of resource coordination. This would avoid any need to support additional unnecessary functionalities over such interface

In light of the discussion and details presented above, the following is proposed:

Proposal 1: Agree that adoption of Solution 1 implies no changes to the current standard

Proposal 2: As an enhancement, it is proposed to specify in 36.300 that an EN-DC X2 interface can be configured to be used for resource coordination only

# Conclusions

This paper has analysed the status of discussions on “PRACH Coordination Between LTE and NR”.

The following conclusions were derived:

Conclusion1: No change to the current standard is needed to adopt Solution 1 for PRACH Coordination Between LTE and NR

Conclusion 2: An enhancement to the standard may consist of allowing EN-DC X2 utilisation configurations that enable the usage of the interface only for the purpose of resource coordination. This would avoid any need to support additional unnecessary functionalities over such interface

Proposal 1: Agree that adoption of Solution 1 implies no changes to the current standard

Proposal 2: As an enhancement, it is proposed to specify in 36.300 that an EN-DC X2 interface can be configured to be used for resource coordination only

On the basis of these conclusions, the following proposals have been made:

Proposal 1: Adoption of Solution 1 implies no changes to the current standard

Proposal 2: As an enhancement, it is proposed to specify in 36.300 that an EN-DC X2 interface can be configured to be used for resource coordination only

It is proposed to agree to the proposals above. In case Proposal 2 is agreeable, the example change provided in the paper can be taken as baseline for a CR to TS36.300