3GPP TSG-RAN WG3 Meeting #116-e	R3-223450

# Online Meeting, 9th  – 19th May 2022

Agenda Item:	09.3.12

Source:	Ericsson

Title: 	MUSIM correction

Document for:	discussion

# 1	Background

At the last RAN3 meeting, we have closed the MUSIM WI. But there are a couple of details to be refined.

This paper discusses the remaining details and propose the correction.

# 2	Discussion

## 2.1 F1 aspect

It is agreed that for MUSIM-GapConfig IE related to MUSIM gap:

- In CU to DU RRC Information, the “MUSIM Gap Config” is included (Optional), when it is included, it indicates that gNB-DU can use the MUSIM Gap.

- In DU to CU RRC Information, the “MUSIM Gap Config” is included (Optional), when it is included, it indicates that gNB-CU can use the MUSIM Gap

Currently the specification only capture the aspect related to ”CU to DU RRC Information”.

In our view, the MUSIM-GapConfig works as in below:

- CU determins if it would like to generate the MUSIM Gap. When it is decided to do so, it will generate ”MUSIM-GapConfig” and send to DU in the CU to DU RRC Information; If CU decides not to generate the MUSIM Gap, it will not include the MUSIM-GapConfig IE;

- If DU recieves the MUSIM-GapConfig, and if it decides to use it, DU should not generate any ” MUSIM-GapConfig”.

- If DU receives the MUSIM-GapConfig, but DU decides not use it, it generates the MUSIM-GapConfig and send it in the DU to CU RRC Information. This is also the case when MUSIM-GapConfig is not received.

- When CU receives the MUSIM-GapConfig IE from DU, it understands that DU has generated a new one and should use it for any further action.

With the above understanding, we propose the below change for F1AP:

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT SETUP REQUEST message, then gNB-DU shall take it into account for MUSIM gap configuration. If thegNB-DU decides to generate MUSIM gap, it shall send the MUSIM gaps information based on the UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

The above correction is submitted in CR R3-22xxxx.

Proposal 1: RAN3 to discuss and agree the above change related to MUSIM gap in F1AP.

## 2.2	NGAP Aspect

It is not specified how NG-RAN node should use Paging Cause Indication for Voice Service IE.

If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE and it indicates “supported”, the NG-RAN node shall, if supported, store it and use it in RAN paging for the UE in RRC_INACTIVE state when sending the Paging Cause,

Proposal 2: RAN3 to discuss and agree the changes on NGAP.

# 3	Proposals

Proposal 1: RAN3 to discuss and agree the above change related to MUSIM gap in F1AP.

Proposal 2: RAN3 to discuss and agree the changes on NGAP.

The CRs are submitted in [1] and [2]

# 4	Reference

[1]	R3-223448	MUSIM Correction on F1AP procedural text, Ericsson, Qualcomm Incorporated

[2]	R3-223449	MUSIM Correction on NGAP, Ericsson