3GPP TSG-RAN3 Meeting #116-e	R3-223485

# E-meeting, May 9th – 19th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0219 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In step 11 of Remote UE’s RRC Reestablishment procedure of TS 38.401, the RB ID included in the SRAP header by Relay UE is the ID for SRB0, thus the description in this step can be modified to be clearer. - In L2 U2N relay, the bearer mapping configurations for Relay UE and Remote UE are different. The bearer mapping for Relay UE are between U2N Remote UE’s DRB/SRB(s) and PC5/Uu Relay RLC channel(s), while the bearer mapping for Remote UE are between U2N Remote UE’s DRB/SRB(s) and PC5 Relay RLC channel(s). However, in the working procedures for Reomte UE in TS 38.401, such as step 30 in clause 8.19.1 and step 24 in clasue 8.19.2, the bearer mapping configuraition information for Remote UE are the same as that of Relay UE, which is not correct. - In Remote UE’s RRC Reestablishment procedure of TS 38.401, if the U2N Relay UE have the local ID of the Remote UE before receiving the RRCReestablishmentRequest message from Remote UE, then it does not need to receive the local ID from gNB. So, the description in step 11 should be modified. - In step 13 of Remote UE’s RRC Reestablishment procedure, the Uu Relay RLC channel for relaying can be used for the SRB1 messages RRCReestablishment and RRCReestablishmentComplete, thus it is not only for the SRB1 toward gNB-DU. In step 13 of Remote UE’s RRC Inactive to other state procedure, the same problem exists. - For 5G ProSe service, the NR UE Sidelink Aggregate Maximum Bit Rate IE originally for NR V2X services is reused to indicate the 5G ProSe UE PC5 Aggregate Maximum Bit Rate. It means NR UE Sidelink Aggregate Maximum Bit Rate IE provides information on the Aggregate Maximum Bitrate of the UE’s sidelink communication for NR V2X services or 5G ProSe services. However, the definitations for this IE in in TS 38.413, TS 38.423, and TS 38.473 are different and not clear. - Some editorials exists |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In step 11 of Remote UE’s initial acces procedure (clause 8.19.1) in TS 38.401, clarify the RB ID included in the SRAP header is the RB ID for SRB0. - In step 30 of Remote UE’s initial acces procedure (clause 8.19.1), step 24 of Rremote UE’s RRC Reestablishment procedure (clause 8.19.2) and RRC Inactive to other states procedure (clause 8.19.3) in TS 38.401,, clarify that the bearer mapping configuration for remote UE is between U2N Remote UE’s DRB/SRB(s) and PC5 Relay RLC channel(s). - Modify the descripton for step 11 in clause 8.19.2 of TS 38.401, by considering the case that Relay UE have the local ID of the Remote UE before receiving the RRCReestablishmentRequest message from Remote UE. - In TS 38.401, modify the descriptions for step 13 of Remote UE’s RRC Reestablishment procedure and step 13 of Remote UE’s RRC Inactive to other state procedure that the Uu Relay RLC channel is used for relaying SRB1 over Uu. - Modify and align the definitations for NR UE Sidelink Aggregate Maximum Bit Rate IE in TS 38.413, TS 38.423, and TS 38.473. - Fix the editorials. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The Descriptions of the bearer mapping configuraton for Remote UE are incorrect. - The description for Step 11 of Remote UE’s RRC Reestablishment procedure is not correct, which do not consider the case Relay UE have the local ID of the Remote UE before receiving the RRCReestablishmentRequest message from Remote UE. - In step 13 of Remote UE’s RRC Reestablishment procedure and step 13 of Remote UE’s RRC Inactive to other state procedure, the descriptions for Uu Relay RLC channel are incorrect. - The definitations for NR UE Sidelink Aggregate Maximum Bit Rate IE in TS 38.413, TS 38.423, and TS 38.473 are not clear and aligned. - Some editorials still exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TS 38.401: 6.1.6, 8.19; TS 38.413: 9.3.1.148; TS 38.423: 9.2.3.107; TS 38.473: 9.3.1.119. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------------Start of Change---------------------------------------------------------------

### 6.1.6	Protocol stacks of L2 UE-to-Network Relay

The protocol stacks for the user plane and control plane of L2 U2N Relay architecture are described in Figure 6.1.5-1 and Figure 6.1.5-2, respectively. The Uu SRAP is terminated between U2N Relay UE and gNB-DU.

Figure 6.1.5-1: User plane protocol stack for L2 UE-to-Network Relay

Figure 6.1.5-2: Control plane protocol stack for L2 UE-to-Network Relay

----------------------------------------------------------------Next Change----------------------------------------------------------------

## 8.19	Overall procedures for L2 UE-to-Network Relay

### 8.19.1	Remote UE initial access

The signalling flow for Remote UE Initial access is shown in Figure 8.19.1-1.

Figure 8.19.1-1: Remote UE Initial Access procedure

1.	The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 RLC Channel.

3.	The U2N Relay UE withholds the received RRC message and sends the SidelinkUEInformation message to the gNB-DU. Before that, if the U2N Relay UE is in RRC_IDLE/RRC_INACTIVE status, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED status. During RRC connection establishment procedure of the U2N Relay UE, gNB may configure the U2N Relay UE with Uu RLC channel(s) for relaying of U2N Remote UE’s SRB0/1.

4.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the SidelinkUEInformation message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

5.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Relay UE to gNB-DU. Such message may request the establishment of Uu RLC channel(s) for the transmission of U2N Remote UE’s SRB0/1 if not configured yet.

6.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Relay UE to gNB-CU.

7.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0/1.

8.	The gNB-DU sends the RRCReconfiguration message to the U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu RLC channel configuration and bearer mapping for relaying of U2N Remote UE’s SRB0/1.

9.	The U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

11.	After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

12.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE , the gNB-DU UE F1AP ID of the U2N Relay UE and the container of SL-PHY-MAC-RLC-Config for at least the PC5 RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message .

13.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

14.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N Relay UE.

15.	The gNB-CU configures the U2N Relay UE with PC5 RLC channel, Uu RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes a Uu RLC channel for relaying of SRB1 towards gNB-DU if not configured yet.

NOTE 1: This step may be performed earlier, e.g., via steps 5~8.

16.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N Relay UE.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

18.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

19.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

20.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

21.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via U2N Relay UE.

22.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

23.	The U2N Remote UE responds with the SecurityModeComplete message.

24.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

25.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE.

27.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N Relay UE.

28.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

29.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

30.	The gNB-CU configures additional Uu RLC channel(s) between the gNB-DU and the U2N Relay UE,  additional PC5/Uu RLC channel(s) for the U2N Relay UE, and additional PC5 RLC channel(s) for the Remote UE for the relaying of U2N Remote UE’s DRB(s) and SRB(s). Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5/Uu RLC channel(s) at the U2N Relay UE, and configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5 RLC channel(s) at the U2N Remote UE.

NOTE 2: This step may be performed earlier.

### 8.19.2	Remote UE RRC Reestablishment

The signalling flow for Remote UE RRC Reestablishment is shown in Figure 8.19.2-1.

Figure 8.19.2-1: Remote UE RRC Reestablishment procedure

1.	The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure. This step may be omitted if PC5 connection was established.

2.	The U2N Remote UE sends an RRCReestablishmentRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10.	The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.19.1.

11.	After receiving/determining the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCReestablishmentRequest message of the U2N Remote UE to gNB-DU.

12.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL      UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCReestablishmentRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Relay UE and the container of SL-PHY-MAC-RLC-Config for at least the PC5 RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

13.	The gNB-CU configures the U2N Relay UE with PC5 RLC channel, Uu RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes a Uu RLC channel for relaying of SRB1 over Uu.

NOTE 1: This step may be performed earlier, e.g., via steps 5~8.

14~23.	The details of those steps can be referred to Steps 5~14 in Section 8.7. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Steps 18~19 may additionally perform the configurations of PC5 RLC channel(s) for relaying of U2N Remote UE’s SRB2 and DRBs.

24.	The gNB-CU configures additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5/Uu RLC channels for the U2N Relay UE, and additional PC5 RLC channel(s) for the Remote UE for relaying of U2N Remote UE’s DRB(s) and SRB(s). Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5/Uu RLC channel(s) at the U2N Relay UE, and configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5 RLC channel(s) at the U2N Remote UE.

NOTE 2: This step may be performed earlier.

### 8.19.3	Remote UE RRC Inactive to other states

The signalling flow for Remote UE from RRC Inactive to other states is shown in Figure 8.19.3-1.

Figure 8.19.3-1: Remote UE RRC Resume procedure

1.	The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR Prose procedure. This step may be omitted if PC5 connection was established.

2.	The U2N Remote UE sends an RRCResumeRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10.	The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.19.1.

11.	After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCResumeRequest message of the U2N Remote UE to gNB-DU.

12.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL   UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCResumeRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Relay UE and the container of SL-PHY-MAC-RLC-Config for at least the PC5 RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

13.	The gNB-CU configures the U2N Relay UE with PC5 RLC channel, Uu RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes a Uu RLC channel for relaying of SRB1 over Uu.

NOTE 1: This step may be performed earlier, e.g., via steps 5~8.

14~19.	The details of those steps can be referred to Steps 5~10 in Section 8.6.2. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Steps 14~15 may additionally perform the configurations of PC5 RLC channel(s) for relaying of U2N Remote UE’s SRB2 and DRBs.

20.	The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5/Uu RLC channel(s) for the U2N Relay UE, and additional PC5 RLC channel(s) for the Remote UE for relaying of U2N Remote UE’s DRB(s) and SRB(s). Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5/Uu RLC channel(s) at the U2N Relay UE, and configure the bearer mapping between U2N Remote UE’s DRB/SRB(s) and PC5 RLC channel(s) at the U2N Remote UE.

NOTE 2: This step may be performed earlier.

### 8.19.4	Service Continuity for L2 U2N relay

#### 8.19.4.1	Inter-gNB-DU switch from direct to indirect path

The signalling flow for U2N Remote UE switch from direct to indirect path with gNB-DU change is shown in Figure 8.19.4.1-1.

Figure 8.19.4.1-1: U2N Remote UE Direct-to-indirect Path Switch with gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to a target U2N Relay UE under a different gNB-DU (i.e., target gNB-DU).

3.	The reconfiguration to target U2N Relay UE is performed among U2N Relay UE, the target gNB-DU and gNB-CU, if the U2N Relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the target U2N Relay UE. If the target Relay UE is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to the target U2N Relay UE is performed in Step 10.

4.	gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

5.	gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU.

7.	The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE.

10.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the target gNB-DU via the target U2N Relay UE. In case the U2N Relay UE is in RRC_IDLE/ INACTIVE state when receiving the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N Relay UE to enter RRC_CONNECTED state.

11.	The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

12.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

13.	The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

#### 8.19.4.2	Intra-gNB-DU switch from direct to indirect path

The signalling flow for U2N Remote UE switch from direct to indirect path without gNB-DU change is shown in Figure 8.19.4.2-1.

Figure 8.19.4.2-1: U2N Remote UE Direct-to-indirect Path Switch without gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to a target U2N Relay UE under the same gNB-DU.

3.	The reconfiguration to target U2N Relay UE is performed among U2N Relay UE, gNB-DU and gNB-CU if U2N Relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the target U2N Relay UE. If the target Relay UE is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to the target U2N Relay UE is performed in Step 9.

4.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the U2N Remote UE to gNB-DU, which contains the path switch configuration at least. The F1-U packets of the U2N Remote UE can be continuously transmitted via previous tunnels if there is no tunnel update in this step.

5.	gNB-DU responds with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message by including the RRCReconfiguration message to gNB-DU. The contents in the RRCReconfiguration message can include at least path switch configuration, PC5 RLC channel configuration for relay traffic and the associated radio bearer(s).

7.	gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE.

9.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB-DU via the target U2N Relay UE. In case the U2N Relay UE is in RRC_IDLE/INACTIVE state when receiving the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N Relay UE to enter RRC_CONNECTED state.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

---------------------------------------------------------------End of Change---------------------------------------------------------------

# ANNEX 2. Correction for TS 38.413

--------------------------------------------------------------Start of Change---------------------------------------------------------------

#### 9.3.1.148	NR UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s sidelink communication for NR V2X services or 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.4 | Value 0 is not valid, and considered as a logical error by the receiving NG-RAN node. |

---------------------------------------------------------------End of Change---------------------------------------------------------------

# ANNEX 3. Correction for TS 38.423

--------------------------------------------------------------Start of Change---------------------------------------------------------------

#### 9.2.3.107	NR UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s sidelink communication for NR V2X services or 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.2.3.4 | Value 0 shall be considered as a logical error by the receiving NG-RAN node. |

---------------------------------------------------------------End of Change---------------------------------------------------------------

# ANNEX 4. Correction for TS 38.473

--------------------------------------------------------------Start of Change---------------------------------------------------------------

#### 9.3.1.119	NR UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s communication for NR V2X services or 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.4 | Value 0 shall be considered as a logical error by the receiving gNB-DU. |

---------------------------------------------------------------End of Change---------------------------------------------------------------