3GPP TSG-RAN3 Meeting #116-e	R3-223548

09 – 19 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0070 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of pre-confiugred measurement gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2022-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | LMF is not aware of the preconfigured measurement gap that is (de)activated by UE initiation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new procedure Measurement Activation Notify |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pre-configured measuremeng gap cannot be well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.x, 9.1.1.x, 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.473 CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==== Start of change ====

8.2.x	Measurement Activation Notify

8.2.x.1	General

The purpose of the Measurement Activation Notify procedure is to enable the NG-RAN node to inform the LMF that the pre-configured measuremernt gap is activated or deactivated by UE initiation. The procedure uses UE-associated signalling.

8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Measurement Activation Notify procedure, successful operation

The NG-RAN node initiates the procedure by sending a MEASUREMENT ACTIVATION NOTIFY message.

The MEASUREMENT ACTIVATION NOTIFY message shall contain the pre-configured measurement gap information that is activated or deactivated by UE initiation.

Upon reception of the MEASUREMENT ACTIVATION NOTIFY message, the LMF shall consider the pre-confiured measurement gap is activated or deactivated by UE initiation.

8.2.x.3	Unsuccessful Operation

Not Applicable.

==== Next Change ====

#### 9.1.1.x	MEASUREMENT ACTIVATION NOTIFY

This message is sent by the NG-RAN node to indicate the LMF the preconfigured measurement gap activated or dactivated by UE initiation.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Measurement Activation Indication | M |  | ENUMERATED (Active, Deactivate, ...) |  | YES | reject |
| PosMeasGapPreConfig | O |  | OCTET STRING | PosMeasGapPreConfig as defined in TS 38.331 [13], indicates preconfigured measurement gap configuration that is activated or deaciveted by UE initiation | YES | ignore |

==== ASN.1 Change ====

MeasurementPreconfigurationRefuse,

MeasurementActivation,

MeasurementActivationNotify

==== next change ====

id-measurementPreconfiguration,

id-measurementActivation,

id-MeasurementActivationNotify

==== next change ====

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementFailureIndication		|

e-CIDMeasurementReport					|

e-CIDMeasurementTermination				|

errorIndication							|

privateMessage							|

assistanceInformationControl			|

assistanceInformationFeedback 			|

positioningInformationUpdate			|

measurementReport						|

measurementUpdate						|

measurementAbort						|

measurementFailureIndication		|

positioningDeactivation		|

measurementActivation       |

measurementActivationNotify,

...

}

==== next change ====

measurementActivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

measurementActivationNotify NRPPa-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivationNotify

PROCEDURE CODE			id-MeasurementActivationNotify

CRITICALITY				ignore

}

==== next change ====

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-PRSConfigRequestType,

id-MeasurementActivationIndication,

id-PosMeasGapPreConfig,

==== next change ====

-- **************************************************************

--

-- Measurement Activation Notify

--

-- **************************************************************

MeasurementActivationNotify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MeasurementActivationNotifyIEs}},

...

}

MeasurementActivationNotifyIEs NRPPa-PROTOCOL-IES ::= {

{ ID id-MeasurementActivationIndication		CRITICALITY reject	TYPE MeasurementActivationIndication	PRESENCE mandatory	}|

{ ID id-PosMeasGapPreConfig					CRITICALITY ignore	TYPE PosMeasGapPreConfig					PRESENCE optional	   },

...

}

MeasurementActivationIndication ::= ENUMERATED (Active, Deactivate, ...)

PosMeasGapPreConfig ::= OCTET STRING

==== next change ====

id-TRPBeamAntennaInformation									ProtocolIE-ID ::= 93

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementActivationIndication, 							ProtocolIE-ID ::= xx

id-PosMeasGapPreConfig	                    					ProtocolIE-ID ::= yy

==== end of ANS.1 change ====