3GPP TSG-RAN WG3 Meeting #116-e	R3-223627

E-meeting, 09 May – 19 May 2022

Title: 	Consent information exchange between E1 interface

Source: 	Huawei

Agenda item:	9.1.6.1.1

Document Type:	Discussion

# 1. Introduction

In RAN3#115-e meeting, one company proposed to introduce consent from CU-CP to CU-UP, to indicate whether the CU-UP is allowed to overwrite the QoS Flow mapping indicated by the gNB-CU-CP. In order to progress and complete the WI, there was a compromise to temporarily capture it in the specifications but with Editor’s note to clarify that whether to have it needs further discussion.

In this contribution, we further analyses this topic, and propose to remove the consent information.

# 2. Background

Consent related specifications are listed below, it could be easily understood that there is no consensus on this IE, the IE is FFS:

| TS 38.401 | #### 8.15.1.1 Broadcast MBS Session Setup xxx In case of shared NG-U termination, - the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs. - at establishment of the BC bearer context in the gNB-CU-UP, the gNB-CU-UP may overwrite the QoS flow mapping indicated by the gNB-CU-CP, if the gNB-CU-CP has provided its consent to do so at BC bearer context setup. Editor’s Note: providing the consent from the gNB-CU-CP to the gNB-CU-UP needs further discussions. #### 8.15.1.2 Multicast MBS Session Activation In case of shared NG-U termination, - the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs. - at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU-UP may overwrite the QoS flow mapping indicated by the gNB-CU-CP, if the gNB-CU-CP has provided its consent to do so at MC bearer context setup. Editor’s Note: providing the consent from the gNB-CU-CP to the gNB-CU-UP needs further discussions. |
| --- | --- |
| 37.483 | #### 9.3.1.115 Consent to Apply Available Shared UP MBS QoS flow mapping This IE provides information whether the gNB-CU-CP consents to the gNB-CU-UP’s decision to apply the MBS QoS flow to MRB mapping already decided at the shared NG-U termination. Editor’s Note: This IE is FFS. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Consent to Apply Available Shared UP Termination MBS QoS flow mapping; M; ENUMERATED (allowed, ...). #### 9.3.3.26 BC Bearer Context To Setup This IE contains MBS session resource related information used to request BC Bearer Context Context Setup. IE/Group Name; Presence; Range; IE type and reference; Semantics description. S-NSSAI; M; 9.3.1.9. BC Bearer Context NG-U TNL Info at 5GC; M; 9.3.1.112. BC MRB To Setup List; M; BC MRB Setup Configuration 9.3.1.114. Consent to Apply Available Shared UP MBS QoS flow mapping; O; 9.3.1.115; Consent to Apply Available Shared NG-U Termination’s MBS QoS flow mapping Editor’s Note: Whether the CU CP needs to indicate its consent to the CU UP is FFS, as well as the semantics of this IE and its usage.. #### 9.3.3.32 MC Bearer Context To Setup This IE contains MBS session resource related information used to request MC Bearer Context Context Setup. IE/Group Name; Presence; Range; IE type and reference; Semantics description. S-NSSAI; M; 9.3.1.9. MC MRB To Setup List; M; MC MRB Setup Configuration 9.3.1.120. Consent to Apply Available Shared UP MBS QoS flow mapping; O; 9.3.1.115; Consent to Apply Available Shared NG-U Termination’s MBS QoS flow mapping Editor’s Note: Whether the CU CP needs to indicate its consent to the CU UP is FFS, as well as the semantics of this IE and its usage.. ### 9.4.5 Information Element Definitions ##### -- BCBearerContextToSetup BCBearerContextToSetup ::= SEQUENCE { snssai SNSSAI, bcBearerContextNGU-TNLInfoat5GC BCBearerContextNGU-TNLInfoat5GC, bcMRBToSetupList BCMRBSetupConfiguration, consent ConsentApplyAvailSharedUPMBSQoSFlowMapping OPTIONAL, iE-Extensions ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} } OPTIONAL, ... } ConsentApplyAvailSharedUPMBSQoSFlowMapping ::= ENUMERATED { allowed, ... } ##### -- MCBearerContextToSetup MCBearerContextToSetup ::= SEQUENCE { snssai SNSSAI, mcMRBToSetupList MCMRBSetupConfiguration, consent ConsentApplyAvailSharedUPMBSQoSFlowMapping OPTIONAL, iE-Extensions ProtocolExtensionContainer { {MCBearerContextToSetup-ExtIEs} } OPTIONAL, ... } |

# 3. Discussion

There was a well-known agreement about how to achieve same QoS flow to MRB mapping between NG-RAN nodes:

- Sync in terms of QoS flow to MRB mapping among NG-RAN nodes is achieved by network implementation.

Therefore in case there are multiple gNBs, they will be able to achieve same QoS flow to MRB mapping, and in case theses gNBs are disaggregated gNBs, different gNB-CU-CPs will be able to make the same decision on QoS flow to MRB mapping.

Conclusion 1: based on existing agreement, different gNB-CU-CPs will make same decision on QoS flow to MRB mapping, the agreed solution is network implementation.

Based on the description of the “consent”, it is sent from gNB-CU-CP to gNB-CU-UP, to indicate whether the CU-UP is allowed to overwrite the QoS Flow mapping indicated by the gNB-CU-CP. But with the agreement above, the different gNB-CU-CP will never provide different QoS flow to MRB mapping to the common gNB-CU-UP, therefore the “consent” is a useless solution.

Conclusion 2: as different gNB-CU-CP will never provide different QoS flow to MRB mapping to the common gNB-CU-UP, the “consent” is a useless solution.

On the other hand, in current network architecture, the gNB-CU-CP is the entity to make decision on QoS flow to DRB mapping, and gNB-CU-UP follows, but in this “consent” solution, the gNB-CU-UP is able to overwrite the QoS Flow mapping indicated by the gNB-CU-CP, which is not aligned with existing architecture principle.

Conclusion 3: the “consent” solution enables the gNB-CU-UP to overwrite the QoS Flow mapping indicated by the gNB-CU-CP, which is not aligned with existing architecture principle.

Proposal: Remove “consent” related aspects in the specifications.

# 4. Conclusions and Proposal

- In this contribution, we analyses the open issue on “consent” solution, get the following conclusions and proposal:

Conclusion 1: based on existing agreement, different gNB-CU-CPs will make same decision on QoS flow to MRB mapping, the agreed solution is network implementation.

Conclusion 2: as different gNB-CU-CP will never provide different QoS flow to MRB mapping to the common gNB-CU-UP, the “consent” is a useless solution.

Conclusion 3: the “consent” solution enables the gNB-CU-UP to overwrite the QoS Flow mapping indicated by the gNB-CU-CP, which is not aligned with existing architecture principle.

Proposal: Remove “consent” related aspects in the specifications

The corresponding CR to TS38.401 and TS37.483 are provided in [1] and [2].

# 5. Reference

- R3-223628 Correction on the FFS consent information, CR to TS 38.401, Huawei

- R3-223629 Correction on the FFS consent information, CR to TS 37.483, Huawei