3GPP TSG RAN3 meeting #116-e		      	        R3-223686

9 - 19 May 2022- Online

Agenda Item:	9.1.4.2

Source:	China Unicom (moderator)

Title:	Summary of Offline Discussion on CB: # QoE3_ASN

Document for:	Discussion

# 1		Introduction

CB: # QoE3_ASN

- Check the details

- Approve the CRs if agreeable

(CU - moderator)

Summary of offline disc R3-223686

# 2		For the Chairman’s Notes

Propose the following:

R3-223512, R3-223665 and R3-223638 can be approved.

For NGAP, R3-223135 and R3-223639 needs to be revised and checked in the next round following the conclusion of CB#QoE2_Stage3.

For F1AP:

Agree to remove “id-QoEInformationList” in section 9.4.5 in R3-223640.

For other changes in R3-223640 need further check in next round.

# 3		Discussion

The discussion will include all the proposed corrections to the ASN.1 as directed by the chairlady [1-6]. The details need to be checked carefully.

3.1 	CR to TS 38.413

As mentioned in R3-223135 [1], it will rely on the discussion results of CB: # QoE2_Stage3 on UE capability info and alignment of IE naming, the moderator suggests it can be discussed in CB#2 and further updated if needed.

Please fill in the table in case you have any comment on this CR[1].

| Company | Comment |
| --- | --- |
| CATT | The CR for UE RADIO CAPABILITY INFO INDICATION is pure ASN.1 issue. Agree to handle in stage3 CB. Also for “the Available RAN Visible QoE Metrics IE in ASN.1 coding” is not pure ASN.1 issue, the tabular needed be aligned. Check whether the stage3 CB include it? |
| ZTE | Agree to leave it to CB#QoE2. |
| Huawei | For capability, this leaves to CB#2; For the naming update, ok with the changes. |
| Nokia | We believe these proposals are all subject to discussions under QoE1 and QoE2 CBs, where in short our positions are: - Remove UE capability over NG: we agree - RVQoE metric naming: we agree - Overall IE renaming: see Nokia proposal in CB QoE2 (stage3) |
| Ericsson | CB: # QoE2_Stage3 |
| Samsung | Agree to leave it to CB#QoE2_Stage3. |

Summary：

Six companies responded, all six companies agreed to leave the CR under CB#QoE2_Stage3 because it is subject to stage3 discussion. The moderator then proposes to check and/or revise this CR after the final decision has been made by CB#QoE2_Stage3.

Proposal 1: R3-223135 needs to be revised and checked after the discussion of CB#QoE2_Stage3.

In R3-223639[4], the QMCConfigInfo is extended and the corresponding ASN.1 of NGAP have been fixed. Moderator has listed the proposals below:

- Add “UEAppLayerMeasInformation” and “UEAppLayerMeasInformation-ExtIEs”

- “uEAppLayerMeasInfoList” => “uEAppLayerMeasInformation”

companies are welcome to share views on above proposals:

| Company | Yes/No to each item | Comment |
| --- | --- | --- |
| CATT | Yes for both |  |
| ZTE | No | The correction makes the ASN.1 not align with the tabular. In tabular, the IE name is “UE Application Layer Measurement Information List” |
| Huawei | Yes to both | This is also related to CB#2, and it is related to the discussion on whether there is a need to have separate IE on QMC configuration and QoE contexts. To answer ZTE’s comment: the conception of list is reflected in the last level of sequence, which is compliant with ASN.1 coding rule. |
| Nokia | No | We agree we should try to make the ASN.1 structure as future proof as possible, and prefer to achieve this as discussed under CB QoE2 (stage 3). |
| Ericsson | Partly agree | In 1), please use ‘Info’ rather than ‘Information’. For 2), we would like to know what the problem with ‘List’ is? The proposed changes introduce misalignment between tabular and asn.1. The asn.1 should be aligned with both the tabular and the asn.1 in XnAP. |

Summary：

Five companies responded, 2 companies agree, 2 companies disagree and 1 company partly agrees. Meanwhile 2 companies prefer to leave the CR under CB#QoE2_Stage3. Since no consensus can be reached, the moderator proposes to check and/or revise this CR after the final decision has been made by CB#QoE2_Stage3 on the naming of UEAppLayerMeasInformation.

Proposal 2: R3-223639 needs to be revised and checked after the discussion of CB#QoE2_Stage3.

3.2 	CR to TS 38.423

As proposed in R3-223512 [2] and R3-223665 [6], the id-UEAppLayerMeasConfigInfo in section 9.3.7 is not used in the ASN.1, so it is proposed to remove this id.

Companies are welcome to share views on whether the id-UEAppLayerMeasConfigInfo should be removed?

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Yes |  |
| ZTE | Yes | It should be removed, which is not used. |
| Huawei | yes |  |
| Nokia | Yes |  |
| Ericsson | OK |  |
| Samsung | Yes |  |

Summary：

Six companies responded, all agree to this correction. Then the moderator proposes to agree the CR R3-223512 and R3-223665.

Proposal 3: CR R3-223512 and R3-223665 can be approved.

As shown in R3-223638[3], the QMCConfigInfo is extended and the corresponding ASN.1 of XnAP have been fixed. Moderator has listed the proposals below:

- “{(SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfo-Item” => “uEAppLayerMeasInfoList” and “iE-Extensions

- Add “UEAppLayerMeasInfoList”

- Add “QMCConfigInfo-ExtIEs”

- “QMCInfoConfig-ExtIEs” => “UEAppLayerMeasInfo-Item-ExtIEs”

Companies are welcome to share views on above proposals:

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Yes |  |
| ZTE | Yes |  |
| Huawei | Yes |  |
| Nokia | No | We agree we should try to make the ASN.1 structure as future proof as possible, and prefer to achieve this as discussed under CB QoE2 (stage 3). |
| Ericsson | OK |  |

Summary：

Five companies responded, four companies agree to this CR while one prefers to discuss it under CB#QoE2_Stage3. Overall the moderator thinks we can agree this CR first and discuss other ASN.1 structure details under the discussion of stage3.

Proposal 4: CR R3-223638 can be approved.

3.2 	CR to TS 38.473

As shown in R3-223640[5], the QoEINFORMATION is extended and the corresponding ASN.1 of F1AP have been fixed. Moderator has listed the proposals below.

- “QoEInformationList” => “QoEInformation”

- “id-QoEInformationList” => “id-QoEInformation” in 9.4.4 PDU Definitions and 9.4.7 Constant Definitions

- Remove “id-QoEInformationList” from 9.4.5 Information Element Definitions

- Add “QoEInformation” and “QoEInformation-ExtIEs”

Companies are welcome to share views on above proposals:

| Company | Yes/No | Comment |
| --- | --- | --- |
| CATT | Yes |  |
| ZTE | No | The correction makes ASN.1 not aligned with the tabular. We don’t have a “QoE Information IE” right? |
| Huawei | Yes | To answer ZTE’s comment, this is compliant with ASN.1 coding, since the list is reflected in the last level of sequence. |
| Nokia |  | Agree to remove unused imports (bullet 3). The extensibility proposal overlaps with our IE naming change discussed under QoE2 (stage 3) CB. |
| Ericsson | Partly agree | 1) No - it should be RANVisibleQoEInformationList 2) No - it should be id-RANVisibleQoEInformationList 3) OK 4) No - the prefix “RANVisible” should be added everywhere. |

Summary：

Five companies responded, two companies agree, one company disagrees and two companies partly agree to the CR. Based on the above discussion, the unused imports “id-QoEInformationList” can be removed while QoEInformationList coding changes cannot reach consensus. Then the moderator proposes to agree on “removing id-QoEInformationList from 9.4.5 ” and keep other changes as it was.

Proposal 5: For F1AP, remove “id-QoEInformationList” in section 9.4.5.

# 4		Conclusion, Recommendations [if needed]

# 5		Reference

- R3-223135, CR to TS 38.413 on ASN.1 corrections of QoE measurement (China Unicom) CR0780r, TS 38.413 v17.0.0, Rel-17, Cat. F

- R3-223512, CR to 38.423 for ASN.1 Corrections on NR QoE (CATT) CR0824r, TS 38.423 v17.0.0, Rel-17, Cat. F

- R3-223638, CR to 38.423 on ASN.1 correction (Huawei)	CR0841r, TS 38.423 v17.0.0, Rel-17, Cat. F

- R3-223639, CR to 38.413 on ASN.1 correction (Huawei, China Unicom)	CR0842r, TS 38.413 v17.0.0, Rel-17, Cat. F

- R3-223640, CR to 38.473 on ASN.1 correction (Huawei, China Unicom, China Mobile) CR0958r, TS 38.473 v17.0.0, Rel-17, Cat. F

- R3-223665, ASN.1 Correction for 38.423 on NR QoE (ZTE) CR0846r, TS 38.423 v17.0.0, Rel-17, Cat. F