3GPP TSG-RAN WG3 Meeting #116-e	R3-223812

# Online, May 9th – 19th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0100 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB Rel-17 Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the BAP mapping configuration function, the description for transfer of BAP related configurations other than BAP header rewriting configuration to the IAB-DU, and transfer of BAP related configurations to the IAB-donor-DU are missing. - Based on the current specification, NA resource configuration of a parent node IAB-DU or IAB-donor-DU could be provided from IAB-donor-CU to an IAB-donor-DU, which is incorrect since IAB-donor-DU has no co-located IAB-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merged the CR in R3-223297 and R3-223118, see below for details. - In the BAP mapping configuration function, add the description for transfer of BAP related configurations other than BAP header rewriting configuration to the IAB-DU, and transfer of BAP related configurations to the IAB-donor-DU. - Remove “an IAB-donor-DU or” so that NA resource configuration of a parent node IAB-DU or IAB-donor-DU is not provided from IAB-donor-CU to an IAB-donor-DU. - Some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification text is unclear and incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.12. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -: R3-223731 - Update the description for the support of IAB support function. Rev 1: R3-223808 - Fix the changes on changes - Replace ‘collocated’ with ‘co-located’ - In the added sentence for donor DU, replace ‘other’ with ‘the’. Rev 2: - Update the cover page to clearly state the reason for change and summary for change. |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

5	Functions of the F1 interface

5.1	General

The following clauses describe the functions supported over F1-C and F1-U.

5.2	F1-C functions

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

5.2.12	IAB support function

The support for IAB comprises several functions.

The BAP mapping configuration function allows the IAB-donor-CU to provide BAP mapping which includes the backhaul routing configuration and/or BH RLC channel mapping information for IAB-donor-DU or IAB-DU. This function also enables the IAB-donor-CU to provide the BAP header rewriting configuration, or other BAP related configurations to the IAB-DU. This function also enables the IAB-donor-CU to provide the BAP related configurations to the IAB-donor-DU.

The gNB-DU resource configuration function is used by the IAB-donor-CU to provide cell resource configuration for an IAB-donor-DU or an IAB-DU, and/or NA resource configuration of a parent node IAB-DU or IAB-donor-DU serving the co-located IAB-MT for an IAB-DU, and/or information about the child node’s cell resource configuration and other periodic configurations to a parent IAB-node or an IAB-donor-DU. This function also allows the IAB-donor-CU to provide the semi-static cell resource configuration of a neighbour node or a peer parent-node of a child node, whereas this neighbor node or a peer parent can be an IAB-donor-DU or an IAB-DU.

The IAB TNL address configuration function enables the IAB-donor-CU to request IP address(es) to be used for IAB-node(s) from an IAB-donor-DU, or to request from an IAB-donor-DU the removal of IP address(es) used for IAB-node(s). This function is also used by the IAB-donor-CU to provide an IAB-donor-DU with the IP address information of the traffic to be transferred to a peer IAB-donor-DU via an inter-donor-DU tunnel.

The IAB UP configuration update function allows the update of BH information or the UP TNL information between the IAB-donor-CU and an IAB-DU.

-------------------------------------------End of changes-------------------------------------------