3GPP TSG-RAN WG3 Meeting #116-e	R3-223854

# E-meeting, 9-19 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0213 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on RA-SDT overall procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Nokia, Nokia Shanghai Bell, Intel Corporation, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE |  |  |  |  |  | Date: |  |  | 2022-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the overall procedures for RA-SDT in section 8.18.1, it’s mentioned that the step 6 and 7 are not excuted in case of partial UE context transfer (SDT without anchor relocation case). However, it’s not clear how the DL F1-U TEID(s) is provided to another gNB for DL SDT transmission. We understand that the DL F1-U TEID(s) obtained in step 5 should be signalled to the last serving gNB-CU-CP within the PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message, and which will be used for transferring of the DL SDT data. - In Step 2, it’s clearly specified the UL SDT data and/or signalling is bufferred in gNB-DU. But when and how to proceed with the buffered UL SDT data and/or signalling in gNB-DU is not mentioned, which should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add some procedural texts in Note 2 of section 8.18.1 on how to use the F1-U DL TEIDs assigned in the receiving gNB-DU, as below: - The F1-U DL TEIDs received from the gNB-DU in step 5 should be forwarded to the other gNB-CU-CP, which will be provided to the gNB-CU-UP by the Bearer Context Modification procedure, and be used for transferring of the DL SDT data. - Add a NOTE to specify when and how to proceed with the buffered UL SDT data and/or signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedural texts for RA-SDT is not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -. CR Creation. Rev 1. Refine the coversheet and the Note 3. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Begin of change >>>>>>>>>>>>>>>>>>>>

8.18	Overall procedure for Small Data Transmission during RRC Inactive

8.18.1	RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.18.1-1.

Figure 8.18.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL SDT data and/or UL SDT signalling.

2.	The gNB-DU buffers the UL SDT data and/or UL SDT signalling.

3.	The step 3 is as defined in step 4 in clause 8.6.2, with including an indication of SDT access. The gNB-DU may also provide SDT assistance information.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2. The UL SDT data, if any, is forwarded to the gNB-CU-UP, and the UL signalling, if any, is forwarded to the gNB-CU-CP via the UL RRC MESSAGE TRANSFER message, in which any UL NAS PDU is delivered to AMF.

NOTE 1:	In case that full UE context is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP first establishes the UE context in the gNB-CU-UP via the Bearer Context Setup procedure and F1-U UL TEIDs are retreived before step 4. The BEARER CONTEXT SETUP REQUSET message may include an indication to suspend non-SDT bearers, and in this case, the BEARER CONTEXT MODIFICATION REQUEST message in step 6 does not include resume indication for SDT DRBs.

NOTE 2:	In case that only partial UE context for SDT including F1-U UL TEIDs is retrieved from another gNB-CU-CP as specified in TS 38.300 [2], the gNB-CU-CP uses those F1-U UL TEIDs for steps 4-5, and the subsequent steps 6-7 are not executed. The F1-U DL TEIDs received from the gNB-DU in step 5 should be forwarded to the other gNB-CU-CP, to be used for transferring of the DL SDT data. In addition, the UL SDT data, if any, is forwarded from the gNB-DU to the gNB-CU-UP of the other gNB-CU-CP for which the partial context is retrieved, and the UL signalling, if any, is forwarded from the gNB-CU-CP to the other gNB-CU-CP via the XnAP RRC TRANSFER message.

NOTE 3: The buffered UL SDT data/signalling in the gNB-DU could be sent to the other gNB-CU-UP/gNB-CU-CP afer step 5. The other gNB-CU-UP may need to buffer the UL SDT data if received before the SDT bearer(s) are resumed.

6.	The gNB-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message including an resume indication for SDT DRBs. The gNB-CU-CP also includes the F1-U DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-CP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>