3GPP TSG-RAN WG3 Meeting #116-e	R3-223878

# Online, 9-19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1881 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing the unnecessary LTE-M satellite indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no need for a separate (and duplicate) indication of UE cat. M1/M2 for NTN scenarios in the UE CAPABILITY INFO INDICATION message. This information is included in the legacy LTE-M Indication IE. The information on whether the UE is accessing from an NTN RAN is obtained from the RAT Type IE. SA2 has rectified their requirement in TS 23.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the unnecessary LTE-M Satellite Indication IE. Impact analysis: This CR is non-backwards compatible. This CR has isolated impact towards the previous version of the specification (same release) because it removes an optional IE. The impact can be considered isolated because it does not put any new requirements on the network or the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary and duplicate signaling remains; discrepancy with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2, 9.1.10, 9.3.3, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 8.9	UE Capability Info Indication

### 8.9.1	General

The purpose of the UE Capability Info Indication procedure is to enable the eNB to provide to the MME UE capability-related information.

### 8.9.2	Successful Operation

Figure 8.9.2-1: UE Capability Info Indication procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by sending a UE CAPABILITY INFO INDICATION message to the MME including the UE capability information. The UE CAPABILITY INFO INDICATION message may also include paging specific UE capability information within the UE Radio Capability for Paging IE. The UE capability information received by the MME shall replace previously stored corresponding UE capability information in the MME for the UE, as described in TS 23.401 [11].

If UE CAPABILITY INFO INDICATION message contains the LTE-M indication IE, the MME shall, if supported, store this information in the UE context and use it according to TS 23.401 [11].

If the UE indicates the support for UE Application Layer Measurement, the eNB shall if supported include the UE Application Layer Measurement Capability IE in the UE CAPABILITY INFO INDICATION message. The MME shall, if supported, store and use this information when initiating UE Application Layer Measurement.

If UE CAPABILITY INFO INDICATION message contains the UE Radio Capability – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

If the UE RADIO CAPABILITY INFO INDICATION message includes the UE Radio Capability for Paging IE and the UE Radio Capability for Paging – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

NEXT CHANGE

### 9.1.10	UE CAPABILITY INFO INDICATION

This message is sent by the eNB to provide UE Radio Capability information to the MME.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Radio Capability | M |  | 9.2.1.27 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MTSI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |
| LTE-M Indication | O |  | 9.2.1.135 |  | YES | ignore |
| UE Radio Capability – NR Format | O |  | 9.2.1.154 |  | YES | ignore |
| UE Radio Capability for Paging – NR Format | O |  | 9.2.1.160 |  | YES | ignore |

NEXT CHANGE

### 9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UNCHANGED PART OMITTED

id-NRUESidelinkAggregateMaximumBitrate,

id-PC5QoSParameters,

id-IntersystemSONConfigurationTransferMCT,

id-IntersystemSONConfigurationTransferECT,

id-UERadioCapabilityID,

id-UERadioCapability-NR-Format,

id-NotifySourceeNB,

id-eNB-EarlyStatusTransfer-TransparentContainer,

id-WUS-Assistance-Information,

id-NB-IoT-PagingDRX,

id-UERadioCapabilityForPaging-NR-Format,

id-PagingCause,

id-SecurityIndication,

id-SecurityResult,

id-LTE-NTN-TAI-Information

FROM S1AP-Constants;

UNCHANGED PART OMITTED

NEXT CHANGE

-- **************************************************************

--

#### -- UE CAPABILITY INFO INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Capability Info Indication

--

-- **************************************************************

UECapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UECapabilityInfoIndicationIEs} },

...

}

UECapabilityInfoIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory}|

{ ID id-UERadioCapabilityForPaging	CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional}|

{ ID id-UE-Application-Layer-Measurement-Capability		CRITICALITY ignore	TYPE UE-Application-Layer-Measurement-Capability							PRESENCE optional}|

{ ID id-LTE-M-Indication				CRITICALITY ignore	TYPE LTE-M-Indication				PRESENCE optional}|

{ ID id-UERadioCapability-NR-Format	CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging-NR-Format	CRITICALITY ignore	TYPE UERadioCapabilityForPaging				PRESENCE optional}

,

...

}

UNCHANGED PART OMITTED

NEXT CHANGE

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM S1AP-CommonDataTypes;

UNCHANGED PART OMITTED

id-RACSIndication									ProtocolIE-ID ::= 330

id-PagingCause										ProtocolIE-ID ::= 331

id-SecurityIndication									ProtocolIE-ID ::= 332

id-SecurityResult										ProtocolIE-ID ::= 333

id-E-RABSecurityResultItem							ProtocolIE-ID ::= 334

id-E-RABSecurityResultList							ProtocolIE-ID ::= 335

id-RAT-Restrictions									ProtocolIE-ID ::= 336

id-UEContextReferenceatSourceeNB					ProtocolIE-ID ::= 337

id-LTE-NTN-TAI-Information  						ProtocolIE-ID ::= 339

END

-- ASN1STOP

END OF CHANGES