3GPP TSG-RAN WG3 Meeting # 116-e	R3-224017

# Online, May 9th 2022 - May 19th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0813 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XnAP corrections for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2022-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The name of Channel occupancy time percentage IE does not indicate that the IE is associated to DL. ASN.1 encoding for Channel occupancy time percentage IE is not aligned with tabular (0 value missing) The semantic description for NR-U ARFCN IE can be clarified with relevant references. The value of maxnoofNR-UChannelID is not sufficient in case of multiple bandwidth in one cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Channel occupancy time percentage IE is renamed as Channel occupancy time percentage DL. ASN.1 encoding is aligned to tabular (0 value added). The semantic description for NR-U ARFCN IE is improved with references to tables in TS 38.101-1 containing allowed values for NR-U ARFCN. The value of maxnoofNR-UChannelID is extended from 4 to 16. Impact Analysis The CR has limited impacts the previous version of the specification, with BC ASN.1 changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Channel occupancy time percentage and NR-U ARFCN IEs lack clarity. The value of maxnoofNR-UChannelID can support multiple bandwidth in one cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.1.3.21, 9.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1 based on discussion during RAN3#116-e meeting. Rev. 2 CR form updated v12.2. |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

(text unchanged skipped)

[49]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[50]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[51]	3GPP TS 37.213: "NR; Physical layer procedures for shared spectrum channel access".

[52]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

-----------------------------------------------  Next Change ---------------------------------------------------------

#### 9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing. Value is in dBm. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |

-----------------------------------------------  Next Change ---------------------------------------------------------

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.40 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [10] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-TDD | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – |  |
| Connectivity Support | M |  | 9.2.2.28 |  | – |  |
| Broadcast PLMN Identity Info List NR |  | 0..<maxnoofBPLMNs> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [10]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the NR Cell Identity IE. | – |  |
| >>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >TAC | M |  | 9.2.2.5 |  | – |  |
| >NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  | – |  |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| >Configured TAC Indication | O |  | 9.2.2.39a | NOTE: This IE is associated with the TAC in the Broadcast PLMN Identity Info List NR IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.2.2.71 | If this IE is included the content of the Broadcast PLMNs IE in the Broadcast PLMN Identity Info List NR IE is ignored. | YES | reject |
| Configured TAC Indication | O |  | 9.2.2.39a | NOTE: This IE is associated with the TAC on top-level of the Served Cell Information NR IE | YES | ignore |
| NPN Broadcast Information | O |  | 9.2.2.71 | If this IE is included the content of the Broadcast PLMNs IE in the top Served Cell Information NR IE is ignored. | YES | reject |
| SSB Positions In Burst | O |  | 9.2.2.64 |  | YES | ignore |
| NR Cell PRACH Configuration | O |  | OCTET STRING | Containing 9.3.1.139 NR Cell PRACH Configuration as of TS 38.473 [41]. | YES | ignore |
| CSI-RS Transmission Indication | O |  | ENUMERATED (activated, deactivated, ...) | This IE indicates the CSI-RS transmission status of the given cell. If the Additional Measurement Timing Configuration List IE is present, this IE is ignored. | YES | ignore |
| SFN Offset | O |  | 9.2.2.75 |  | YES | Ignore |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated with the NR CGI. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) |  | – |  |
| NR-U Channel Info List |  | 0..1 |  |  | YES | ignore |
| >NR-U Channel Info Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | – |  |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth for NR bands restricted to operation with shared spectrum channel access, as defined in TS 37.213 [51]. Allowed values are specified in TS 38.101-1 [52] in Table 5.4.2.3-2, Table 5.4.2.3-3 and Table 5.4.2.3-4. | – |  |
| >>Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60MHz, 80MHz, …) |  | – |  |
| Additional Measurement Timing Configuration List | O | 1 .. <maxnoofMTCItems> |  |  | YES | Ignore |
| >Measurement Timing Configuration Index | M |  | INTEGER (0..16) | “0” refers to the configuration contained in the Measurement Timing Configuration IE. Any value between “1” and “16” refers to a configuration within the Additional Measurement Timing Configuration List IE. | – |  |
| >CSI- RS MTC Configuration List | M | 1 .. <maxnoofCSIRSconfigurations> |  | This list explicitly expresses the CSI-RS configurations contained in the MTC | – |  |
| >>CSI-RS Index | M |  | INTEGER (0..95) | Index of CSI-RS as in MTC | – |  |
| >>CSI-RS Status | M |  | ENUMERATED (activated, deactivated, …) | This IE indicates the CSI-RS transmission status of the configuration. | – |  |
| >>CSI-RS Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCells> |  | This list expresses the cells and CSI-RSs neighbouring the CSI-RS in the CSI-RS Index IE. | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>CSI-RS MTC Neighbour List | O | 1 .. < maxnoofCSIRSneighbourCellsInMTC> |  | This list expresses the CSI-RSs served by the NR CGI, which are neighbouring the CSI-RS of the served cell and contained in the MTC indicated by the neighbouring NR cell. | – |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [10]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [10]. First bit = 1Rx, second bit = 2Rx, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by one gNB. Value is 256. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

-----------------------------------------------  Start of ASN.1 Changes ---------------------------------------------------------

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						NR-U-ChannelID,

channelOccupancyTimePercentageDL	ChannelOccupancyTimePercentage,

energyDetectionThreshold			EnergyDetectionThreshold,

iE-Extension		ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} } 	OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-U-ChannelID ::= INTEGER (1..maxnoofNR-UChannelIDs, ...)

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)
EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)

-----------------------------------------------  Next ASN.1 Change ---------------------------------------------------------

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN 						INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

-----------------------------------------------  End of Changes ---------------------------------------------------------