3GPP TSG-RAN WG3 #116-e	R3-224039

Online, May 9th-19th, 2022

Title:	[Draft] Reply LS on Positioning in RRC_INACTIVE State

Response to:	LS (R3-223032) on LS on Positioning in RRC_INACTIVE State from SA2

Release:	Release 17

Work Item:	NR_pos_enh-Core/5G_eLCS_Ph2

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Jiancheng Sun

Tel. Number:

E-mail Address:	sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

1. Overall Description:

RAN3 thanks SA2 for the LS on Positioning in RRC_INACTIVE State and would like to provide the following information for SA2 consideration.

In Rel-17 SDT WI, two main use cases are supported, SDT with anchor relocation and SDT without anchor relocation.

- For SDT with anchor relocation, the UL/DL NAS PDU and/or NRPPa message can be directly exchanged between the receiving gNB (i.e. the new serving gNB) and AMF after N2 Path Switch procedure. In this case, DL-only and RAT-Independent positioning, UL-only positioning, and UL+DL positioning are all supported.

- For SDT without anchor relocation, the UL/DL NAS PDU can be forwarded between receiving gNB and the anchor gNB. However, the NRPPa message can not be fowarded between the receiving gNB and the anchor gNB in this case.

In consequence, in Positioning Rel-17, UL-only positioning and UL+DL positioning for RRC_INACTIVE UE are not supported in case of SDT without anchor relocation.

However, RAN3 agreed to discuss INACTIVE positioning in case of "without anchor relocation" in the Rel-18.

We kindly request SA2 to take above into account.

2. Actions:

To SA2 group.

ACTION: 	RAN3 asks SA2 group to take the above information into account.

3. Date of Next RAN WG3 Meetings:

RAN3 Meeting #117-e           August 15 – 25, 2022		Electronic Meeting

RAN3 Meeting #117bis-e      October 10 – 18, 2022 		Electronic Meeting