3GPP TSG-RAN WG3 #108-e	R3-223715

Online, 1-11 June 2020

Agenda Item:	9.3.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB: # 2_DynamicACL

Document for:	Approval

# Introduction

This documents is the Sod for the following CB:

CB: # 2_DynamicACL - Whether/How to transfer the source SN’s IP address to the target node? - Whether/How to forward the source IP address for newly established DRBs/QoS Flows via E1AP? - Whether/How to forward the MN’s source IP address / SN’s source IP address to the SN/ MN for newly establish DRBs/QoS Flows via X2AP/XnAP? - Whether/How to forward the MN’s source IP address to gNB-DU/ng-eNB-DU via F1AP/W1AP? - Try to close this topic, and provide CRs if agreeable. (HW - moderator) Summary of offline disc R3-223715

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## ACL in EN-DC/MR-DC to SA handover

In R3-223199, it states that the direct data forwarding path availability solutions for EN-DC/MR-DC to SA handover were agreed at last meeting in agenda item 9.3.1. The overall solutions agreed are summarized here for reference.

-	Case 1a, for EN-DC to SA gNB inter-system handover, the target SA gNB will identify the path availability for direct data forwarding between source SN and itself by the source node ID in source to target transparent container in Handover Request message.

-	Case 1b, for MR-DC to eNB/gNB handover, the source MN will inquire the source SN by target node ID. And the source SN will respond with the data forwarding path availability indication in SN Modification Request Ack message, if direct data forwarding is available between source SN and target node.

With those agreements, the following additional proposal is proposed in R3-223199 on top of the overall solution agreed at last meeting for ACL.

Proposal 1: The source SN’s IP address is transferred to the target node in source to target transparent container during S1/NG handover.

Besides, R3-223520 makes the same proposal for EN-DC to SA case, but thinks that the MR-DC to SA case was already covered by previous CRs. The moderator would like to remind that even in this case, the source SN’s IP is needed to be transferred to the target node in inter-system handover.

The moderator asks companies to provide your companies views here.

| Company | Do you agree on proposal 1? | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

## ACL for new DRBs/Qos Flows

Another remaining issue is that if new DRBs or QoS Flows are established during a UE connection, the source IP used for data forwarding for these DRBs/QoS Flows needs to be signalled over the RAN interfaces.

R3-223199 provides the following analysis and proposals.

For both the cases of single connectivity and dual connectivity, and in case of split RAN, the following changes are needed:

- The source IP for the newly added DRB/QoS Flow needs to be communicated from the gNB-CU-UP to the gNB-CU-CP via the E1AP Bearer Context Modification Response message

For dual connectivity cases, the following changes are needed:

- If a new DRB setup request is received for a UE during LTE-DC or EN-DC, the master eNB needs to transfer the new source IP allocated for the new bearers to the SN in X2AP SN Modification Request message. The SN also needs to transfer to the MN the source IP that will be used for data forwarding for the newly added DRB in X2AP SN modification request ack.

- If a new Qos Flow setup request is received for a UE during MR-DC, the master gNB needs to transfer the new source IP allocated for the new Qos Flows to the SN in XnAP SN Modification Request message. The SN also needs to transfer to the MN the source IP that will be used for data forwarding for the newly added QoS Flow in XnAP SN modification request ack.

- For split RAN, the MN’s IP address for those new DRBs/Qos Flows shall be forwarded to the gNB-DU/ng-eNB-DU over F1AP/W1AP in UE Context Modification Request message.

- For split RAN, the MN’s IP address for those new DRBs/Qos Flows shall be forwarded to the gNB-CU-UP over E1AP in Bearer Context Modification Request message.

Proposal 2: The source IP address for newly established DRBs/Qos Flows is transferred from gNB-CU-UP to gNB-CU-CP via E1 Bearer Context Modification Request/Response message message.

Proposal 3: The MN’s source IP address used for data forwarding towards the SN for new DRBs/Qos Flows established during ongoing LTE DC/EN-DC/MR-DC is transferred to the SN in X2/Xn SN Modification Request message.

Proposal 4: The SN’s source IP address used for data forwarding from the SN for new DRBs/Qos flows established during ongoing LTE DC/EN-DC/MR-DC is transferred to the SN in X2/Xn SN modification request ack message.

Proposal 5: The MN’s IP address is forwarded to the gNB-DU/ng-eNB-DU over F1AP/W1AP in UE Context Modification Request message.

In R3-223601, proposal same as proposal 5 is also proposed.

Please provide your companies views to above proposal 2 – 5.

| Company | Comments |
| --- | --- |
| Huawei | Agree to proposal 2 ~ 5 above. |

## LS from SA2

In R3-223601, it states that the LS (S2-2201272) from SA2 on indirect data forwarding has been received before current RAN3 meeting, which relates to the following RAN3 agreement made in 114-e meeting:

The ACL enhancement for indirect data forwarding for NG/S1 handover is pursued in Rel-17. The agreement will be revisited after the reply LS from SA2 is received.

It is proposed that:

Proposal 1: It is proposed for RAN3 to not consider the ACL enhancement on Indirect Data Forwarding in either Rel-16 or Rel-17.

Please provide your companies views here.

| Company | Comment |
| --- | --- |
| Huawei | Agree |

Furthermore, in R3-223396, a draft reply LS to SA2, CT4 is provided.

Please comment on the draft LS here:

| Company | Comment |
| --- | --- |
| Huawei | In the LS from S2-2201272, there seems no further questions from SA2 that needs RAN3 feedback. From this perspective, the LS is not necessary. |

## CR checking

In order to speed up the discussion a bit, if your views to questions in section 3.1 and 3.2 are positive, please also provide your comments on the CRs here or in the CRs in the draft folder directly if any.

Comments on the CRs:

| Tdoc No. | Title |  | Comments |
| --- | --- | --- | --- |
| R3-223200 | NGAP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0790r, TS 38.413 v16.9.0, Rel-16, Cat. F | [Huawei]: Agree to all the CRs. |
| R3-223201 | F1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0878r, TS 38.473 v16.9.0, Rel-16, Cat. F |  |
| R3-223202 | W1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0014r, TS 37.473 v16.8.0, Rel-16, Cat. F |  |
| R3-223203 | NGAP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0791r, TS 38.413 v17.0.0, Rel-17, Cat. A |  |
| R3-223204 | F1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0879r, TS 38.473 v17.0.0, Rel-17, Cat. A |  |
| R3-223205 | W1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0015r, TS 37.473 v17.0.0, Rel-17, Cat. A |  |
| R3-223321 | Dynamic ACL over E1 CR 38.463 (Ericsson, Deutsche Telekom, Huawei) | CR0698r, TS 38.463 v16.9.0, Rel-16, Cat. F |  |
| R3-223322 | Dynamic ACL over E1 CR 37.483 (Ericsson, Deutsche Telekom, Huawei) | CR0008r, TS 37.483 v17.0.0, Rel-17, Cat. A |  |
| R3-223323 | Dynamic ACL over S1 CR 36.413 (Ericsson, Deutsche Telekom, Huawei) | CR1879r, TS 36.413 v16.9.0, Rel-16, Cat. F |  |
| R3-223324 | Dynamic ACL over S1 CR 36.413 (Ericsson, Deutsche Telekom, Huawei) | CR1880r, TS 36.413 v17.0.0, Rel-17, Cat. A |  |
| R3-223325 | Dynamic ACL over X2 CR 36.423 (Ericsson, Deutsche Telekom, Huawei) | CR1693r, TS 36.423 v16.9.0, Rel-16, Cat. F |  |
| R3-223326 | Dynamic ACL over X2 CR 36.423 (Ericsson, Deutsche Telekom, Huawei) | CR1694r, TS 36.423 v17.0.0, Rel-17, Cat. A |  |
| R3-223327 | Dynamic ACL over Xn CR 38.423 (Ericsson, Deutsche Telekom, Huawei) | CR0803r, TS 38.423 v16.9.0, Rel-16, Cat. F |  |
| R3-223328 | Dynamic ACL over Xn CR 38.423 (Ericsson, Deutsche Telekom, Huawei) | CR0804r, TS 38.423 v17.0.0, Rel-17, Cat. A |  |

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-223199 | Discussion on remaining issues for ACL (Huawei, Deutsche Telekom, Ericsson) | discussion |
| --- | --- | --- |
| R3-223200 | NGAP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0790r, TS 38.413 v16.9.0, Rel-16, Cat. F |
| R3-223201 | F1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0878r, TS 38.473 v16.9.0, Rel-16, Cat. F |
| R3-223202 | W1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0014r, TS 37.473 v16.8.0, Rel-16, Cat. F |
| R3-223203 | NGAP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0791r, TS 38.413 v17.0.0, Rel-17, Cat. A |
| R3-223204 | F1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0879r, TS 38.473 v17.0.0, Rel-17, Cat. A |
| R3-223205 | W1AP CR for ACL remaining issues (Huawei, Deutsche Telekom, Ericsson) | CR0015r, TS 37.473 v17.0.0, Rel-17, Cat. A |
| R3-223321 | Dynamic ACL over E1 CR 38.463 (Ericsson, Deutsche Telekom, Huawei) | CR0698r, TS 38.463 v16.9.0, Rel-16, Cat. F |
| R3-223322 | Dynamic ACL over E1 CR 37.483 (Ericsson, Deutsche Telekom, Huawei) | CR0008r, TS 37.483 v17.0.0, Rel-17, Cat. A |
| R3-223323 | Dynamic ACL over S1 CR 36.413 (Ericsson, Deutsche Telekom, Huawei) | CR1879r, TS 36.413 v16.9.0, Rel-16, Cat. F |
| R3-223324 | Dynamic ACL over S1 CR 36.413 (Ericsson, Deutsche Telekom, Huawei) | CR1880r, TS 36.413 v17.0.0, Rel-17, Cat. A |
| R3-223325 | Dynamic ACL over X2 CR 36.423 (Ericsson, Deutsche Telekom, Huawei) | CR1693r, TS 36.423 v16.9.0, Rel-16, Cat. F |
| R3-223326 | Dynamic ACL over X2 CR 36.423 (Ericsson, Deutsche Telekom, Huawei) | CR1694r, TS 36.423 v17.0.0, Rel-17, Cat. A |
| R3-223327 | Dynamic ACL over Xn CR 38.423 (Ericsson, Deutsche Telekom, Huawei) | CR0803r, TS 38.423 v16.9.0, Rel-16, Cat. F |
| R3-223328 | Dynamic ACL over Xn CR 38.423 (Ericsson, Deutsche Telekom, Huawei) | CR0804r, TS 38.423 v17.0.0, Rel-17, Cat. A |
| R3-223520 | Discussion on ACL for MR-DC to SA handover scenario (CATT) | discussion |
| R3-223521 | CR on support of ACL for handover from EN-DC to SA (CATT) | CR0824r, TS 38.413 v16.9.0, Rel-16, Cat. F |
| R3-223522 | CR on support of ACL for handover from EN-DC to SA (CATT) | CR0825r, TS 38.413 v17.0.0, Rel-17, Cat. A |
| R3-223601 | Discussion on ACL issues (ZTE Corporation) | discussion |
| R3-223602 | Correction on ACL over W1AP (ZTE Corporation) | CR0016r, TS 37.473 v16.8.0, Rel-16, Cat. F |
| R3-223603 | Correction on ACL over F1AP (ZTE Corporation) | CR0954r, TS 38.473 v16.9.0, Rel-16, Cat. F |
| R3-223396 | Reply LS on On ACL support for Indirect Data Forwarding (Ericsson) | LS out To: SA2 CC: CT1, CT4 Move to 9.3.2 |