3GPP TSG-RAN WG3 #116-e	R3-223758

Online, 9-19 May 2022

Agenda Item:	9.3.9

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on NoGap Configuration

Document for:	Approval

# Introduction

CB: # 56_GapConfig

- Check MR-DC scenario

- Provide CRs if agreeable

(HW - moderator)

Summary of offline disc R3-223758

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (Round 1)

Based on the online discussion, there appears to be consensus that the two CRs could be agreed for single-connectivity case.

Proposal 1: Agree CRs in R3-223243/R3-223244 for single-connectivity case.

Question #1: If you have different view, or any comments to R3-223243/R3-223244, please provide your comments below.

| Company | Comment |
| --- | --- |

For NR-DC case, the field description for the interFrequencyConfig-NoGap-r16 in TS 38.331 is copied and highlighted as follows.

MeasConfig field descriptions

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

The moderator further checked the discussions in RAN2#116 meeting, with the following captured in the chair notes.

R2-2111505 Report of [Offline-011][NR16]RRC Measurements Other and LTE (Ericsson) Ericsson - [011] Noted, agreements reflected below R2-2110982 Discussion on inter-frequency no gap measurement in NR-DC Huawei, HiSilicon discussion Rel-16 NR_newRAT-Core - [[011] RAN2 agrees that based on the current specifications, it is not clear whether the measurement configurations from both the MN and the SN can contain the interFrequencyConfig-NoGap-r16 filed in the NR-DC scenario. Besides, UE’s corresponding behaviour for performing inter-frequency no gap measurement is also ambiguous. - [011] Only MN controls the inter-frequency measurement without gaps ((interFrequencyConfig-NoGap-r16)) feature. - [011] interFrequencyConfig-NoGap-r16 provided by MN applies to all the inter-frequency measurements configured by MN and SN R2-2111468 CR on inter-frequency gapless measurement Huawei, HiSilicon CR Rel-16 38.331 16.6.0 2862 - F TEI16 - [011] agreed

As observed from R2-2111505 (see the proposal 5 wherein), RAN2 already discussed the NR-DC case, and the agreed CR (R2-2111468) is the outcome.

In addition, there are some online comments that there can be MN-SN coordination (e.g. SN should be aware of the interFrequencyConfig-NoGap-r16). Based on the above observation, the moderator think that any possible additional work should be continued in RAN2 first.

Even if RAN2 finally agrees that there is need e.g., to signal the interFrequencyConfig-NoGap-r16 included in the inter-node RRC message from the MN to SN, the moderator think that currently there is no RAN3 specification impact foreseen.

- In TS 38.331, the M-NG-RAN node to S-NG-RAN node Container IE (i.e. CG-ConfigInfo) is already included in the S-NODE ADDITION REQUEST message

- In TS 38.473, the CG-ConfigInfo IE is already included in the CU to DU RRC Information in the UE CONTEXT SETUP REQUEST message.

Hence the moderator understands any possible work should be brought up in RAN2 first, then RAN3 can further check the potential impacts, if any.

Proposal: For NR-DC, any further work to support interFrequencyConfig-NoGap should be brought up in RAN2 first. RAN3 can further study upon any progress.

Question #2: Do you agree the above proposal, or any other views?

| Company | Comment |
| --- | --- |
| Huawei | Yes |

# Conclusion, Recommendations

TBD

# References

| R3-223243 | Correction on interFrequencyConfig-NoGap (Huawei, BT, Orange, Deutsche Telekom) | CR0888r, TS 38.473 v16.9.0, Rel-16, Cat. F *** E///: Need more time to check CB: # 56_GapConfig - Check MR-DC scenario - Provide CRs if agreeable (HW - moderator) Summary of offline disc R3-223758 |
| --- | --- | --- |
| R3-223244 | Correction on interFrequencyConfig-NoGap (Huawei, BT, Orange, Deutsche Telekom) | CR0889r, TS 38.473 v17.0.0, Rel-17, Cat.A |