3GPP TSG-RAN WG3#116-e	R3-223951

- E-meeting, 9 – 19 May 2022	was R3-223456 and R3-223383

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0013 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS shared CU UP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces the possibility for querying available radio bearer configuration of from the CU-UP before possibly adopting it as an additional possibility to the currently existing “consent” to any existing configuration, which is probably not always a suitable solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CU CP may select or not the shared CU UP depending on the offered area and/or the currently offered radio bearer configuration. In order to determine the current radio bearer configuration offered for sharing by a given CU UP it can query the CU UP before any reservation is made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CU-UP lacks the possibility to first query an existing the radio bearer configuration provided by a shared NG-U termination before consenting to it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1.1.2, 8.6.2.1.2, 9.3.1.115, 9.3.1.x (new), 9.3.3.26, 9.3.3.27, 9.3.3.32, 9.3.3.33, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

- 8.6.1.1.2	Successful Operation

Figure 8.6.1.1.2-1: BC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BC BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of an appropriate Shared NG-U Termination, if available if either of the following conditions are fulfilled:

-	the QueryRBConfiguration IE set to "true" is contained in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message, t

-	the Consent to Apply Available Shared UP MBS QoS flow mapping IE set to "allowed" is contained in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message and the radio bearer configuration of the Shared NG-U Termination is different than the one requested from the gNB-CU-CP.

Not modified

- 8.6.2.1.2	Successful Operation

Figure 8.6.2.1.2-1: MC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the MC BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall indicate in the MC BEARER CONTEXT SETUP RESPONSE message within the Available MC MRB Configuration IE in the MC Bearer Context To Setup Response IE the radio bearer configuration of an appropriate Shared NG-U Termination, if available if either of the following conditions are fulfilled:

-	the QueryRBConfiguration IE set to "true" is contained in the MC Bearer Context To Setup IE in the MC BEARER CONTEXT SETUP REQUEST message, t

-	the Consent to Apply Available Shared UP MBS QoS flow mapping IE set to "allowed" in the MC Bearer Context To Setup IE in the MC BEARER CONTEXT SETUP REQUEST message and the radio bearer configuration of the Shared NG-U Termination is different than the one requested from the gNB-CU-CP.

Not modified

- 9.3.1.115	Consent to Apply Available Shared UP MBS QoS flow mapping

This IE provides information whether the gNB-CU-CP consents to the gNB-CU-UP’s decision to apply the MBS QoS flow to MRB mapping already decided at the shared NG-U termination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Consent to Apply Available Shared UP Termination MBS QoS flow mapping | M |  | ENUMERATED (allowed, ...) |  |

- 9.3.1.x	Query RB Configuration

This IE indicates that the gNB-CU CP requests the radio bearer configuration of an appropriate Shared NG-U Termination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Query RB Configuration | M |  | ENUMERATED (true, ...) |  |

Not modified

- 9.3.3.26	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| BC Bearer Context NG-U TNL Info at 5GC | M |  | 9.3.1.112 |  |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.114 |  |
| Consent to Apply Available Shared UP MBS QoS flow mapping | O |  | 9.3.1.115 | Consent to Apply Available Shared NG-U Termination’s MBS QoS flow mapping |
| Query RB Configuration | O |  | 9.3.1.x |  |

#### 9.3.3.27	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | C-ifunicast |  | 9.3.1.116 |  |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifunicast | This IE shall be present if the MBS NG-U Information at NG-RAN IE within the BC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information. |

Not modified

- 9.3.3.32	MC Bearer Context To Setup

This IE contains MBS session resource related information used to request MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| MC MRB To Setup List | M |  | MC MRB Setup Configuration 9.3.1.120 |  |
| Consent to Apply Available Shared UP MBS QoS flow mapping | O |  | 9.3.1.115 | Consent to Apply Available Shared NG-U Termination’s MBS QoS flow mapping |
| Query RB Configuration | O |  | 9.3.1.x |  |

#### 9.3.3.33	MC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN | C-ifunicast |  | 9.3.1.121 |  |
| MC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.120 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifunicast | This IE shall be present if the MBS NG-U Information at NG-RAN IE within the MC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information. |

Not modified

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

- -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

- -- B

- -- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC,

bcMRBToSetupList						BCMRBSetupConfiguration,

consent									ConsentApplyAvailSharedUPMBSQoSFlowMapping		OPTIONAL,

queryRBConfiguration					QueryRBConfiguration		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Not modified

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

ConsentApplyAvailSharedUPMBSQoSFlowMapping ::= ENUMERATED {

allowed,

...

}

Not modified

MCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

mcMRBToSetupList						MCMRBSetupConfiguration,

consent									ConsentApplyAvailSharedUPMBSQoSFlowMapping		OPTIONAL,

queryRBConfiguration					QueryRBConfiguration		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Not modified

QueryRBConfiguration ::= ENUMERATED {

true,

...

}

End of changes