3GPP TSG-RAN WG3 Meeting #116-e	R3-223693

E-meeting, May 9-May 19, 2022

Agenda Item:	9.1.6.2

Source:	Qualcomm Incorporated (moderator)

Title:	Summary of Offline Discussion on MBS ASN.1 correction

Document for:	Agreement

# Introduction

CB: # MBS4_ASN

- Check the details

- Approve the CRs if agreeable

(Qualcomm - moderator)

Summary of offline disc R3-223693

Phase 1 deadline: UTC 2AM, Thursday (May 12)

Phase 2 deadline: UTC 2AM, Monday (May 16)

# For the Chairman’s Notes

To be added

# Discussion

## XnAP

There are following change proposals [5] for XnAP.

- Make MRB-ID length extensible in ASN.1

MRB-ID ::= INTEGER (1..32,…)

- Change IE name in ASN.1 to align with tabular

- Change the IE name MBS-MappingandDataForwardingInfofromSource to MBS-MappingandDataForwardingRequestInfofromSourceNGRANNode to align with :

9.2.1.39	MBS Mapping and Data Forwarding Request Info from source NG-RAN node

- Change the IE name MBS-DataForwardingInfofromTarget to MBS-DataForwardingResponseInfofromTargetNGRANNode in ASN.1 section to align with

9.2.1.40	MBS Data Forwarding Response Info from target NG-RAN node

Question 1 : Which of above XnAP ASN.1 change proposals do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

## E1AP

Following correction proposals received in [6].

- Update the mbsNGUInformationAt5G in the LocationDependentMBSNGUInformationAtNGRAN-Item to mbsNGUInformationAtNGRAN.

- Update the locationindpendent to locationindependent.

- Update DRB-ID to MRB-ID for BCMRBRemoveConfiguration.

- Update DRB-ID to MRB-ID for MCMRBRemoveConfiguration.

- Update some sub-IEs of BCMRBModifyConfiguration-Item to optional to align with tabular.

BCMRBModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

bcBearerContextF1U-TNLInfoatDU 	BCBearerContextF1U-TNLInfoatDU	OPTIONAL,

sdap-config						SDAP-Configuration				OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

Question 2 : Which of above E1AP ASN.1 change proposals do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

## F1AP

Paper [3] proposed following changes to align with tabular.

- in MulticastContextSetupFailureIEs, the criticality of the id-gNB-DU-MBS-F1AP-ID should be ignore

- in MulticastDistributionSetupResponseIEs, the id-MulticastF1UContext-FailedToBeSetup-List should be ignore, and the presence should be optional

- in MulticastDistributionSetupFailureIEs, the criticality of the id-gNB-DU-MBS-F1AP-ID should be ignore, and the presence should be optional

- in BroadcastContextReleaseRequestIEs, id-Cause is missing

- in BroadcastContextModificationRequestIEs, id-MBS-ServiceArea is missing

- in BroadcastContextModificationRequestIEs, id-MBS-CUtoDURRCInformation should be mandatory

- in MBS-ServiceAreaInformationList, the MBS-AreaSessionID is missing

Question 3 : Which of above F1AP ASN.1 change proposals do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

## NGAP

A series of correction and optimization proposals are received in [1] [2] [4].

### ASN.1 and tabular mismatch issues

- MBS Area Session ID is missed in multiple locations

- Distribution Setup Request

- Distribution Setup Response

- Distribution Setup Failure

- Distribution Release Request

- Distribution Release Response

- Location dependent case of MBS-ServiceArea [2] and MBS-SessionTNLInfoNGRAN [1][2]

- [1] The MB-SMF related “transfer” IEs are defined as OCTET STRING in the tabular but not in the ASN.1, which means the MB-SMF containers are not transparent to the AMF as intended. Encoding of MB-SMF IEs changed to “OCTET STRING (CONTAINING xxxTransfer)”

- [2] Change the MBSSessionInformationFailureTransfer IE in BroadcastSessionModificationFailureIEs from mandatory to optional, to align with tabular, and align with Setup failure case.

- [4] Delete the id-Alternative-SharedNG-U-Multicast-TNL-Information from MBS-DistributionSetupResponseTransferIEs

Question 4 : Which of above NGAP ASN.1 alignment change proposals do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

### ASN.1 optimizations

- [1] MBS-QoSFlows-ToBeSetupModList is removed. It was used only in the Multicast Session Update Request Transfer IE (while  MBS-QoSFlows-ToBeSetupList is used in the MBS Session Setup or Modification Request Transfer IE). Presumably the same list should be used in both IEs, so it is harmonzed to use MBS-QoSFlows-ToBeSetupList.

- [1] The ASN.1 encoding of Multicast Group Paging Area required correction since it did not match the tabular.

MBS-AreaTAIList ::= SEQUENCE (SIZE(1..maxnoofTAIforPaging)) OF TAI

MulticastGroupPagingArea ::= SEQUENCE {

mBS-AreaTAIList		MBS-AreaTAIList,

iE-Extensions		ProtocolExtensionContainer { { MulticastGroupPagingArea-ExtIEs} } OPTIONAL,

...

}

Question 5: Which of above NGAP ASN.1 optimization proposals do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

### Correction on the TNL address IE

The N3 tunnel shall be identified using UPTransportLayerInformation. But, some MBS IEs used TransportLayerAddress (which does not include TEID).

Two correction proposals received.

- [1] Change TransportLayerAddress in MBS-SessionTNLInfoNGRAN and MBS-SessionTNLInfoNGRANList with a new SEQUENCE (MBS-SessionTNLInfoNGRANSingle) including UPTransportLayerInformation.

- [2] Change the TransportLayerAddress in MBS-SessionTNLInfoNGRAN and MBS-SessionTNLInfoNGRANList to UPTransportLayerInformation

Question 6: Which of the above two TNL address corrections do you prefer?

| Company | Answer | Comment |
| --- | --- | --- |

### Editorial corrections/optimizations

[1] and [4] proposed a series of editorial corrections.

- [1] 9.3.5.1: IEs renamed to avoid misuse of the words “Item” and “Transfer” in IE names.

- [1] 9.3.5.2: The MBS Session TNL Information 5GC Item IE is renamed (since names ending in Item are reserved for an item in a list). The new name is “Shared NG-U Multicast TNL Information IE” to align with its usage elsewhere (e.g. in the MBS Distribution Setup Response Transfer IE in 9.3.5.8). The IE description is modified due to the renaming of the IE.

- [1] 9.3.5.3, 9.3.5.5, 9.3.5.6: The MB-SMF related “transfer” IEs are renamed to match the NGAP messages that carry them. For these 3 IEs, they are reused over NGAP in both Setup and Modification messages.

- [1] General: tabular is aligned with ASN.1 where needed, e.g. “Item” level added in Lists.

- [1] Some editorial corrections in ASN.1/tabular, e.g. aligning naming with typical NGAP conventions, i.e., change number h in [1]

- [4] Rename AssociatedMBSQoSFlowInformationToBeSetupItem to  AssociatedQosFlowInformationToBeSetupItem.

Question 7: Which of the above editorial changes do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

### Other corrections

Miscellaneous ASN.1 correction proposed in [1]:

- [1] 9.3.5.5, 9.3.5.6, 9.3.5.8: Criticality columns are removed since these “transfer” IEs are not carried in an NGAP “request” message terminating at the NG-RAN node.

- [1] 9.3.5.13: Criticality columns are added since the Multicast Session Update Request Transfer IE is carred in an NGAP “request” message terminating at the NG-RAN node.

- [4] The “gNB” in call flows in section 8.17.2.3 and section 8.17.3.2 shall align with the other parts in TS(“NG-RAN node”).

Question 8: Which of the above correction do you agree with?

| Company | Answer | Comment |
| --- | --- | --- |

# Conclusion, Recommendations

If needed

# References

- R3-223055 NGAP ASN.1 review for MBS (Nokia, Nokia Shanghai Bell)

- R3-223079 NR MBS NGAP asn.1 correction (Huawei, CBN, Qualcomm Incorporated)

- R3-223080 NR MBS F1AP asn.1 correction (Huawei, CBN)

- R3-223607 Correction on MBS ASN.1 for 38413 (ZTE Corporation)

- R3-223608 Correction on MBS ASN.1 for 38423 (ZTE Corporation)

- R3-223630 NR MBS E1AP asn.1 correction (Huawei)