3GPP TSG-RAN WG3 Meeting #116-e                                                              R3-223745

- Online, 9-19 May 2022

Agenda Item:	9.3.9

Source:	Huawei (moderator)

Title:	Summary of offline discussion on EHC parameters

Document for:	Discussion

# Introduction

CB: # 52_EHC

- Agree the updates in Tabular and ASN.1

- Rewording or remove the procedure text?

(HW - moderator)

Summary of offline disc R3-223745

# For the Chairman’s Notes

Propose the following:

# Discussion

## Tabular and ASN.1

Moderator’s note: There seems to be a consensus that the tabular and ASN.1 in [1] and [2] are acceptable. In this part of CB, we want to check the views of companies, whether we agree the tabular and ASN.1 in [1] and [2].

Please provide your views and comments in the table, if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

## Discussion on the procedure text

Moderator’s note:  During the online session, some companies have concerns on the current procedure text. The main argument is EHC header compression protocol can be configured for uplink and downlink independently. Therefore, it is not appropriate to include the word ‘only’ when describing the behavior of gNB-CU-UP. Correspondingly, the following rewording is suggested:

If the EHC Downlink IE is included in the EHC parameters IE contained in the BEARER CONTEXT SETUP REQUEST message and the value of drb-ContinueEHC-DL IE is set to ‘true’, the gNB-CU-UP shall, if supported, configure Ethernet header compression for downlink and continue the downlink EHC header compression protocol as specified in TS 38.331 [10]. If the EHC Downlink IE is included in the EHC parameters IE contained in the BEARER CONTEXT SETUP REQUEST message and the value of drb-ContinueEHC-DL IE is set to ‘false’, the gNB-CU-UP shall, if supported, configure Ethernet header compression for downlink and reset the downlink EHC header compression protocol during PDCP re-establishment as specified in TS 38.331 [10].

If the EHC Uplink IE is included in the EHC parameters IE contained in the BEARER CONTEXT SETUP REQUEST message and the value of drb-ContinueEHC-UL IE is set to ‘true’, the gNB-CU-UP shall, if supported, configure Ethernet header compression for uplink and continue the uplink EHC header compression protocol as specified in TS 38.331 [10]. If the EHC Uplink IE is included in the EHC parameters IE contained in the BEARER CONTEXT SETUP REQUEST message and the value of drb-ContinueEHC-UL IE is set to ‘false’, the gNB-CU-UP shall, if supported, configure Ethernet header compression for uplink and resets the downlink EHC header compression protocol during PDCP re-establishment as specified in TS 38.331 [10].

The moderator would like to check whether companies are fine with the above rewording with the word ‘only’ removed. If there are other rewording suggestions, please also share it in the comments.

- Please provide your views for whether agree the text proposal in [7] and fill the comments in the table, if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

# References

- R3-223231, “Correction on EHC parameters”, Huawei, et al.

- R3-223232, “Correction on EHC parameters”, Huawei, et al.