3GPP TSG-RAN WG3 Meeting #116-e	R3-223873

# Online, 9th – 19th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draftCR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 corrections for NR Positioning Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New information has been added over NRPPa that should be reflected in Stage 2. - Assistance data that may be transferred from gNB to LMF: - TRP type (all methods) - On-demand DL-PRS information (Multi-RTT, DL-AoD, DL-TDOA) - TRP Tx TEG association information (Multi-RTT, DL-TDOA) - TRP beam antenna information (DL AoD) - UE configuration data that may be transferred from serving gNB to LMF: - UE Tx TEG association information (UL-TDOA) - Measurement results that may be transferred from gNBs to LMF: - UL-SRS-RSRPP (Multi-RTT, UL-TDOA, UL-AoA) - Multiple UL AoA/ZoA (Multi-RTT w/ NOTE 1, UL-TDOA w/ NOTE 1, UL-AoA) - UL Angle of Arrival (azimuth and/or elevation) (Multi-RTT w/ NOTE 1, UL-TDOA w/ NOTE 1, UL-AoA) - SRS Resource Type (Multi-RTT w/ NOTE 1, UL-TDOA w/ NOTE 1, UL-AoA) - LoS/NLoS information for each measurement (Multi-RTT, UL-TDOA, UL-AoA) - ARP ID of the measurement (Multi-RTT, UL-TDOA, UL-AoA) - NOTE 1: When used with UL-AoA for hybrid positioning - TRP measurement request information that may be transferred from LMF to gNBs: - Expected UL AoA/ZoA and uncertainty range (Multi-RTT, UL-TDOA, UL-AoA) - Number of TRP Rx TEGs (Multi-RTT, UL-TDOA) - Number of TRP RxTx TEGs (Multi-RTT) - Measurement characteristics request indicator (Multi-RTT, UL-TDOA, UL-AoA) - Measurement time occasions for a measurement instance (Multi-RTT, UL-TDOA, UL-AoA) - Response Time (Multi-RTT, UL-TDOA, UL-AoA) - Search window information (Multi-RTT, UL-TDOA) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above information is added to the appropriate tables in section 8.10 (Multi-RTT), 8.11 (DL-AoD), 8.12 (DL-TDOA), 8.13 (UL-TDOA) and 8.14 (UL-AoA). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 is incomplete because it does not include information introduced in NRPPa in Release 17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.2.3, 8.10.2.4, 8.11.2.3, 8.12.2.3, 8.13.2.1, 8.13.2.2, 8.13.2.3, 8.14.2.2, 8.14.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modification

## 8.10	Multi-RTT positioning

** unchanged text skipped **

#### 8.10.2.3	Information that may be transferred from the gNB to LMF

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.10.2.3-1.

Table 8.10.2.3-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information

TRP Tx TEG association information

The configuration data for a target UE that may be transferred from the serving gNB to the LMF is listed in Table 8.10.2.3-2.

Table 8.10.2.3-2: UL information/UE configuration data that may be transferred from serving gNB to the LMF

UE configuration data

UE SRS configuration

SFN initialization time for the SRS configuration

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurement

UL-SRS-RSRP

UL-SRS-RSRPP

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type NOTE 1

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

NOTE 1: When used with UL-AoA for hybrid positioning.

#### 8.10.2.4	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-1.

Table 8.10.2.4-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

Periodicity of the SRS for each SRS resource set

SSB Information

Carrier frequency of SRS transmission bandwidth

The TRP measurement request information that may be signalled from the LMF to the gNBs is listed in Table 8.10.2.4-2.

Table 8.10.2.4-2: TRP Measurement request information that may be transferred from LMF to gNBs.

Information

TRP ID, and NCGI of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Number of TRP RxTx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-3.

Table 8.10.2.4-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS - Aperiodic SRS Resource Trigger List - Activation Time

UL-SRS: - Release all

Next Modification

## 8.11	DL-AoD positioning

** unchanged text skipped **

#### 8.11.2.3	Information that may be transferred from the gNB to LMF

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.11.2.3-1.

Table 8.11.2.3-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN, and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information

TRP beam antenna information

Next Modification

## 8.12	DL-TDOA positioning

** unchanged text skipped **

#### 8.12.2.3	Information that may be transferred from the gNB to LMF

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.12.2.3-1.

Table 8.12.2.3-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN, and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information

TRP Tx TEG association information

Next Modification

## 8.13	UL-TDOA positioning

** unchanged text skipped **

#### 8.13.2.0	Assistance Data that may be transferred from the gNB to the LMF

The assistance data that may be transferred from the gNB to the LMF is listed in Table 8.13.2.0-1.

Table 8.13.2.0-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, and TRP IDs of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

#### 8.13.2.1	Configuration Data that may be transferred from the gNB to the LMF

The configuration data for a target UE that may be transferred from the serving gNB to the LMF is listed in Table 8.13.2.1-1.

Table 8.13.2.1-1: UE configuration data that may be transferred from serving gNB to the LMF

UE configuration data

UE SRS configuration

Timing information of the TRP, which configured the UE SRS transmission

UE Tx TEG association information

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA

UL-SRS-RSRP

UL-SRS-RSRPP

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type NOTE 1

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

NOTE 1: When used with UL-AoA for hybrid positioning.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

Next Modification

## 8.14	UL-AoA positioning

** unchanged text skipped **

#### 8.14.2.0	Assistance Data that may be transferred from the gNB to the LMF

The assistance data that may be transferred from the gNB to the LMF is listed in Table 8.14.2.0-1.

Table 8.14.2.0-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, and TRP IDs of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

#### 8.14.2.1	Configuration Data that may be transferred from the gNB to the LMF

The configuration data for a target UE that may be transferred from the serving gNB to the LMF is listed in Table 8.14.2.1-1.

Table 8.14.2.1-1: UE configuration data that may be transferred from serving gNB to the LMF

UE configuration data

UE SRS configuration

Timing information of the TRP, which configured the UE SRS transmission

#### 8.14.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results are listed in Table 8.14.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.14.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL Angle of Arrival (azimuth and/or elevation)

Multiple UL Angle of Arrival (azimuth and/or elevation)

SRS Resource Type

UL-SRS-RSRP

UL-SRS-RSRPP

Time stamp of the measurement

Quality for each measurement

Beam information for each measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

#### 8.14.2.3	Information that may be transferred from the LMF to gNB

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.14.2.3-1.

Table 8.14.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.14.2.3-2.

Table 8.14.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Expected UL AoA/ZoA and uncertainty range

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.14.2.3-3.

Table 8.14.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger list - Activation time

UL-SRS: - Release all