3GPP TSG-RAN WG3 #117-e	R3-225026

Online, 15 – 24 August 2022

Agenda Item:	21.2

Source:	NTT DOCOMO (moderator)

Title:	Summary of Offline Discussion on CB: # CPResiliency_FailureCase

Document for:	Approval

Introduction

CB: # CPResiliency_FailureCase

- Discuss on failure scenarios associated with the gNB-CU-CP, based on current NG-RAN architecture

- Discuss, draft and agree conclusions for the study, for inclusion in the TR.

(NTTDocomo - moderator)

Summary of offline disc R3-225026

The 1st round of discussion is set to deadline on 17th Aug (Wednesday) 12:00 UTC.

The 2nd round of discussion is set to deadline on 19th Aug (Friday) 23:59 UTC.

For the Chairman’s Notes

Proposal 1: Approve the Draft TR for the description of "4. General".

Proposal 2: All the Failure scenarios have been covered by current draft TR.

Proposal 3: To achieve fast gNB-CU-CP recovery, following two mechanisms are needed.

Mechanism 1: Reduce UP-interruption time

Mechanism 2: Fast gNB-CU-CP-recovery

Proposal 4: If Mechanism 1 is agreed, a mechanism involving UEs to reduce UP-interruption is also suggested to be considered.

Proposal 5: To discuss the each “Company view” are listed in “5. Study failure scenarios associated with the gNB-CU-CP” or in informative Annex.

Proposal 6: Approve the Draft TR for the description of “5. Study failure scenarios associated with the gNB-CU-CP”.

Proposal 7: Approve the Draft TR for the description of “Annex A”.

Proposal 8: To discuss whether separate study recommended to investigate possible solutions and potential alternatives can be put into the conclusions.

Proposal 9: Approve the Draft TR for the description of “6. Conclusion”.

Discussion (1st round)

To complete this SI, we will begin with a chapter-by-chapter discussion of the TR descriptions.

Failure scenarios

Failure scenarios to be discussed

In each company's contribution, the failure scenarios of the gNB-CU-CP to be considered is mentioned.

- In [1], it is mentioned that a failure for the entire gNB-CU-CP should be discussed.

- In [5][11][14], these are mentioned that the need for geo-redundancy needs to be discussed. In [5], it is mentioned that cases requiring fast recovery need to be considered.

- In [3], it is mentioned that a mechanism for adjacent nodes to automatically switch to a redundant gNB-CU-CP should be considered.

- In [8][13], these are mentioned that the reduction of UP interruption time for UEs should be discussed.

As described above, the Failure scenarios to be considered in the future are "a failure for the entire gNB-CU-CP" and "geo-redundancy" from the perspective of "fast recovery".

Question 1-1: Do you agree that the two scenarios to consider are "a failure for the entire gNB-CU-CP" and "geo-redundancy"?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | No for geo-redundancy | We disagree to make entire CU-CP failure and geo-redundancy the main, or even worse, the exclusive focus for the study. Furthermore, Geo-redundancy is not related to the architecture itself but results from the operator’s deployment and strategy. |
| Deutsche Telekom | Yes, but | We have to differentiate. In the study we can consider all possible failure scenarios related to the CU-CP. We see the worst-case failure scenario in a total outage of the logical CU-CP node or of a central site where a CU-CP is deployed. The 2 deployment scenarios we described in our paper are more related to the possible impact such a failure has and based on deployment on possible solutions. Geo-redundancy means that a different approach for a solution to be established can be defined compared to a deployment without geo-redundancy. |
| Samsung |  | We’re fine with capturing all possible failure scenarios in TR. |
| KDDI |  | We’re fine with capturing all possible failure scenarios in TR, both entire gNB-CU-CP failure and limited gNB-CU-CP failure. Most people don’t care about that but in our understanding geo redundancy is not a scenario but a solution to address entire gNB-CU-CP failure. |
| Nokia | Yes | These are the scenarios with will have the most impact. However, the study should not be limited to only certain ones and capture all the described ones by the different companies. |
| NTT Docomo | Yes | Sorry for confusion. We think all scenarios should be captured in TR. And then, we can recommend that the solution for the failure scenario for "a failure for the entire gNB-CU-CP" and "geo-redundancy" with “fast recovery” view. Because there is no solution with multi-vendor environment for operator in listed failure scenarios. |
| ZTE | Yes | OK to capture the possible failure scenarios in TR. |
| NEC |  | Geo-resiliency is a solution rather a scenario |
| Intel | Yes | Same view with Samsung. |
| LG Uplus | Yes | We are fine with all scenarios. |
| Huawei | No for geo-redundancy | We are fine to capture failure scenarios, but we are not sure what geo-redundancy failure means, as E/// commented, this should be up to deployment… |

Conclusion for 1-1:

8 companies preferred to capture all scenarios in TR.

4 companies pointed out that "geo-redundancy" is not a scenario, but only a "solution" or "deployment," and is not suitable for scenario discussion.

Question 1-2: Regarding "fast recovery", [8] and [13] focus their discussion on U-plane interruption time. Would "fast recovery" mean improving "u-plane interruption time"?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | Yes, but why is this question needed? | Unclear what this question aims to clarify. It seems obvious. |
| Deutsche Telekom | Yes | Fast recovery from our perspective means minimization of UP interruption time for the UEs connected to the failed CU-CP. |
| Samsung | Yes | We don’t understand the intention with this question well. We think “fast recovery” includes the minimization of “u-plane interruption time”. |
| Nokia | Yes | Same view as Deutsche Telekom |
| NTT Docomo | Yes | Same view as Deutsche Telekom. We wanted to make sure that the companies have same view for definition of “fast recovery”. According to the answers so far, it seems that minimizing "u-plane interruption time" is a common goal of ours. |
| ZTE | Yes, but | Same concern as Ericsson and Samsung |
| NEC | Yes | We agree to identify a timer on U-plane interruption, the resiliency solution can be activated in case the timer on U-plane interruption expires. |
| Intel | Yes | Same view as DT. |
| LG Uplus | Yes | Same view as Deuthch Telekom |
| Huawei | See comment | Not sure why we should discuss this question, we should focus on failure scenarios… |

Conclusion for 1-2:

All companies have the same understanding of "fast recovery", which is to minimize "u-plane interruption time".

Question 1-3: Please, indicate, if you need other perspectives to discuss.

| Company | Comment, if there are any other perspectives |
| --- | --- |
| Ericsson | none |
| Deutsche Telekom | No |
| Nokia | No |
| Huawei | No |

Conclusion for 1-3:

No company had any new points to discuss regarding the failure scenario. However, Question 1-1 was not clear, and we will check again in the second round to confirm whether there are any points of view that were missing.

Necessity of discussion in standardization

There are differing opinions among the companies as to whether standardization is necessary.

- In [11] [13] [15], the proposals take the position that no new specification is needed. 
They mention that resiliency can be ensured by using virtualization techniques and can be addressed within the scope of the current TS38.401 specification. In [15], it is also mentioned that mechanisms such as AMF recovery are not applicable to RAN.

- In [1][3][5][14], the proposals take the position that a solution should be studied in RAN3.
They mention that the current specification lacks a solution for multi-vendor environments and that this needs to be considered. In #3, failure detection and synchronization are discussed. In #5, the need for quick recovery mechanisms such as mirroring is mentioned.

- In [8], the proposals take the position that a solution should be studied in RAN2 & RAN3.
This mentions the need for improvements from the UE perspective and the needs to consider cooperative ehaviour not only between nodes but also with Ues.

Specific solutions are out of scope in this SI. Therefore, at the conclusion of the discussion, it is sufficient to make a recommendation on the approximate scope of the solution.

Thus, we now have three alternatives for conclusions in this SI discussion

Alt.1: No new specification is needed.

Alt.2: A solution should be studied in RAN3.

Alt.3: A solution should be studied in RAN2 & RAN3.

Question 2-1: In [1][3][5][14], these are mentioned that standardization is necessary for a multi-vendor environment. Do you agree on this point?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | Yes but this is not the point | This question is irrelevant, because solutions are out of SI scope. |
| Deutsche Telekom | Yes | Generally, all RAN3 specifications addressing a split architecture for the RAN should be for a multi-vendor environment. Any specified resiliency solution should also consider that basic requirement. With the note in TS 38.401 “For resiliency, a gNB-DU may be connected to multiple gNB-Cus by appropriate implementation” this is in principle only addressed for the case that CU and DU are from the same vendor. From DT’s perspective the conclusion of the SI should not be on a specific solution but RAN3 should recommend for which of the failure scenario(s) listed in the TR specified solutions should be possibly considered (final decision of RAN Plenary). |
| Samsung | Yes, but | If needed after study, the standardization work could follow. At this time, it’s unclear what and how would be enhanced. |
| Nokia | Agree | At this meeting, the study is not focusing on listing what “solutions” to pursue with our without spec changes. However, it is fully valid to recommend to further study solutions for the failure scenarios compiled at this meeting. |
| NTT Docomo | Yes | Same view as Deutsche Telekom and Nokia |
| ZTE |  | Agree with Ericsson that there is no need to discuss the solution in the SI. |
| NEC | Yes |  |
| Intel | Yes | Same view as Nokia. |
| LG Uplus | Yes | Same view as Nokia |
| Huawei | See comment | Similar view as Ericsson |

Conclusion for 2-1:

The 8 companies feel the need for standard specifications. However, some companies believe that the need is unclear at this stage, when solutions are not being discussed.

The 2 companies mentioned that it is out of the scope of this SI to discuss whether standardized specifications are necessary in a multi-vendor environment, as this would be within the scope of the solution discussion.

Moderator thinks there is a need to discuss solutions first.

Question 2-2: In [8], it is mentioned that any mechanisms involving the UE can contribute to reduce the UP interruption time. Do you agree on this point?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | Yes | As explained in our paper, there is currently no way for a UE to detect the “absence” of RRC messages, hence this may lead to a lengthy loss of UP. This is a failure case which should be investigated at some point with the help of RAN2 (but we also note that, strictly speaking, it is not in the current SI scope). |
| Deutsche Telekom | Yes | But for us it is still unclear how the detection of the absence of RRC messages on the UE side will help to reduce the outage time of the CU-CP if there is only a single CP anchor available. But this is already a discussion on the solution space which is not in focus of the SI. |
| Samsung | Yes | How to work is unclear yet, but UE could be involved to reduce the UP interruption time. |
| KDDI | Yes | We are also fine to explore the UE involvement, but it is not RAN3 scope. We are not sure whether RAN3 can recommend something on RAN2 future work. |
| Nokia | Neither agree or disagree | This meeting is to discuss failure scenarios, and not what solution is needed. The mechanism to detect a failure (either from the network side or the UE side) falls in our view as part of the potential solutions discussion. |
| NTT Docomo | Yes | Same view as DT. We still are not sure about the RAN2 approach. However, I hope that will be clarified during the Solution discussion, not in this SI. |
| ZTE | Yes, but | The UE involved mechanism needs to checked by RAN2. |
| NEC | YES | But not in the scope of RAN3 but RAN2. |
| Intel | Yes or No | Same view as Nokia. |
| LG Uplus | Yes | Same view as Deuthsche Telekom |
| Huawei | No | Are we talking about potential solution, it is out of SI scope; in addition, how this conclusion could come, why UE involvement would improve interruption time, seems there has been no analysis so far… |

Conclusion for 2-2:

The 8 companies say that mechanisms including UEs will help reduce UP interruption time. However, details need to be confirmed in the SOLUTION discussion. It is also pointed out that this is outside the scope of RAN3 since it is within the scope of RAN2.

The 2 companies claim that it is outside the scope of RAN3 because it is a discussion of solutions.

Question 2-3: Which is the agreeable alternative?

| Company | Agreeable alternative | Comment |
| --- | --- | --- |
| Ericsson | OUT OF SCOPE | As already mentioned, discussion about solutions is out of SI scope. |
| Deutsche Telekom |  | Unclear what is meant by this question. Alternative to what? To that mechanism mentioned in Q2-2 |
| Samsung |  | It’s not easy to conclude the alternative approach in this phase. |
| NTT Docomo |  | The conclusion of this SI is to recomend in the TR's conclusion that the study of a solution in RAN2/RAN3 was proposed as the next step. At this stage, when solutions are not discussed, do you think there is any problem with recommending all possible solutions? (Of course, RAN plenary will make the decision.) |
| Huawei |  | Agree with E///. |

Conclusion for 2-3:

No conclusion for this.

Description for TR

General description

[2, 5,  8, 14] contain Text Proposals for "General" descriptions, which describe the background and assumed structure of this SI. These contributions cite the potential single point of failure as an issue with respect to the connection of a single gNB-CU-CP to multiple gNB-CU-UPs and gNB-DUs, suggesting that the impact of a failure would be significant.

Since there is no conflict between these descriptions, the contents of each Text Proposal have been merged and included in the Draft TR.

Please comment on this.

Question 3-1: Do you agree with merging the TPs of the various companies?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | Yes with comments | [2]: this is mostly agreeable, but recovery actions seem out of SI scope and it is questionable whether they should be included in the TR. the considered failure scenarios can used as base. Scenarios (A), (B) and (C) should be aggregated in a single “Node failure” scenario as the differentiation is on the reasons, and it does not seem to have an impact on the subsequent recovery actions. In any case, further discussion in a potential subsequent study could involve RAN2 as discussed in our paper. [5]: Proposals are in general agreeable, with the exception of scenario DS1. Some observations: scenario DS2 (TNLAs established to multiple CUs) is a deployment choice enabled by the current standard. On the other hand, DS1 (DU connected to a single CU only) deliberately avoids using an already existing (and specified) solution, so we believe it’s not a valid scenario. All other proposals seem to refer to deployment scenarios rather than failure cases. [7]: This is the TR skeleton – no issue here. You could perhaps include a reference to the SID in RP-221438. |
| Deutsche Telekom | Yes | For us the merging is fine. Comment to Ericsson with respect to DS1: Why is DS1 not agreeable? We have areas in our networks where antenna sites are connected only to a single central location, i.e., geo-redundancy as described in DS2 is not feasible in such cases. |
| Samsung | Yes | Merging is fine. |
| KDDI | Yes | Merging is fine. |
| Nokia | Agree | Merging is fine |
| NTT Docomo | Yes | Sorry, [7] is wrong reference. We collected it to [14]. Comment to Ericsson with respect to “multi-vendor environment”: Is it your opinion that the following statements can be supported in a multi-vendor environment? For resiliency, a gNB-DU may be connected to multiple gNB-CUs by appropriate implementation. I am thinking that at least some additional specifying is needed. |
| ZTE | Yes | Merging is fine. |
| NEC | Yes | Merging is fine. |
| Intel | Yes | Merging is fine. |
| LG Uplus | Yes | Merging is fine. |
| Huawei | See comments | Merging is fine, but we don’t think DS2 is needed, since it looks just one proprietary deployment, while we think we just need to capture typical deployment which already indicated in Figure 4-1/2 |

Conclusion for 3-1:

All companies agree to merge.

At several points, there are arguments against the description, so these points will be continued.

Question 3-2: If Q1 is yes, do you have any comments for the description of DRAFT TR?

| Company | Comment, if there are any objections |
| --- | --- |
| Ericsson | See above. We should try to set a principle where we discuss failure cases for the node itself, as opposed to impacts of failure in different deployments. Otherwise, a single meeting will not be enough. |
| Nokia | Agree with the draft TR and structure |
| Huawei | See comments above. |

Conclusion for 3-2:

There was discussion from Ericsson that differences by deployment should not be included.

However, the difference in deployment is a point that could affect the consideration of a failure scenario, and we would like to continue the discussion.

Question 3-3:  If Q1 is No, What is your assumed description?

| Company | Comment, if there are any objections |
| --- | --- |

Conclusion for 3-3:

No conclusion for this.

Failure Scenario

Regarding the Failure Scenario, each company has described it from various perspectives. Therefore, it appears that it would be better to describe all the TPs proposed by each company as they are described in DRAFT.

Regarding the points where the companies have different opinions, after discussion at this meeting, it is described in the “Conclusion” section.

Question 4-1: Do you agree with merging the TPs of the various companies?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| Ericsson | Yes with some exceptions | Not everything should be captured; proposed changes are on the draft itself |
| Deutsche Telekom | Yes | In principle, we could merge similar failure scenarios from different companies into a single aligned description, but this is hard to achieve within a single meeting. Because of that we are fine with separately listing the failure scenarios addressed by different companies. |
| Samsung | Yes | Merging similar failure scenarios into single section and description would be good. The failure scenarios could be list-up with some description and the similar failure scenario from different companies could be merged. |
| KDDI | Yes | We share the view with Deutsche Telekom, separately listing is a good way, since we don’t have enough time. |
| Nokia | Yes | Given there is only single meeting for discussing the failure scenarios, it may not be possible to have same level of detail aligned from all contributions in regard to text structure itself. Thus, in similar view as Deutsche Telekom, the scenarios contributed should be listed and described even if some have more/less level of detail compared to others. |
| ZTE | Yes | OK for merging similar failure scenarios. |
| NEC | Yes |  |
| Intel | Yes | Agree with DT and Samsung. |
| LG Uplus | Yes |  |
| Huawei | Yes to merge | But we need to see concrete descripitons. |

We do not believe that much discussion is necessary regarding the description of each company's Failure Scenario, as it is the result of each company's own study.

Question 4-2: Please indicate, if you disagree with this clarification, what should be discussed about the Failure scenario if any?

| Company | Comment |
| --- | --- |
| Ericsson | As far as we talk about failure scenarios only, this is agreeable. |
| Deutsche Telekom | We are ok with that approach. |
| Nokia | OK with the proposal |
| Huawei | ok |

Conclusion for 4-1 and 4-2:

In principle, similar scenarios should be merged, but some commented that in this SI, where time is limited, the moderator should just make a listing.

There is also a comment that not all scenarios should be included, and we would like to continue the discussion on this point in the drafted TR.

Conclusion

The Conclusion section is expected to describe the Failure scenario and scope to be considered, based on the answers to Questions 1-1 through 2-3 in this document.

Question 5-1: Please indicate, If other points of view need to be included for the "Conclusion" section.

| Company | Comment |
| --- | --- |
| Ericsson | Disagree. As with any TR in output from a SI, technical aspects are discussed in the TR body, not the conclusions. The Conclusions section should contain guidance for RAN, if agreeable by RAN3, for potential future discussion and action. |
| Deutsche Telekom | From DT’s perspective the conclusion section should include a recommendation (if agreeable) for which of the failure scenario(s) listed in the TR specified solutions should be possibly considered (final decision of RAN Plenary). |
| Samsung | We think the conclusion should include the summary of the failure scenarios and the recommendation for potential future work. |
| KDDI | Share the view with Samsung. If we conclude that there are failure scenarios, then potential future work is explore solutions for those failure scenarios. At this moment, we cannot conclude that all listed failure scenarios can be addressed by appropriate implementation noted in TS38.401, so we need further work. |
| Nokia | The conclusion should be limited to guidance/recommendation from RAN3 recommendation regarding the study, and not be the technical discussion. |
| NTT Docomo | We are considering the following conclusion statement if agreeable. - This study allowed us to analyze the cases where failures occur with the current architecture. (Depends on Q1-0) - As a result, we have identified certain cases for which some solution should be considered. (Depends on Q1-1 and Q1-2) - Therefore, we recommend that the solution be considered in RAN2/RAN3 in the next SI/WI in the future. (Depends on Q2-1~Q2-3) |
| ZTE | Agree with the majority that the recommendation from RAN3 is needed. |
| NEC | The conclusion can include the summary of the failure scenarios and suggested way forward for the study. |
| Intel | Same view with Samsung, Nokia, and NEC |
| LG Uplus | Same view with Samsung |
| Huawei | No. similar view as E///. |

Conclusion for 4-3:

In conclusion in TR, Describe the recommendations for the next step. Discussion on the contents will be held in the 2nd round of TR.

Discussion (2nd round)

Failure Scenario

In the 1st round, I missed to ask the most basic question.

In the contributions of each company, various perspectives are discussed as follows. We believe that the study of the failure scenario in this SI can be regarded as complete when all the opinions of each company are captured in the TR.

- In [2], failure scenarios for the entire gNB-CU-CP are studied, and available recovery actions are summarized for each.

- In [4], the failure condition is discussed, and it is recommended that switching be done before completely crashed.

- In [5], possible failure scenarios in centralized gNB-CU-CP deployments are studied based on two deployment scenarios.

- In [8], the impact on the C-plane when a failure occurs in the gNB-CU-CP, and the impact on the U-plane that follows it, are investigated.

- In [12], the impact of geo-redundancy on the current architecture is being considered.

- In [13], failure scenarios in virtualized environments and the impact when they occur are analyzed.

- In [14], it is summarized the failure scenarios that can occur in gNB-CU-CP in real environments.

- In [16], it is analyzed whether auto-recovery like AMF can be implemented in RAN. It also examines the usefulness of virtualization in the current RAN architecture and its impact in case of failure.

Question 6-1: Do you agree that it is all covered as failure scenarios to complete this SI? If not, please tell us about the missing failure scenario.

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| NTT Docomo | Yes | I don't see any more Failure scenarios to discuss. |
| Ericsson | Yes but | Once again, we have to repeat that solutions are out of SI scope. [14], for example, which we could not specifically comment on in the first round because of a wrong reference, contains solutions. This part should be expunged from the captured text. We note that TN aspects, other than the support for multiple TNLA associations, are out of 3GPP scope. This should be at least mentioned in the TR in the relevant places (a number of company contributions focus specifically on TN failures, which cannot be addressed by our specifications). Furthermore, also considering the limited time available for reviewing the TR content, we suggest moving all the “company views” currently captured in Sec. 5.x, to an informative Annex, removing company names. |
| KDDI |  | It seems extremely challenging to complete the work in this meeting, so I’m afraid but we may want to extend the SI one more quarter. Please find our comments below anyway. - Scenario vs Solution In TR, hopefully it’s better to be listed separately Scenario and Solution. But we are fine with the current TR descriptions if we stick to complete the work in this meeting. - Scenario descriptions We think the following information should be captured, especially failure impacts to other nodes may be import aspects for the further work. With the current TR, most of the information is well captured, so we are ok, if we close the work in this meeting. - Failure type: Software failure/Hardware failure/Power outage/Transport network failure - Failure trigger: Disaster/Human error/Maintenance - Failure impacts to other nodes: Whether failure is visible to other nodes (DU/AMF/UE/OAM…), invisible because the failure addressed by CU virtualization function? When CU recover from the failure? (After several minutes or hours?) No response to other nodes and required system reboot? Request other node to change TNLA associations? Others? - Conclusion We are fine with the current proposed wording, but if we mention failure impact something like emergency call is not available for several hours, then the issue seems to be more serious than just “UP interruption”. - Out of 3GPP scope I’m wondering if there was any past agreement relevant to Ericsson’s comment “We note that TN aspects, other than the support for multiple TNLA associations, are out of 3GPP scope.” |
| NTT Docomo | - | For Ericsson’s comment, Thanks for the comment. We would like to confirm whether we need further scenario. And we think we had a sufficient scenario and only needed to make the stated arrangements. We are fine to remove the description related to the solution. (We understand that this is related to the description of [2]/[12]/[14]/[16]. We are updated the Draft TR) About TN aspects, We are ok to add annotations as you wrote in your draft TR. It is also OK to move "company views" to Annex. We have updated the draft TR and would like you to check the description of each company. |
| NTT Docomo | - | For KDDI’s comment, 1. Scenario vs Solution As mentioned above, the solution is certainly outside the scope of this SI. It seems to me that it would be better to delete them and discuss them together in the next phase. 2. Scenario descriptions Capturing all scenario as Annex is proposed, how about this? 3. Conclusion You mean that “UP interruption” is not enough. How about followings? beneficial to further study and discuss improvement solutions to achieve fast recover CU-CP in case of emergency call, and to reduce UP interruption , potentially including UE aspects. |
| Huawei | See comments | In our understanding, the failure scenarios on the table should be enough. |
| Deutsche Telekom | Yes | To avoid lengthy discussion about the deployment scenarios which were taken from DT’s input we have made some updates in the TR to remove those scenarios together with related figures. For that some modifications were made in Sec. 4 as well as in the Annex on DT’s input. |
| ZTE | Yes | Seems to be ok after the clarification. |
| Nokia | Yes | However, the scenarios should be at the bare minimum listed in the body of the TR, even if the details are in the Annex as part of different views of the proponents of each TDoc. |
| Samsung | Yes | We don’t see more failure scenarios. |

Conclusion

Seven companies agreed that it is all covered as failure scenarios to complete this SI.

So it is proposed:

Proposal 2: All the Failure scenarios have been covered by current draft TR.

One company commented on the gNB-CU-CP recovery.

if we mention failure impact something like emergency call is not available for several hours, then the issue seems to be more serious than just “UP interruption”.

Also, UP interruption time is a necessary perspective, as discussed in Question 1-2.

So it is proposed:

Proposal 3: To achieve fast gNB-CU-CP recovery, following two mechanisms are needed.

Mechanism 1: Reduce UP-interruption time

Mechanism 2: Fast gNB-CU-CP recovery

And for Mechanism 1, a mechanism involving UE may be also beneficial, as discussed in Question 2-2.

Proposal 4: If Mechanism 1 is agreed, a mechanism involving UEs to reduce UP-interruption is also suggested to be considered.

Some companies are mentioned the following points in terms of how they would be described in the TR. Moderator would like to handle them in Draft TR discussion.

- The solution part should be expunged from the captured text.

- The most of TN aspects should be at least mentioned as out of scope in the TR in the relevant places

- moving all the “company views” currently captured in Sec. 5.x, to an informative Annex, removing company names.

- the scenarios should be at the bare minimum listed in the body of the TR

According to the comments for Question 2-1 and 2-2, at least, it seems that some improvement for fast recovery could be made, and that the solution should at least be discussed. Since the solution is outside the scope of the current SI, we think that establish a new SI/WI in order to discuss “solution”.

Question 6-2: Do you prefer to establish new SI or WI to discuss solution for Failure scenarios?

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| NTT Docomo | Yes |  |
| Ericsson | In principle yes, but this is not for RAN3 to discuss now | As can be seen in our TP for proposed conclusions, this is not for RAN3 to discuss now, but at best it should be a RAN3 recommendation for further discussion in RAN, if agreeable by RAN3. |
| KDDI | Yes |  |
| NTT Docomo | - | For Ericsson’s comment, We have same view. It was assumed that some would argue that a discussion of solutions is unnecessary, so we would like to hear the companies' assumptions. |
| Huawei | No | In our understanding, we don’t think from RAN3 perspective, RAN3 is able to suggest another SI or WI now, since the current SI just tries to address failure scenarios. However, it is up to proponent to raise the proposal in the plenary. |
| Deutsche Telekom | Yes, but | A decision of further work has to be made by the RAN Plenary, but from our perspective, it would be useful to have a recommendation in the conclusions, if agreed by RAN3, about failure scenario(s) listed in the TR for which solutions should be possibly considered in next step (SI or WI; decision by RAN Plenary). |
| ZTE | No | Share the view with Huawei. |
| Nokia | Yes | Based on proposals and contributions, it is clear in our view that companies differ on what a solution for a given failure is, and more importantly what the solution is for that failure. We hence support extension of the SI to further analyze the failure and enhancements or existing solutions (if any). |
| Samsung |  | We have the similar view as Ericsson. It could be discussed in RAN plenary. |

Conclusion

4 companies prefer to discuss on new SI/WI to discuss solution for Failure scenarios.
2 companies thinks it is too early to discuss now.
6 companies mentioned that it is up to RAN plenary discussion. And some companies also mentioned if agreeable, RAN3 recommendation is useful.

Therefore, it should be left to the Plenary to make a decision on the new SI/WI, but it would be good to make some recommendation on this SI.

Moderator would like to handle contents of “recommendation” in Draft TR discussion.

Draft TR

As with Round 1, we are uploading a Draft TR. If you propose changes, please update the Draft TR as well.

For “4. General” section in TR

In Question 3-1, all company agreed to merge. Some points are controversial and we would like to continue discussing those points on Draft TR.

Question 7-1: Do you agree with description on “4. General” section in draft TR? If not, please point this out and suggest changes.

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| NTT Docomo | Yes | - |
| Ericsson | Yes provided our comments are taken into account | Comments and changes are already provided on the draft itself. |
| Deutsche Telekom | Yes, but | As stated earlier, to avoid lengthy discussion about the deployment scenarios which were taken from DT’s input we have made some updates in the TR on Sec. 4 to remove those scenarios together with related figures. This required also some minor modifications in the Annex on DT’s input. |
| Nokia | Yes, provided our comments are taken into account | Comments given in the draft |

Conclusion

The following points were discussed, including discussions within the Draft TR

- Treatment of Resiliency of gNB-CU-CP in TS 38.401

- Deployment scenario

With respect to the above, no description is included to avoid lengthy discussions. Moderator believes that this will provide a consensus on the description. Therefore it is proposed that:

Proposal 1: Approve the Draft TR for the description of "4. General".

For “5. Study failure scenarios associated with the gNB-CU-CP” section in TR

Based on the comment for Question 4-1 and 4-2, moderator will be just listing from each company’s view. In terms of fairness, we assume that there is no need to make edits to each company's opinion.

Question 8-1: Do you agree with description on “5. Study failure scenarios associated with the gNB-CU-CP” section in draft TR? If not, please point this out and suggest changes.

| Company | Yes/No | Comment, if there are any objections |
| --- | --- | --- |
| NTT Docomo | Yes | I don't see any more Failure scenarios to discuss. |
| Ericsson | Yes with slight edits | We have taken the liberty of making small edits and adding comments where it seemed to make sense. TN aspects, other than the support for multiple TNLA associations, are out of 3GPP scope. This should be at least mentioned in the TR in the relevant places (a number of company contributions focus specifically on TN failures, which cannot be addressed by our specifications). Furthermore, also considering the limited time available for reviewing the TR content, we suggest moving all the “company views” currently captured in Sec. 5.x to an informative Annex, removing company names. |
| NTT Docomo | - | For Ericsson’s comment, About TN aspects, We are ok to add annotations as you wrote in your draft TR. It is also OK to move "company views" to Annex. We have updated the draft TR and would like you to check the description of each company. |
| Deutsche Telekom | Yes | We are fine with the shift of company inputs to the annex. We also agree that failure recovery for parts of the TN is out of 3GPP’s scope, the TR editor may state that appropriately in the text of Sec. 4 (?). But as this is visible by inputs of several companies, this has strong impact on CU-CP functionality and may be considered for later solution description (if agreed by RAN Plenary as a follow-up activity). Therefore we would propose to keep such failure scenarios at least in the lists provided by companies in the Annex. |
| Nokia | No | In our view, the failure cases should be at the bare minimum listed in this section, even if the detailed descriptions are in an annex. |
| Samsung |  | As Nokia commented, we also think the failure scenarios need to be listed in the body of TR, but after reviewing company’s contributions, it seems not easy in this meeting. So we’re fine with the updated description after moving company’s TPs into Annex. |

Conclusion

The following points were discussed, including discussions within the Draft TR

- Moving all the “company views” currently captured in Sec. 5.x, to an informative Annex, removing company names.

- The scenarios should be at the bare minimum listed in the body of the TR

Therefore, it is believed that the current description can be agreed. So it is proposed that:

Proposal 5: To discuss the each “Company view” are listed in “5. Study failure scenarios associated with the gNB-CU-CP” or in informative Annex.

- The solution part should be expunged from the captured text.

- The issue seems to be more serious than just “UP interruption”

- The most of TN aspects should be at least mentioned as out of scope in the TR in the relevant places

For above points, draft TR is already reflected. So moderator try to proposed that:

Proposal 6: Approve the Draft TR for the description of “5. Study failure scenarios associated with the gNB-CU-CP”.

Proposal 7: Approve the Draft TR for the description of “Annex A”.

For “6. Conclusion” section in TR

According to the comments for Question 2-1 and 5-1, recommendations should be made for Failure scenarios that require discussion of solutions if possible. Is it possible to narrow it down to "a failure for the entire gNB-CU-CP" for example?

Question 9-1: Can you identify scenarios that need to be discussed?  If yes, please suggest.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | Yes but | As discussed in our contributions, the "failure for the entire gNB-CU-CP" scenario risks overlooking the fact that the gNB-CU-CP in reality is not a monolithic “box”. We have given the example of virtualized implementations, where a total failure, while catastrophic, is also very rare. So, by making this simplification without putting it in context, we will be taking a too pessimistic assumption. |
| NTT docomo |  | For Ericsson’s comment, Thanks for the comment and add text in draftTR. I believe you understand my point: "For an operator, even if it is a rare case, if the impact is enormous, it cannot be overlooked." Of course, I understand that even in the current specification, consideration is given as an implementation. Therefore, we would like to make changes to the added text. |
| Deutsche Telekom | Yes | The total outage of the gNB-CU-CP deployed in a central location is seen as a worst case from our perspective. We agree that implementation specific aspects may reduce the risk of a total outage, but nevertheless it may occur also in combination with other failures that may be outside of RAN3’s scope (power outage, TN failure, etc.). |
| Nokia |  | Given that at this meeting we could not even discuss available recovery actions , it is important to recommend the study to analyze the failure scenarios provided at this meeting. |

Please give us any other comments on draft TR..

Question 9-2: Do you agree with description on “6. Conclusion” section in draft TR? If not, please point this out and suggest changes.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | Yes with changes | Our reasoning is explained in our answer to the previous question; consistently, we have taken the liberty of proposing some changes to the conclusions. |
| Deutsche Telekom | Yes with changes | We are generally fine with it, but we have made some minor changes in the Sec. 6 (see the uploaded TR version). |
| Nokia | Yes, with changes | Comments provided in the draft as what would be an acceptable wording |
| Samsung |  | Basically we’re fine with conclusion under discussion. We don’t have strong suggestion for the conclusion. |

Conclusion

The following points were discussed, including discussions within the Draft TR

- Treatment of Resiliency of gNB-CU-CP in TS 38.401

No description is included in the same as " 4. General".

- About the Recommendation in this TR

As mentioned in Conclusion 2-2, unless we discuss not only the Failure scenario but also the Solution, we cannot consider the necessity of standardization. Therefore, moderator thinks that the current description is appropriate, which states that the moderator will discuss existing and potential solutions without specifying scenarios.

Proposal 8: To discuss whether separate study recommended to investigate possible solutions and potential alternatives can be put into the conclusions.

Proposal 9: Approve the Draft TR for the description of “6. Conclusion”.

Conclusion, Recommendations [if needed]

If needed

References

R3-224280, Discussion on Failure Scenarios associated with gNB-CU-CP (Nokia, Nokia Shanghai Bell)

R3-224281, Text Proposal for gNB-CU-CP Failure Scenarios (Nokia, Nokia Shanghai Bell)

R3-224302, Discussion of gNB-CU-CP resiliency scenario (NEC)

R3-224303, TP for gNB-CU-CP resiliency scenario (NEC)

R3-224324, Discussion on failure scenarios associated with the gNB-CU-CP (Deutsche Telekom AG)

R3-224369, Work plan for Resiliency of gNB-CU-CP SI (NTT DOCOMO, INC.)

R3-224373, TR skeleton for Resiliency of gNB-CU-CP (NTT DOCOMO, INC.)

R3-224576, gNB-CU-CP Failure Case for Resiliency Study (Ericsson LM)

R3-224577, Final Considerations and Conclusions on gNB-CU-CP Resiliency (Ericsson LM)

R3-224582, Work plan for Resiliency of gNB-CU-CP SI (NTT DOCOMO, INC.)

R3-224626, Initial consideration on resiliency of gNB-CU-CP (ZTE)

R3-224627, TP to draft TR for resiliency of gNB-CU-CP (ZTE)

R3-224754, (TP for FS_gNB_CU_CP_resiliency for TR 38.xxx) Local Redundancy of Logical CU-CP (Samsung)

R3-224787, TP for TR enhancements for resiliency of gNB-CU (NTT DOCOMO INC.)

R3-224897, Discussions on enhancements for resiliency of gNB-CU-CP (Huawei)

R3-224898, Tentative TP for FS_gNB_CU_CP_resiliency on resiliency of gNB-CU-CP (Huawei)