3GPP TSG-RAN3 Meeting # 117-e	R3-225049

# Online, 15th - 24th   August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1897 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Masked IMEISV for UEs using CP CIoT EPS optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone, Qualcomm Incorporate, AT&T, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IMEISV value with a mask is used to identify a terminal model without identifying an individual Mobile Equipment. Currently, the eNB can get this information from MME via the INITIAL CONTEXT SETUP REQUEST or HANDOVER REQUEST messages. However, for UEs using Control Plane CIoT EPS optimisation, the eNB cannot get the “Masked IMEISV” from MME since AS security is not activated. One possible way to fix this issue is to add the Masked IMEISV IE into the UE INFORMATION TRANSFER, so that eNB can retrieve this IE from MME after msg3. Additionally, the masked IMEISV IE should also be added in the procedures related to CP CIoT EPS optimisation DL NAS TRANSPORT and the CONNECTION ESTABLISHMENT INDICATION messages to cover all signalling aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the Masked IMEISV IE in the UE INFORMATION TRANSFER, DL NAS TRANSPORT and the CONNECTION ESTABLISHMENT INDICATION messages from MME to eNB. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the changes only affect the CP CIoT EPS optimisation procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB cannot know the Masked IMEISV for UEs using CP only CIoT EPS solution |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.9.2, 8.3.11.2, 8.6.2.2, 9.1.4.20, 9.1.4.22, 9.1.7.2, 9.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0874 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Remove the updates to MME CP RELOCATION INDICATION message |  |  |  |  |  |  |  |  |

<Start of changes>

### 8.3.9	Connection Establishment Indication

#### 8.3.9.1	General

The purpose of the Connection Establishment Indication procedure is to enable the MME to complete the establishment of the UE-associated logical S1-connection, and/or trigger the eNB to obtain and report UE Radio Capability. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: Connection Establishment Indication procedure. Successful operation.

The MME initiates the procedure by sending a CONNECTION ESTABLISHMENT INDICATION message to the eNB.

If the UE-associated logical S1-connection is not established, the MME shall allocate a unique MME UE S1AP ID to be used for the UE and include that in the CONNECTION ESTABLISHMENT INDICATION message.

If the UE Radio Capability IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall store this information in the UE context, use it as defined in TS 36.300 [14].

If the Enhanced Coverage Restricted IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the DL CP Security Information IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall forward this information to the UE as described in TS 36.300 [14].

If the CE-Mode-B Restricted IE is included in the CONNECTION ESTABLISHMENT INDICATION message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the End Indication IE is included in the CONNECTION ESTABLISHMENT INDICATION message and set to "no further data", the eNB shall consider that there are no further NAS PDUs to be transmitted for this UE.

If the Subscription Based UE Differentiation Information IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [11].

If the UE Level QoS Parameters IE is contained in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall, if supported, store this information in the UE context, and use it as specified in TS 23.401 [11].

If the UE Radio Capability ID IE is contained in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall, if supported, use it as defined in TS 23.401 [11].

If the Masked IMEISV IE is contained in the CONNECTION ESTABLISHMENT INDICATION message the eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

<Next changes>

### 8.3.11	UE Information Transfer

#### 8.3.11.1	General

The purpose of the UE information transfer procedure is for the MME to send the UE information including QoS Parameters and UE Radio capability to the eNB, for a NB-IoT UE using Control Plane CIoT EPS Optimisation.

#### 8.3.11.2	Successful Operation

Figure 8.3.11.1: UE Information Transfer Procedure. Successful operation.

The MME initiates the procedure by sending the UE INFORMATION TRANSFER message to the eNB.

If the UE Level QoS Parameters IE is contained in the UE INFORMATION TRANSFER message, the eNB shall store this information in the UE context, and use it as specified in TS 23.401 [11].

If the UE Radio Capability IE is contained in the UE INFORMATION TRANSFER message, the eNB shall store this information in the UE context, and use it as specified in TS 23.401 [11].

If the Subscription Based UE Differentiation Information IE is included in the UE INFORMATION TRANSFER message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [11].

If the Pending Data Indication IE is contained in the UE INFORMATION TRANSFER message, the eNB shall store this information in the UE context, and use it as specified in TS 23.401 [11].

If the Masked IMEISV IE is contained in the UE INFORMATION TRANSFER message the eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

<Next changes>

#### 8.6.2.2	DOWNLINK NAS TRANSPORT

Figure 8.6.2.2-1: DOWNLINK NAS Transport Procedure

If the MME only needs to send a NAS message transparently via the eNB to the UE and a UE-associated logical S1-connection exists for the UE or if the MME has received the eNB UE S1AP ID IE in an INITIAL UE MESSAGE message, the MME shall send a DOWNLINK NAS TRANSPORT message to the eNB including the NAS message as a NAS-PDU IE. If the UE-associated logical S1-connection is not established, the MME shall allocate a unique MME UE S1AP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the MME UE S1AP ID IE in the DOWNLINK NAS TRANSPORT, the eNB establishes the UE-associated logical S1-connection.

The NAS-PDU IE contains an MME – UE message that is transferred without interpretation in the eNB.

The DOWNLINK NAS TRANSPORT message may contain the Handover Restriction List IE, which may contain roaming or access restrictions.

If the Handover Restriction List IE is contained in the DOWNLINK NAS TRANSPORT message, the eNB shall store this information in the UE context.

The eNB shall use the information in Handover Restriction List IE if present in the DOWNLINK NAS TRANSPORT message to:

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the DOWNLINK NAS TRANSPORT message and there is no previously stored Handover restriction information, the eNB shall consider that no roaming and no access restriction apply to the UE.

If the Subscriber Profile ID for RAT/Frequency priority IE is included in DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, use it as defined in TS 36.300 [14].

If the Additional RRM Policy Index IE is included in DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, use it as defined in TS 36.300 [14].

If the SRVCC Operation Possible IE is included in DOWNLINK NAS TRANSPORT message, the eNB shall store it in the UE context and, if supported, use it as defined in TS 23.216 [9].

If the UE Radio Capability IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall store this information in the UE context, use it as defined in TS 36.300 [14].

If the Enhanced Coverage Restricted IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the CE-Mode-B Restricted IE is included in the DOWNLINK NAS TRANSPORT message and the Enhanced Coverage Restricted IE is not set to restricted and the Enhanced Coverage Restricted information stored in the UE context is not set to restricted, the eNB shall store this information in the UE context and use it as defined in TS 23.401 [11].

If the NR UE Security Capabilities IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 33.401 [15].

If the End Indication IE is included in the DOWNLINK NAS TRANSPORT message and set to "no further data", the eNB shall consider that besides the included NAS PDU in this message, there are no further NAS PDUs to be transmitted for this UE.

If the Pending Data Indication IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall use it as defined in TS 23.401 [11].

If the Subscription Based UE Differentiation Information IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [11].

If the UE Radio Capability ID IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, use it as defined in TS 23.401 [11].

If the Masked IMEISV IE is included in the DOWNLINK NAS TRANSPORT message the eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Interaction with the NAS Delivery Indication procedure:

If the DL NAS PDU Delivery Acknowledgment Request IE set to "requested" was included in the DOWNLINK NAS TRANSPORT message (see 23.401 [11]), the eNB shall trigger the NAS Delivery Indication procedure, if the downlink NAS PDU was successfully delivered to the UE.

Interaction with the UE Capability Info Indication procedure:

If the UE Capability Info Request IE set to “requested” is included in the DOWNLINK NAS TRANSPORT message, the eNB shall trigger the UE Capability Info Indication procedure if UE capability related information was successfully retrieved from the UE.

<Next changes>

#### 9.1.4.20	CONNECTION ESTABLISHMENT INDICATION

This message is sent by the MME to complete the establishment of the UE-associated logical S1-connection.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| DL CP Security Information | O |  | 9.2.3.49 |  | YES | ignore |
| CE-Mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| End Indication | O |  | 9.2.3.54 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| UE Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.1.15 | Includes QoS parameters. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |

<Next changes>

#### 9.1.4.22	UE INFORMATION TRANSFER

The message is sent by the MME to transfer UE information over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| S-TMSI | M |  | 9.2.3.6 |  | YES | reject |
| UE Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.1.15 | Includes QoS parameters. | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |

<Next changes>

#### 9.1.7.2	DOWNLINK NAS TRANSPORT

This message is sent by the MME and is used for carrying NAS information over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| DL NAS PDU Delivery Acknowledgment Request | O |  | 9.2.3.48 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| UE Capability Info Request | O |  | 9.2.3.51 |  | YES | ignore |
| End Indication | O |  | 9.2.3.54 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |

<Next changes>

-- **************************************************************

--

##### -- DOWNLINK NAS TRANSPORT

--

-- **************************************************************

DownlinkNASTransport ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{DownlinkNASTransport-IEs}},

...

}

DownlinkNASTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-HandoverRestrictionList		CRITICALITY ignore	TYPE HandoverRestrictionList		PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP	CRITICALITY ignore	TYPE SubscriberProfileIDforRFP		PRESENCE optional}|

{ ID id-SRVCCOperationPossible		CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional}|

{ ID id-DLNASPDUDeliveryAckRequest	CRITICALITY ignore	TYPE DLNASPDUDeliveryAckRequest	PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted	CRITICALITY ignore	TYPE EnhancedCoverageRestricted		PRESENCE optional}|

{ ID id-NRUESecurityCapabilities	CRITICALITY ignore	TYPE NRUESecurityCapabilities		PRESENCE optional}|

{ ID id-CE-ModeBRestricted			CRITICALITY ignore	TYPE CE-ModeBRestricted				PRESENCE optional}|

{ ID id-UECapabilityInfoRequest		CRITICALITY ignore	TYPE UECapabilityInfoRequest	PRESENCE optional}|

{ ID id-EndIndication				CRITICALITY ignore	TYPE EndIndication					PRESENCE optional}|

{ ID id-PendingDataIndication		CRITICALITY ignore	TYPE PendingDataIndication			PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex	CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex		PRESENCE optional}|

{ ID id-UERadioCapabilityID			CRITICALITY reject	TYPE UERadioCapabilityID			PRESENCE optional}|

{ ID id-Masked-IMEISV				CRITICALITY ignore	TYPE Masked-IMEISV				PRESENCE optional},

...

}

<Next changes>

-- **************************************************************

--

-- Connection Establishment Indication

--

-- **************************************************************

ConnectionEstablishmentIndication::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {ConnectionEstablishmentIndicationIEs} },

...

}

ConnectionEstablishmentIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability			PRESENCE optional }|

{ ID id-EnhancedCoverageRestricted	CRITICALITY ignore	TYPE EnhancedCoverageRestricted	PRESENCE optional }|

{ ID id-DL-CP-SecurityInformation	CRITICALITY ignore	TYPE DL-CP-SecurityInformation	PRESENCE optional }|

{ ID id-CE-ModeBRestricted			CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-EndIndication				CRITICALITY ignore	TYPE EndIndication				PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-UE-Level-QoS-Parameters		CRITICALITY ignore	TYPE E-RABLevelQoSParameters	PRESENCE optional}|

{ ID id-UERadioCapabilityID			CRITICALITY reject	TYPE UERadioCapabilityID		PRESENCE optional}|

{ ID id-Masked-IMEISV				CRITICALITY ignore	TYPE Masked-IMEISV				PRESENCE optional},

...

}

<Next changes>

-- **************************************************************

-- UE Information Transfer

--

-- **************************************************************

UEInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  UEInformationTransferIEs} },

...

}

UEInformationTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-S-TMSI						CRITICALITY reject 	TYPE S-TMSI						PRESENCE mandatory}|

{ ID id-UE-Level-QoS-Parameters		CRITICALITY ignore	TYPE E-RABLevelQoSParameters	PRESENCE optional}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability			PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-PendingDataIndication		CRITICALITY ignore	TYPE PendingDataIndication		PRESENCE optional}|

{ ID id-Masked-IMEISV				CRITICALITY ignore	TYPE Masked-IMEISV				PRESENCE optional},

...

}