3GPP TSG-RAN WG3 Meeting #117bis-e	R3-226037

# Online, 10th Oct. – 18th Oct., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0900 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP Corrections related to Excess Packet Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the previous RAN3#115-e meeting it has been agreed to change the current M6 configuration IE by introducing the M6 Delay Threshold IE to enable the calculation of the PDCP Excess Packet Delay in the UL. However, this addition does not reflect the specification of Excess Packet Delay in RAN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The M6 Delay Threshold IE is renamed to Excess Packet Delay Threshold IE. This IE is changed from single enumerated value to a list of multiple values to align with RAN2´s specifications. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has a impact on the protocol. This CR has functionality impact. This CR impact can be considered isolated. This CR is NBC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN2 and RAN3 specifications are not in sync while RAN3 specification will fail to provide functionality already assumed by agreed RAN2 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.174, 9.3.1.x, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS38.423 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.174	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  | - |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  | - |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity, …) | Number of reports | YES | ignore |
| Excess Packet Delay Threshold | O |  | 9.3.1.x | Indicates the excess packet delay threshold, as specified in TS 38.314 [42] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Excess Packet Delay Threshold

This IE defines the parameters for Excess Packet Delay Threshold configuration to support the calculation of the PDCP Excess Packet Delay in the UL per DRB as specified in TS 38.314 [48].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Excess Packet Delay Threshold Item |  | 1..<maxnoofThresholds> |  |  |
| >Excess Packet Delay Threshold Value | M |  | ENUMERATED (ms0.25, ms0.5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300,ms500, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of Thresholds allowed for a UE. Value is 255. |

<<<<<<<<<<<<<<<<<<<< End of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

-------Text no need to change omitted-------

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-ExcessPacketDelayThreshold,

id-M7ReportAmount,

-------Text no need to change omitted-------

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholds

-- TEXT OMITTED –

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT		PRESENCE optional		}|

{ ID id-ExcessPacketDelayThreshold	CRITICALITY ignore	EXTENSION ExcessPacketDelayThreshold		PRESENCE optional		},

...

}

ExcessPacketDelayThreshold ::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF ExcessPacketDelay-thresholdItem

ExcessPacketDelay-thresholdItem ::= SEQUENCE {

excessPacketDelay-thresholdValue			ExcessPacketDelay-thresholdValue,

iE-Extensions		ProtocolExtensionContainer { { ExcessPacketDelay-thresholdItem-ExtIEs} }	OPTIONAL,

...

}

ExcessPacketDelay-thresholdItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExcessPacketDelay-thresholdValue ::= ENUMERATED {

ms0.25, ms0.5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500,

...

}

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< End of 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

-- TEXT OMITTED –

maxnoofPLMNforQMC					INTEGER ::= 16

maxnoofUEAppLayerMeas				INTEGER ::= 16

maxnoofSNSSAIforQMC					INTEGER ::= 16

maxnoofTAforQMC						INTEGER ::= 8

maxnoofThresholds					INTEGER ::= 255

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

-- TEXT OMITTED –

id-IncludeBeamMeasurementsIndication					ProtocolIE-ID ::= 340

id-ExcessPacketDelayThreshold									ProtocolIE-ID ::= 341

id-PagingCause											ProtocolIE-ID ::= 342

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized									ProtocolIE-ID ::= 345

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< End of 4th set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>