3GPP TSG-RAN WG3 Meeting #117bis-e	 R3-225465

E-meeting, 10th – 18th Oct 2022

Title: 	Discussion on MC support for RRC Inactive

Source: 	Samsung

Agenda item:	15.3

Document Type:	Discussion

1. Introduction

According to the approved The Rel-18 NR MBS enhancement WID, the following objectives has impact to RAN3.

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

- Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

Note: collaboration with SA2 is expected in due course for the above objectives.

2. Discussion

In RAN3#117e meeting, it was agreed:

- It is the common understanding that the following information, among others, may be taken into account by the gNB when deciding to enable UEs receiving multicast in RRC inactive state:

- Capability of UE (of whether support the mode “multicast over RRC inactive”)

- Rel-17 multicast context, e.g. the QoS parameters not associated to any specific UE;

- Parameters available at the local gNB without enhancement on interfaces, e.g. cell load.

When the UE moves within the same RNA area, the UE can perform cell reselection without notify the cell change to the network. The network tracks the UE at the RNA level. It was agreed the gNB decides which MBS UEs can be moved into RRC Inactive when the MBS session is active. But the gNB can not move all the UE into RRC Inactive mode since the NG shared tunnel is established when MBS users exists in the NG-RAN. If the gNB isn’t aware there are some MBS users, the gNB may release the NG-U shared tunnel or may not setup the NG-U shared tunnel at all. Thus we think it is a reasonable assumption that each cell should keep a suitable number of UE in RRC connected mode. Otherwise, the gNB may release itself from the distribution tree.

When UE in RRC Connected mode moves out of the cell, the number for connected UE is reduced. In extreme case, no any RRC connected mode UE exists in the cell. To avoid that no RRC connected mode UE in the gNB is receiving MBS data, when the number of users in connected mode decreases to a certain extent, the gNB can send a RAN Paging message to the RRC Inactive mode UEs. The paging message can tell the UE to enter the connection mode to receive MBS services.

If the gNB has no RRC inactive UE context, e.g. the gNB is the new serving for the RRC inactive UE while the UE context is stored in the old gNB, the gNB can send a Multicast Group Paging message to bring some UEs into RRC connected and keep them in the RRC connected. The purpose of the Multicast Group Paging procedure is to provide the paging information to enable the gNB-DU to multicast group page UEs which have joined an MBS Session about its activation. However, for Rel-18 MBS UE, which is configured to receive or is receiving multicast session in RRC inactive, the group paging message also be used for MBS purpose, and bring the UE into RRC connected mode.

Proposal 1: gNB keeps a suitable number of UE in RRC connected. RAN Paging message or Multicast group paging message is used to move RRC Inactive mode into RRC Connected for MBS purpose.

In RAN2, multicast reception in RRC Inactive was discussed. How to delivery PTM configuration to the inactive mode UE will be further investigated, either using dedicated signalling, which need to bring the UE into RRC connected mode, or using a solution based on SIB+MCCH. Combination of both options is not precluded.

No matter which option is selected, when the UE perform cell reselection to neighbour cells during active multicast session, it is possible the UE need to resume RRC connection, e.g. the configuration of MBS in the new cell is not available since the UE is the first user in this cell, the UE needs to come into RRC Connected to get the MRB configuration. The UE may need to indicate to the new gNB, the resume cause is for MBS.

When the UE is resumed in a gNB other than the last serving gNB, the new gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB. The last serving gNB may provide the UE context and may decide not to relocate the UE context and to keep the UE in RRC_Inactive. This is the legacy Rel-17 operation for non-MBS service.

For the MBS service, how to notify PTM configuration to the UE is pending to RAN2 conclusion. If the PTM configuration is notified to the UE via dedicated message, when the UE moves to a new gNB, the new gNB needs to keep the UE in RRC Connected mode and transmits the PTM configuration to the UE. In this case, the UE context shall be transferred from the old gNB to the new gNB.

If the PTM is notified to the UE via broadcast signalling, when the inactive UE doesn’t receive PTM configuration in the new gNB. e.g. the UE is the first MBS user under this gNB, the UE performs RRC resume procedure and the new gNB keeps the UE in RRC connected mode. The UE context shall be transferred to the new gNB. In other cases, when the UE triggers RRC Resume for MBS purpose, it is also better to transfer the UE context to the new gNB, in order to support the MBS service continuity when the UE perform the further mobility.

If the UE performs the RRC Resume with the purpose for MBS, it is straightforward to forward it to the old gNB. When the old gNB receives it, the old gNB always transfers the UE context to the new gNB.

Proposal 2: The new gNB notifies the Retrieve UE context procedure is triggered for the MBS, then the old gNB always transfers the UE context to the new gNB.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: gNB keeps a suitable number of UE in RRC connected. RAN Paging message or Multicast group paging message is used to move RRC Inactive mode into RRC Connected for MBS purpose.

Proposal 2: The new gNB notifies the Retrieve UE context procedure is triggered for the MBS, then the old gNB always transfers the UE context to the new gNB.

4. Reference

- RP-213568 New WID: Enhancements of NR Multicast and Broadcast Services

- RP-222388 Status Report to TSG