3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225779

Electronic Meeting, October 10th – 18th, 2022

Agenda item:		12.2.2.1

Source:				Intel Corporation

Title:	Discussion on new procedures for AI/ML information exchange (including TP for TS 38.423)

Document for:		Discussion and Decision

# Introduction

During RAN3 #117bis-e meeting, it was agreed to define a new procedure over Xn to be used for AI/ML related information.

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information FFS whether to use the existing procedure or dedicated new procedure for other input, output and feedback information. The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

In this contribution, we focus on what information can be used for this procedure and the necessity of using this new procedure for other input/output/feedback information.

# Discussion

## Signaling Procedure for Predicted Information and Other Information

As agreed in RAN3 #117e meeting, following predicted information are agreed to be exchanged over Xn:

Regarding AI/ML based Energy Saving, the following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn Regarding AI/ML based Load Balancing, the following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn Regarding AI/ML based Load Balancing, the following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn - Predicted cell-granularity UE trajectory can be exchanged over Xn for AI/ML based mobility optimization.

As captured in [1], the following information is required to be collected from a neighbouring NG-RAN node as an input for Model Training and Model Inference over Xn interface:

| Use case | IE |
| --- | --- |
| Common | Current resource status |
|  | Predicted resource status |
|  | Impacted UE performance |
|  | System performance |
| Energy Saving | Current energy efficiency |
|  | Predicted energy efficiency |
|  | Current energy state |
| Mobility | Predicted cell-granularity UE trajectory |
|  | UE history information |
|  | Position, QoS and performance info of historical Handed-over UEs |
|  | Handover event of previous handed-over UEs |

Regardless of whether the information is used as input/output/feedback, as observed from the above table, there are in general two categories of data: 1) data about current system performance/information (i.e. current data); 2) predicted data generated by Model Inference.

During RAN3 #117-e meeting, it was marked FFS whether the new procedure defined for predicted information can also be used for other information.

For the current data, e.g. resource status, UE history information, handover event, etc, they can already be supported by the existing signalling/message. For example, the current resource status of the neighbouring NG-RAN nodes can be exchanged over the existing RESOURCE STATUS REQUEST/RESPONSE messages via Xn interface.

If the new procedure is also used for current data, both existing and new procedure will be used to exchange the same information, which is unnecessary and a duplication.

Observation 1: Exchanging resource status, UE history information (including position QoS and performance information of historical handed-over UEs), handover event can already be supported by the existing message/signalling over Xn interface, which would be a duplication if they are also exchanged by the new procedure defined for predicted information.

There’s also other information which needs to be exchanged over Xn, e.g. UE performance, system KPI, etc. On one hand, the performance information is only available after certain AI/ML action is taken. When the predicted information is requested, it may not be available and may not be able to be encoded into the response/update message. On the other hand, the feedback information can also be used as input for Model Training and Model Inference. Therefore, it is more desirable to use the same procedure/message to transmit feedback information together with other input information. For example, as discussed in the companion contribution [2], the existing Resource Status Request/Response/Update procedure can be re-used for feedback information as well.

Observation 2: The feedback information may not be suitable to be carried by the new procedure for predicted information, as it may not be available when the predicted information is requested. The existing procedure to exchange current information is better to be re-used to carry the feedback information.

Proposal 1: The new procedure over Xn is only used to transmit AI/ML related predicted information, i.e. predicted resource status, predicted energy efficiency and predicted cell-granularity UE trajectory. The existing procedure should be used to transmit other information, such as resource status, UE performance, etc.

## Prediceted Information Request, Response and Update

It was agreed during RAN3 #117e meeting, request/response/update procedure is used as baseline for new procedure used for predicted information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

As discussed in the above section, the predicted resource status from the neighboring NG-RAN nodes is essential for all three use cases. Moreover, the predicted energy efficiency is additionally required for the AI/ML based network energy saving. The requested predicted information is not always use case specific and some information can be commonly used by more than one use case. For better forward-compatibility and easy to expand to other use cases, it is proposed to use separate IEs for different predicted information, rather than for different AI/ML-based use case.

Proposal 2: The Predicted Information Request/Response/Update messages should be designed predicted information specific rather than use-case specific, including separate IEs to request/report the predicted energy efficiency and/or the predicted resource status.

It was further agreed that a non-UE associated procedure is used as a start point for this new procedure.

The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point. FFS on whether UE associated procedure is needed.

Similar to the existing Resource Status Report procedure, the Predicted Information Request message is used to initiate the neighbouring NG-RAN node to start a predicted information generation, stop a predicted information generation or add cells to report for a predicted information generation. The Predicted Information Response message is provided to the requesting NG-RAN node if the predicted information is available; otherwise, a Predicted Information Failure message will be triggered, indicating a failure predicted information request.

Fig. Successful Operation of Predicted Information Reporting Initiation

Fig. Failure Operation of Predicted Information Reporting Initiation

Proposal 3: Design the Predicted Information Request message to initiate a class-1 Predicated Information Reporting Initiation procedure, with the Predicted Information Response/Failure messages to indicate a successful operation and a failure operation, respectively.

Similar to the existing Resource Status Update message, the Predicted Information Update message can be defined to report the result of the predicted information admitted by the NG-RAN node, following a successful ack of the Predicted Information Reporting Initiation procedure.

Fig. Successful Operation of Predicted Information Reporting

Proposal 4: Specify the Predicted Information Update message to report the result of the predicted information admitted by the NG-RAN node, if available.

As discussed earlier, the Predicted Information Report procedure is only used to exchange predicted information, i.e. predicted resource status and predicted energy efficiency. Both of them are cell-specific. Therefore, a UE-associated procedure is not needed for predicted information exchange.

Proposal 5: Use non-UE associated signalling for the Predicted Information Report Initiation and Predicted Information Report procedures.

## AI/ML Capability Request/Response

However, providing predicted data to a neighbouring NG-RAN node depends on whether the current NG-RAN node owns the corresponding AI/ML capability or not. It is possible that when one predicted information is being requested, the requested node may not be able to provide such information in the Predicted Information Update message due to lack of Model Inference. For example, if one NG-RAN node does not have a specific AI/ML model deployed, it is impossible for such NG-RAN node to provide predicted data when a neighbouring NG-RAN node requests for it.

To avoid requesting predicted data unavailable from an incapable neighbouring NG-RAN node, we believe it is necessary to exchange AI/ML capabilities between NG-RAN nodes over Xn interface before requesting data collection. In doing so, separate AI/ML capabilities of supporting different use cases can also be considered. For example, AI/ML based network energy saving model may require the predicted energy efficiency and the predicted resource status from the neighbouring NG-RAN nodes, while for AI/ML based load balancing and mobility optimization, the predicted resource status is only required. Based on the deployed model(s) at the neighbouring NG-RAN node, it can respond accordingly to the received AI/ML Capability Request with its supported use case, e.g. energy efficiency prediction, resource status prediction, or both.

By requesting the AI/ML capability per use case basis can also avoid unnecessary configuration/requesting for requesting other input information, which can further reduce complexity for signaling exchange. For example, if the neighbouring NG-RAN node does not support AI/ML based energy saving use case, one NG-RAN node does not need to request the neighbouring NG-RAN node to provide the Predicted Energy Efficiency and Predicted Resource Status information.

Proposal 6: AI/ML capability (i.e. whether it supports which AI/ML functionality) is exchanged between NG-RAN nodes over Xn interface. Separate AI/ML capabilities of supporting different use cases can be considered.

# Conclusion

In this contribution, we discussed about the new procedures defined for predicted information exchange and AI/ML capability request/response.

We propose the following observations and proposals:

Observation 1: Exchanging resource status, UE history information (including position QoS and performance information of historical handed-over UEs), handover event can already be supported by the existing message/signalling over Xn interface, which would be a duplication if they are also exchanged by the new procedure defined for predicted information.

Observation 2: The feedback information may not be suitable to be carried by the new procedure for predicted information, as it may not be available when the predicted information is requested. The existing procedure to exchange current information is better to be re-used to carry the feedback information.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Proposal 1: The new procedure over Xn is only used to transmit AI/ML related predicted information, i.e. predicted resource status, predicted energy efficiency and predicted cell-granularity UE trajectory. The existing procedure should be used to transmit other information, such as resource status, UE performance, etc.

Proposal 2: The Predicted Information Request/Response/Update messages should be designed predicted information specific rather than use-case specific, including separate IEs to request/report the predicted energy efficiency and/or the predicted resource status.

Proposal 3: Design the Predicted Information Request message to initiate a class-1 Predicated Information Reporting Initiation procedure, with the Predicted Information Response/Failure messages to indicate a successful operation and a failure operation, respectively.

Proposal 4: Specify the Predicted Information Update message to report the result of the predicted information admitted by the NG-RAN node, if available.

Proposal 5: Use non-UE associated signalling for the Predicted Information Report Initiation and Predicted Information Report procedures.

Proposal 6: AI/ML capability (i.e. whether it supports which AI/ML functionality) is exchanged between NG-RAN nodes over Xn interface. Separate AI/ML capabilities of supporting different use cases can be considered.

The corresponding text proposal for TS 38.423 is provided in Annex.

# References

[1] TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

[2] R3-225776, "Discussion on Stage 2 of AI/ML based Use Cases", Intel Corporation

# Annex – Text Proposal for TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Capability | AI/ML CAPABILITY REQUEST | AI/ML CAPABILITY RESPONSE |  |
| Predicted Information Reporting Initiation | PREDICTED INFORMATION REQUEST | PREDICTED INFORMATION RESPONSE | PREDICTED INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Predicted Information Reporting | PREDICTED INFORMATION UPDATE |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.X	AI/ML procedures

### 8.X.1	AI/ML Capability

#### 8.X.1.1	General

This procedure is used by an NG-RAN node to request AI/ML capability for prediction service(s) from another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: AI/ML Capability, successful operation

The NG-RAN node1 initiates the procedure by sending the AI/ML CAPABILITY REQUEST message to the NG-RAN node2.

If the Energy Efficiency Prediction Request IE is contained in the AI/ML CAPABILITY REQUEST message, the NG-RAN node2 shall evaluate its AI/ML capability for energy efficiency prediction and respond the Energy Efficiency Prediction Support IE in the AI/ML CAPABILITY RESPONSE message.

If the Resource Status Prediction Request IE is contained in the AI/ML CPABILITY REQUEST message, the NG-RAN node2 shall evaluate its AI/ML capability for resource status prediction and respond the Resource Status Prediction Support IE in the AI/ML CAPABILITY RESPONSE message.

Interactions with other procedures:

This procedure may be initiated by the NG-RAN node1 before initiating the Predicted Information Reporting Initiation procedure.

#### 8.X.1.3	Unsuccessful Operation

Void.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Predicted Information Reporting Initiation

#### 8.X.2.1	General

This procedure is used by an NG-RAN node to request the reporting of prediction information from another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Predicted Information Reporting Initiation, successful operation

The NG-RAN node1 initiates the procedure by sending the PREDICTED INFORMATION REQUEST message to the NG-RAN node2 to start a predicted information generation, stop a predicted information generation or add cells to report for a predicted information generation. Upon receipt, the NG-RAN node2:

-	shall initiate the requested predicted information generation according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells prediction measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the prediction measurements initiated before for the given prediction measurement IDs, in case the Registration Request IE is set to "add". If the prediction measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the PREDICTED INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific prediction measurements, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "add" in the PREDICTED INFORMATION REQUEST message, the Cell To Report List IE shall be included.

If the NG-RAN node2 is capable to provide all requested predicted information, it shall initiate the prediction measurements as requested by the NG-RAN node1 and respond with the PREDICTED INFORMATION RESPONSE message.

If the Validity Time IE in the PREDICTED INFORMATION REQUEST is present, the NG-RAN node2 shall generate the predicted information for the corresponding validity time.

If the Reporting Periodicity IE in the PREDICTED INFORMATION REQUEST message is present, this indicates the periodicity for the reporting of the predicted information. The NG-RAN node2 shall generate the predicted information at each corresponding report periodicity. If the Validity Time IE is also present in the PREDICTED INFORMATION REQUEST message, the NG-RAN node node2 shall generate the predicted information at each corresponding report periodicity since validity time. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.2-2: Predicted Information Reporting Initiation, unsuccessful operation

If any of the requested prediction measurements cannot be initiated, the NG-RAN node2 shall send the PREDICTED INFORMATION FAILURE message with an appropriate cause value.

If the requested predicted information cannot be generated within the time configured by the Validity Time IE, the NG-RAN node2 shall send the PREDICTED INFORMATION FAILURE message with an appropriate cause value.

#### 8.X.2.4	Abnormal Conditions

Void.

### 8.X.3	Predicted Information Reporting

#### 8.X.3.1	General

This procedure is initiated by an NG-RAN node to report the result of the predicted information admitted by the NG-RAN node following a successful Predicted Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2-1: Predicted Information Reporting, unsuccessful operation

The NG-RAN node2 shall report the results of the admitted predicted information in the PREDICTED INFORMATION UPDATE message. The admitted predicted information is the information that was successfully initiated during the preceding Predicted Information Reporting Initiation procedure.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.1.3	Messages for Global Procedures

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.3.aa	AI/ML CAPABILITY REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate AI/ML capability request.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Energy Efficiency Prediction Request | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Resource Status Prediction Request | O |  | ENUMERATED (true, …) |  | YES | ignore |

#### 9.1.3.bb	AI/ML CAPABILITY RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to transfer the requested AI/ML capability information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Energy Efficiency Prediction Support | O |  | ENUMERATED (supported, Not supported, …) |  | YES | ignore |
| Resource Status Prediction Support | O |  | ENUMERATED (supported, Not supported, …) |  | YES | ignore |

#### 9.1.3.cc		PREDICTED INFORMATION REQUSET

This message is sent by NG-RAN node1 to NG-RAN node2 to request the predicted information reporting according to the parameters given in the message.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the prediction information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates prediction measurement object the NG-RAN node2 is requested to report. First Bit = Energy Efficiency Prediction, Second Bit = Resource Status Prediction, Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1.. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1.. <maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1.. <maxnoofBPLMNs> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1.. <maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  | – |  |
| Validity Time | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the time that model inference output becomes valid in the system. | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of the requested predicted information. It also indicates the validity time of the corresponding prediction. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.dd	PREDICTED INFORMATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the measurements for all the requested predicted information are successfully initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.ee	PREDICTED INFORMATION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that any of the requested measurements cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.ff PREDICTED INFORMATION UPDATE

This message is sent by the NG-RAN node2 to the NG-RAN node1 to report the results of the requested predicted information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Prediction ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Prediction Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Prediction Measurement Result Item |  | 1.. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Energy Efficiency Prediction | O |  | Energy Efficiency 9.2.2.cc |  | – |  |
| >>Resource Status Prediction | O |  | Radio Resource Status 9.2.2.50 |  | – |  |
| Validity Time | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. It indicates the time that model inference output becomes valid in the system. | YES | ignore |
| Confidence Level | O |  | INTEGER (1..100) | The confidence level of prediction in percentage. | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	NG-RAN Node and Cell Configuration related IE definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.cc	Energy Efficiency

The Energy Efficiency IE indicates the energy efficiency status of the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Energy Efficiency Type | M |  |  |  |  |  |
| >NG-RAN data energy efficiency |  |  |  |  |  |  |
| >>NG-RAN Data Energy Efficiency | O |  | TBD |  |  |  |
| >Network slice energy efficiency |  |  |  |  |  |  |
| >>Energy Efficiency of eMBB Network Slice | O |  | TBD |  |  |  |
| >>Energy Efficiency of uRLLC Network Slice | O |  | TBD |  |  |  |
| >>Energy Efficiency of mIoT Network Slice | O |  | TBD |  |  |  |
| >PNF power consumption |  |  |  |  |  |  |
| >>Average Power | O |  | TBD |  |  |  |
| >>Minimum Power | O |  | TBD |  |  |  |
| >>Maximum Power | O |  | TBD |  |  |  |
| >PNF energy consumption |  |  |  |  |  |  |
| >>PNF Energy Consumption | O |  | TBD |  |  |  |
| >energy state |  |  |  |  |  |  |
| >>Energy State | O |  | ENUMERATED (high, low, active, deactive) |  |  |  |