3GPP TSG-RAN WG3 Meeting #118	R3-226271

Toulouse, France, Nov. 14th – Nov. 18th, 2022

Agenda Item:	13.2

Source: 	Canon Research Centre France

Title:	Discussion on the DU Migration of Mobile IAB-node

Document for:	Discussion

# Introduction

In this paper, we discuss the DU migration of a mobile IAB-node and its decoupling with the co-located MT. The discussion is based on the following agreements made at the previous RAN3 meetings [1] and [2]:

At RAN3#117: - RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT stays connected to the same donor before and after the mobile IAB-DU migration. - RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT executes inter-donor migration. - The mobile IAB-node may perform multiple consecutive partial migrations without inter-donor migration of its mobile IAB-DU. - For DU migration cases, to execute the handover of the served UEs, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source CU and the target CU, respectively. - The UEs connected to the mobile IAB-node are handed over from the cell of the logical mobile IAB-DU (i.e., the source logical mobile IAB-DU) that has an F1AP association with the source CU to the cell of the logical mobile IAB-DU (i.e., the target logical mobile IAB-DU) that has an F1AP association with the target CU. At RAN3#117bis: - mIAB-DU migration and mIAB-MT handover can be executed independently from each other. Details on the scenarios need to be further discussed - For partial migration of mIAB-node, the inter-donor HO of mIAB-MT is decided and triggered by the donor CU serving the mIAB-MT. - The donor CU serving the mIAB-DU decides whether to execute mIAB-DU migration or inter-donor F1 transport migration for the mIAB-DU. - For inter-donor partial migration, the donor CU serving the mIAB-DU is informed about the mIAB-MT HO. FFS on signalling details concerning the indication. - For partial migration of mIAB-node, the donor CU serving the mIAB-DU and the target donor CU for the mIAB-MT HO can directly exchange Xn IAB Transport Migration messages, in case direct Xn connectivity exists (or is established) between the two donor CUs.

# Discussion

## Decoupling MT and DU Migration

Executing a mobile IAB-MT (mIAB-MT) migration at the same time as the co-located mobile IAB-DU (mIAB-DU) migration may lead to failure of at least one of the procedures or it may drastically increase the complexity of signalling. As a safe approach, mIAB-MT migration should be executed and completed while the co-located mIAB-DU stays connected to the same donor-CU, and mIAB-DU migration should be executed and completed while the co-located mIAB-MT stays connected to the same donor-CU.

To avoid concurrent migrations of mIAB-MT and mIAB-DU, a single donor-CU should control the sequence of mIAB-MT and mIAB-DU migrations for a mobile IAB-node. As, on one hand, the donor-CU serving the mIAB-DU decides whether to execute the mIAB-DU migration, and, on the other hand, the donor CU serving the mIAB-DU is informed about the mIAB-MT handover, the donor-CU serving the mIAB-DU appears as the appropriate donor-CU to control the sequence. Moreover, the donor-CU serving the mIAB-DU may know when a mIAB-MT migration is completed as it can directly exchange Xn IAB Transport Migration messages with the target donor CU.

Observation 1: the F1 terminating donor-CU of an IAB-node is the appropriate donor-CU to control the sequence of IAB-MT and IAB-DU migrations for the mobile IAB-node.

Thus, when the non-F1 terminating donor-CU of a mobile IAB-node detects the need to execute a mIAB-MT migration to connect the mIAB-MT to a target non-F1 terminating donor-CU, the non-F1 terminating donor-CU should let the F1 terminating donor-CU trigger the mIAB-MT migration at the appropriate time.

Proposal 1: The non-F1 terminating donor-CU of a mobile IAB-node should let the F1 terminating donor trigger the inter-donor migration of the mIAB-MT at the appropriate time.

As a possible way to control the sequence, the non-F1 terminating donor-CU detecting the need for a mIAB-MT migration to connect the mIAB-MT to a target non-F1 terminating donor-CU, may first inform the F1 terminating donor-CU. Then, the F1 terminating donor-CU will trigger the inter-donor mIAB-MT migration at the appropriate time, and will respond to the non-F1 terminating donor-CU.

## IAB-DU Migration

The triggering events and the decision process for a F1 terminating donor CU to execute the mobile IAB-DU (mIAB-DU) migration for a mobile IAB-node may be left to implementation. Besides, for flexible IAB network management, it should be allowed to migrate the mIAB-DU towards a target F1 terminating donor-CU different from the non-F1 terminating donor-CU serving the co-located mobile IAB-MT (mIAB-MT).

Proposal 2: The IAB-DU of a mobile IAB-node may be migrated towards a target F1 terminating donor-CU different from the non-F1 terminating donor-CU serving the co-located IAB-MT.

Then, when the F1 donor CU serving the mIAB-DU decides whether to execute mIAB-DU migration, it may request the mobile IAB-node to activate a second logical DU to be served by a target F1 donor-CU identified in the request.

For instance, the F1 donor-CU may trigger the gNB-CU Configuration Update procedure to request the mobile IAB-node to setup a new F1 connection with an identified target F1 donor-CU. Then, the activated logical DU triggers a F1 setup procedure with the target F1 donor-CU. In case the request from the F1 donor-CU does not identify a target F1 donor-CU, then the activated logical DU triggers a F1 setup procedure with the non-F1 terminating donor-CU of the mobile IAB-node.

Proposal 3: When requested by its serving F1 donor CU to setup a new F1 connection, a mobile IAB-node may execute a F1 setup procedure with the target F1 donor-CU indicated in the request. In the absence of such indication, the mobile IAB-node executes the F1 setup procedure with the non-F1 terminating donor-CU.

To trigger the handover of UEs served by the mobile IAB-node during the mIAB-DU migration, the target F1 donor-CU may inform the source F1 donor-CU about the activation of new cell(s) in the second logical DU of the mobile IAB-node. For this purpose, the target F1 donor-CU may execute the NG-RAN node configuration update procedure with the source F1 donor-CU. Once informed, the source F1 donor-CU can send a handover command to the UEs served by the migrating mobile IAB-node.

Proposal 4: The NG-RAN node configuration update procedure may be used by the target F1 terminating donor-CU of a mobile IAB-node to inform the source F1 terminating donor-CU about the ID(s) of the cell(s) served by the second logical DU of the mobile IAB-node.

# Conclusion

In this contribution, we have proposed a solution to avoid concurrent inter-donor MT and DU migrations at a mobile IAB-node. Besides, we have proposed some procedures to manage the DU migration of a mobile IAB-node. Finally, the following observation and proposals have been made:

Observation 1: the F1 terminating donor-CU of an IAB-node is the appropriate donor-CU to control the sequence of IAB-MT and IAB-DU migrations for the mobile IAB-node.

Proposal 1: The non-F1 terminating donor-CU of a mobile IAB-node should let the F1 terminating donor trigger the migration of the mIAB-MT at the appropriate time.

Proposal 2: The IAB-DU of a mobile IAB-node may be migrated toward a target F1 terminating donor-CU different from the non-F1 terminating donor-CU serving the co-located IAB-MT.

Proposal 3: When requested by its serving F1 donor CU to setup a new F1 connection, a mobile IAB-node may execute a F1 setup procedure with the target F1 donor-CU indicated in the request. In the absence of such indication, the mobile IAB-node executes the F1 setup procedure with the non-F1 terminating donor-CU.

Proposal 4: The NG-RAN node configuration update procedure may be used by the target F1 terminating donor-CU of a mobile IAB-node to inform the source F1 terminating donor-CU about the ID(s) of the cell(s) served by the second logical DU of the mobile IAB-node.

# References

[1] Chairman notes of RAN3#117e meeting

[2] Chairman notes of RAN3#117-bis-e meeting