3GPP TSG-RAN WG3 Meeting #118	R3-226436

14th – 18th Nov, 2022

Agenda Item:	13.2

Source: 	Lenovo

Title: 	Inter-donor full migration procedure of mobile IAB

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, general procedure for mobile IAB-node inter-donor full migration is discussed.

- Discussion

IAB inter-donor full migration procedure

In R17, partial migration procedure for IAB inter-donor topology adaptation was already defined, where the IAB-MT is migrated from a source IAB-donor-CU to a target IAB-donor-CU while its IAB-DU retains F1AP with the source IAB-donor-CU.

And based on the discussion in R17, two logical IAB-DUs at the boundary IAB node are to be further discussed in the scope of Full Migration, where following two implementation alternatives can be applied [2]. Further based on the inputs from RAN1/RAN2 and RAN4, Alt.1 can work well and has fewer standards impacts.

- Alt1: the two logical DUs use separate physical cell resources

- Alt2: the two logical DUs use the same physical cell resources

For the mobile IAB-node, two logical IAB-DUs can be applied during the full migration where the second logical IAB-DU can setup F1 interface with target IAB-donor-CU at the same time that the first IAB-DU still has the F1 connection with the source IAB-donor-CU as agreed in last meeting [3].

For DU migration cases, to execute the handover of the served UEs, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source CU and the target CU, respectively. The UEs connected to the mobile IAB-node are handed over from the cell of the logical mobile IAB-DU (i.e., the source logical mobile IAB-DU) that has an F1AP association with the source CU to the cell of the logical mobile IAB-DU (i.e., the target logical mobile IAB-DU) that has an F1AP association with the target CU.

Based on the assumption of two logical IAB-DUs in mobile IAB-node, two potential full migration procedures are proposed as follow in case of Xn connectivity and IP connectivity are available.

Full migration Option 1:

Figure 1 shows an example of mobile IAB-node full migration procedure, where the mobile IAB-node is migrated from a source IAB-donor-CU1 to a target IAB-donor-CU2. In this procedure, the IAB-MT is firstly migrated to the target IAB-donor-CU2 and IAB-DU2 setups the F1 interface with the target IAB-donor-CU2 via the target path.

Figure 1. An example procedure of inter-donor full migration (Option 1)

Step 1: HO preparation for IAB-MT is performed between source IAB-donor-CU1 and target IAB-donor-CU2, and IAB-donor-CU1 sends the RRCReconfiguration for HO to IAB-MT.

Step 2: IAB-MT performs random access procedure at the target IAB-donor-DU2 (or at the target parent IAB-node) and migrates to the IAB-donor-CU2.

After step 2, the F1 interface of IAB-DU1 and the RRC connection for served UE are migrated to the target path as defined in R17 (partial migration).

Step 3: F1 interface setup between IAB-DU2 and IAB-donor-CU2 via the target path.

Step 4: IAB-donor-CU1 triggers HO procedure for served UE and perform HO preparation for the served UE with IAB-donor-CU2, and IAB-donor-CU1 sends the RRCreconfiguration for HO to the served UE via the target path.

Step 5: UE performs random access procedure at IAB-DU2 and migrate to IAB-donor-CU2.

Step 6: F1 removal between IAB-DU1 and IAB-donor-CU1.

Full migration Option 2:

Figure 2 shows another example procedure of mobile IAB-node full migration, where the mobile IAB-node is migrated from a source IAB-donor-CU1 to a target IAB-donor-CU2. In this procedure, the IAB-DU2 can setup the F1 interface with the target IAB-donor-CU2 in advance via the source path, and source IAB-donor-CU1 can perform the HO for UE and IAB-MT parallelly.

Figure 2. An example procedure of inter-donor full migration (Option 2)

Step 1: F1 interface setup between IAB-DU2 and IAB-donor-CU2 via the source path, the step 1 can be performed parallel or performed before IAB-MT migration.

Step 2: IAB-donor-CU1 triggers HO procedure for served UE and perform HO preparation for the served UE with IAB-donor-CU2, and IAB-donor-CU1 sends the RRCreconfiguration for HO to the served UE via the source path.

Step 3: The served UE performs random access procedure at IAB-DU2 and migrate to IAB-donor-CU2.

Step 4: HO preparation for IAB-MT is performed between source IAB-donor-CU1 and target IAB-donor-CU2, and IAB-donor-CU1 sends the RRCreconfiguration for HO to IAB-MT. The step 4 may be performed at an earlier stage, e.g., after step 1, or before step 1.

Step 5: IAB-MT performs random access procedure at the target IAB-donor-DU2 (or at the target parent IAB-node) and migrates to the IAB-donor-CU2.

After step 5, the F1 interface of IAB-DU2 and the RRC connection for the served UE will be migrated to the target path. If there still some remaining UEs have not trigger the HO to IAB-donor-CU1, the F1 interface of IAB-DU1 needs to be also migrated to the target path.

Step 6 (Optional): IAB-donor-CU1 triggers HO procedure for the remaining UEs and perform HO preparation for the remaining UEs with IAB-donor-CU2, and IAB-donor-CU1 sends the RRCreconfiguration for HO to the remaining UEs via the target path.

Step 7 (Optional): The remaining UEs perform random access procedure at IAB-DU2 and migrate to IAB-donor-CU2.

Step 8: F1 removal between IAB-DU1 and IAB-donor-CU1.

Proposal: RAN3 considers the baseline IAB inter-donor full migration procedure base on the procedure Option 1 or Option 2.

- Option 1: follow the sequence of (1) IAB-MT migration (2) IAB-DU2 setup with target CU (3) UE migration.

- Option 2: follow the sequence of (1) IAB-DU2 setup with target CU (2) UE migration and IAB-MT migration

# Conclusion

This contribution aims to discuss inter-donor full migration for mobile IAB-node. And following observations and proposals are concluded.

Proposal: RAN3 considers the baseline IAB inter-donor full migration procedure base on the procedure Option 1 or Option 2.

- Option 1: follow the sequence of (1) IAB-MT migration (2) IAB-DU2 setup with target CU (3) UE migration.

- Option 2: follow the sequence of (1) IAB-DU2 setup with target CU (2) UE migration and IAB-MT migration

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 112 e-meeting.

- Chairman notes of RAN3 117 e-meeting.