3GPP TSG-RAN WG3 Reunion Meeting #118	R3-226449

# Toulouse, lovely France, finally facetime, 14th - 18th November 2022

Agenda Item:	8.1

Source:	Ericsson

Title:	Current status on Rel-18 NPN discussions and outlook to expected specification work in RAN WG3

Document for:	Discussion

# 1	Introduction

We have received an LS from SA2 [1] on the progress and open issues for NPN enhancements in Rel-18 along the SA2 Feasibility Study for which SA2 is maintaining a TR 23.700-08 [2] and SA#97 has approved a respective Work Item [3], a WI proposal was also provided to RAN#97 [4] but not treated there.

The LS received from SA2 in [1] does not need any reply from RAN3, nevertheless, the sourcing company would like to provide some insight in SA2 work and impacts on RAN WGs’ work in Rel-18.

This document does not discuss the organisational and formal aspects of respective work in RAN WG3 but focusses on the technical aspects only.

# 2	Discussion

## 2.1	Aspects of SA2 work impacting specification work in RAN WG3

Key Issue #1 Enabling support for idle and connected mode mobility between SNPNs without new network selection

-	SA2 concluded that similar principles used for Equivalent PLMNs shall be applied for SNPNs as well

-	resulting in NGAP supporting equivalent SNPNs, including connected mode mobility

-	equivalent SNPNs taken into consideration for idle/connected network selection and cell(re)selection

-	confirmation that all Tracking Areas within a UEs Registration Area equivalent SNPNs need to supported homogenously.

Key Issue #2: Support of Non-3GPP access for SNPN

-	indication of selected NID from a N3IWF and TNGF via NGAP

Key Issue #3: Enabling NPN as hosting network for providing access to localized services

-	no NG-RAN impact identified

Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

-	ongoing discussions on this topic may result in additional information to be provided in SIB/on-demand

Key Issue #5: Enabling access to localized services via a specific hosting network

-	no NG-RAN impact identified

Key Issue #6: Support for returning to home network

-	no NG-RAN impact identified

Observation 1:	Conclusions on KI#1 on introduction of equivalent SNPNs (following similar principles as equivalent PLMNs) will require specification work in RAN2 and RAN3.

Observation 2:	Conclusions on KI#2 on support of Non-3GPP access for SNPN, has some little impact on specifications under RAN3 control.

Observation 3:	Discussion on KI#4 on enabling UE to discover, select and access NPN as hosting network and receive localized services, may result in RAN2 related work for system broadcast

## 2.2 Impact on RAN3 specifications

### TS 38.300 (RAN2 main responsibility)

-	Section 9.4 on Roaming and Access Restrictions contain specification text about equivalent PLMNs which will need to be made applicable for SNPNs as well.

-	Section 16.6.1 contains a statement about NR-NR DC with a single SNPN which needs to be revisited

-	Section 16.6.2 on mobility may requires RAN3 and RAN2 work for cell(re)selection and handover

### TS 38.413

-	will require some discussions on the “Target ID” in the Handover Required message. We haven’t included the NID IE there in Rel-16 due to lack of Equivalent SNPN support, but it seems, that this will be necessary now, in Rel-18.

### TS 38.413 and 38.423

-	will require obvious changes on the Mobility Restriction List

### TS 38.423 and TS 38.473

-	RRC (and respective information in Served Cell Information NR IE) support already shared RAN for multiple SNPNs, so the multiplicity of SNPNs is already supported

### TS 38.473

-	F1AP may be impacted based on the outcome of discussions on KI#4.

### TS 37.483

-	no impact, multiplicity of SNPNs is already supported

Observation 4:	We have identified specification work to be expected mainly for TS 38.300, 38.413 and 38.423.

Observation 5: Further impact on TS 38.473 may depend on outcome for KI#4

# 3	Conclusion and Proposals

We tried to summarize expected impact on RAN3 specification work and have observed the following:

Observation 1:	Conclusions on KI#1 on introduction of equivalent SNPNs (following similar principles as equivalent PLMNs) will require specification work in RAN2 and RAN3.

Observation 2:	Conclusions on KI#2 on support of Non-3GPP access for SNPN, has some little impact on specifications under RAN3 control.

Observation 3:	Discussion on KI#4 on enabling UE to discover, select and access NPN as hosting network and receive localized services, may result in RAN2 related work for system broadcast

Observation 4:	We have identified specification work to be expected mainly for TS 38.300, 38.413 and 38.423.

Observation 5: Further impact on TS 38.473 may depend on outcome for KI#4.

# 4	References

[1]	S2-2209860/R3-226167 "Progress and open issues for NPN enhancements in Rel-18", LSin at RAN3#118

[2]	TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2 (Release 18) ", latest version as available at RAN3#118

[3]	SP-220805 "New WID: Enhanced support of Non-Public Networks Phase 2", new WID approved at SP#97

[4]	RP-222291 "WID on Further Enhancement of Private Network Support for NG-RAN", WID proposal, noted at RAN#97