3GPP TSG-RAN WG3 Meeting #118	R3-226495

14 November – 18 November 2022

Toulouse, France

Title: 	(TP for AI/ML BLCR to TS38.423) Procedures for exchanging AI/ML-related information

Source: 	Ericsson

Agenda item:	12.2.2.1

Document Type:	Discussion and Decision

# Introduction

The general procedure for exchanging AI/ML-related information in the RAN was discussed during the RAN3#117bis-e meeting, and the following agreements and open issues were captured:

Procedures used for AI/ML support in the NG-RAN shall be use case agnostic.

The cases of i) Model Training and Model Inference at the NG-RAN and ii) Model Training at OAM and Model Inference at the RAN, make use of the same procedures, with the exception that procedures for exchange of training data and feedback data (which is not related to model performance feedback) will be different for i) and ii).

WA: The new procedure is introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer AI/ML data.

It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

The exact information to be included in this new procedure need to be discussed on a case by case basis.

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information.

It is FFS what is the exact name of the procedure.

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

Event-based reporting and how to determine an event are FFS.

The new procedure is non-UE associated procedure. If needed, the procedure can be used to capture UE-associated information.

The response message of the new procedure for AI/ML Related Information indicates if the requested information can be provided.

In this paper, we elaborate on the open issues related to information exchange for AI/ML in the RAN, regarding the standard impacts arising thereof.

# New XnAP procedures for exchanging AI/ML information

## Previous agreements and open issues

According to previous meetings progress, RAN3 agreed on defining a new procedure over Xn to exchange AI/ML-related information:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information

Furthermore, it was also agreed that the new procedure for signalling the said AI/ML-related information should follow the general paradigm of a subscription-reporting mechanism:

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

In addition, in the previous meeting the following was agreed:

Procedures used for AI/ML support in the NG-RAN shall be use case agnostic.

Observation 1:  RAN3 has agreed on defining a new XnAP procedure for reporting AI/ML-related information. The new procedure should be based on a subscription-reporting mechanism, like the Xn: Resource Status Reporting procedure and should be use-case-agnostic.

During RAN3#117bis-e a working assumption was discussed and supported by a majority of companies. However, the WA could not be agreed, and it has been minuted as a point to be continued:

WA: The new procedure is introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer AI/ML data.

We have argued in previous meetings that, in order to provide the flexibility required by AI/ML algorithms and to guarantee future-proofness, the new procedure should be defined in an information agnostic manner. 
The node which receives the request, i.e., the reporting node, does not need to know if the data will be used as, e.g., input or feedback. The only thing it needs to know is what data should be reported. So, the procedures should follow the general paradigm of a subscription-reporting mechanism, regardless of whether the type of information being transferred is an input (for the requesting node), an output (of the AI/ML model at the sending node), or feedback (for the requesting node). This can be an advantage if we consider that sometimes the very same information can be used for different purposes. Additionally, this would provide an NG-RAN node the necessary flexibility to subscribe for and receive/report, with the same procedure, different types of information needed by different AI/ML use cases. This avoids duplication of information signalling that is used for different purposes, e.g., as input for one AI/ML process and as feedback for another AI/ML process.

However, during RAN3#117bis-e some companies raised the question of whether there should be dedicated procedures for specific data types.

Related to this discussion, the following was noted:

It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

We would like to make a thorough analysis of this approach, taking AI/ML information one by one.

Case 1: the need for a dedicated procedure for Input/Training Data

One of the main principles agreed so far is that AI/ML algorithms and models are implementation specific. It is therefore perfectly possible that an AI/ML model uses, e.g., prediction information as inputs, or as feedback information or as training data. As an example, PRB utilization may be used as an input to derive a load balancing action and as feedback to determine the effect of the load balancing actions. It seems therefore not correct to design a dedicated procedure for Inputs/Training data because data used as inference input/training may at the same time consist of outputs/feedback. The result would be a duplication of information signalling over different procedures (e.g., one for input/training, one for feedback, etc). Besides, there is no interoperability advantage for the requesting and reporting node in specifying that a procedure is carrying input/training data as the requesting node knows what use it will make of such data independently of the procedure that reports them, while the reporting node knows it has to report such data, independently of the procedure used to report them. Fragmentation of AI/ML data signalling over many procedures unnecessarily complicates the AI/ML design.

Conclusion 1: there is no need to introduce a procedure purely dedicated to the reporting of Input/Training information

Case 2: the need for a procedure to collect predictions

Similarly to what it is discussed in Case 1, prediction information can be used for different purposes. For example, they can be used by an Actor to take specific actions; or they can be used by a Model Inference function as inference inputs, to derive new outputs. 
In some cases, predictions may also be used as feedback: for example an NG-RAN node may run its own prediction of the resource status of a neighbour cell and it could use the resource status prediction received for that neighbour cell as a preliminary feedback to validate the correctness of its predictions.
Signalling such information in a dedicated procedure would lead to duplication of information signalling over different procedures (e.g., one for input/training, one for feedback, etc), depending on whether predictions are used as inference inputs, as inference outputs (on the basis of which to take actions), or feedback.
As already mentioned, there seems to be no interoperability advantage in specifying a dedicated procedure for predictions either, as the requesting node knows what use it will make of such predictions independently of the procedure that reports them, while the reporting node knows it has to report such predictions, independently of the procedure used to report them. Again, fragmentation of AI/ML data signalling over many procedures unnecessarily complicates the AI/ML design.

Conclusion 2: there is no need to introduce a procedure purely dedicated to the reporting of prediction information

Case 3: the need for a procedure to collect feedback information

At the last RAN3 meeting, proposals were made to define such dedicated procedures with the purpose of associating feedback information to an AI/ML action. First of all, it should be mentioned that defining such dedicated procedure would incur in the same issues highlighted above, namely duplication of information signalling.

For example, Energy Efficiency metrics could be used as feedback information to an EE AI/ML algorithm (e.g., they could be used to see how cell deactivation has an impact on energy efficiency), however, they could be also used as model inference inputs (for example, an algorithm can take the EE of a neighbour as input to determine what is the best EE action to minimise energy consumption). Hence, defining a dedicated procedure for feedback information leads to information signalling duplication.

In another example, legacy resource status information (e.g., PRB usage, CAC) can be a perfect candidate for feedback information to a Load Balancing AI/ML algorithm (e.g., it can provide feedback on how certain mobility actions had an effect on load), but it can also be used as an input (e.g., given a neighbour cell load, certain actions are inferred aimed at balancing load). 
Moreover, this information is reported via legacy signalling. According to the agreements taken at the last RAN3 meeting, it is not possible to report such information in the new procedures introduced for AI/ML, and therefore it is not possible to associate this information explicitly to an AI/ML action, when they are used as feedback.

Even when looking at the use of a new procedure to associate feedback to AI/ML actions, it is very difficult to identify the action such feedback is related to and report such action in a dedicated signalling message. As an example, if the EE metric is used as feedback, how can it be possible to define the one or more actions that led to the new EE measurements? Energy Efficiency is the result of multiple actions within the network, and it is simply impossible to identify them all and encode them in a signalling message. 
We believe that a node requesting information for feedback purposes is also aware of the AI/ML actions such feedback should be associated to. Hence it is up to the requesting node to associate the feedback to the appropriate AI/ML action, to the extent possible. 
RAN3 can facilitate the association of feedback information to actions by providing assistance information with the feedback, e.g., by providing the APIDs associated to a UE together with the performance of handed over UEs.

Conclusion 3: there is no need to introduce a procedure purely dedicated to the reporting of feedback information. It is possible to facilitate association of feedback to AI/ML actions by signalling assistance information together with the feedback, e.g., by signalling the APIDs of handed over UEs together with the UE performance feedback

On the basis of the above conclusions, we propose the following:

Proposal 1:  The new procedure for exchange of AI/ML related information should be information agnostic, i.e., it should be the same regardless of whether the type of information being transferred is an input, an output, or feedback.

As a consequence, it is proposed to turn the working assumption minuted as “to be continued” into an agreement.

Proposal 2:  Agree that the new procedure introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer any type of AI/ML data.

Once again, we would like to iterate that there is no added value for the requesting node to explicitly denote if the information consists of an input, a training sample, an output, or feedback because the requesting node is totally aware of what the information will be used for and of which AI/ML action was triggered (at the requesting node) before feedback is received.

Namely, defining the new procedure for AI/ML information reporting as “data type agnostic” does not create any interoperability issue, while it simplifies the specifications. Besides, a distinction stating which information is inputs/training/output/feedback data may also be incorrect, as the same information may constitute, e.g., both an input and feedback.

Also note that the node requesting AI/ML data may require that data samples of different measurements are received at the same time. As an example, it may be requested that both a prediction information (e.g., predicted resource status) and feedback information (e.g., UE performance feedback) are received at the same time because they can be used as input to a model inference function (e.g., based on predicted neighbour load and current UE performance at neighbour cell, a mobility action is inferred). With a single procedure this is possible. With different procedures, data samples of different measurements may be subject to different delays. Hence, timing among data samples may become unsynchronised, which has negative effects on, e.g., inference and feedback.

Based on that we propose that there is no need to introduce another procedure to transfer different types of AI/ML data.

Proposal 3:  We propose there is no need to introduce any additional procedure to transfer different types of AI/ML data.

Another agreement that was taken for the new procedure is as follows:

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

A related FFS was also captured:

Event-based reporting and how to determine an event are FFS.

We believe that one-time and periodic reporting are, at least for the time being, the measurement types needed to support AI/ML use cases. We do not exclude that some form of event-based measurements could also be included in the same framework. For example, if UE feedback information is requested then such UE performance measurement could be reported when a UE hands over for AI/ML reasons to a cell of the reporting node. After the UE is handed over, the reporting could be done according to the periodicity established in the AI/ML Data Reporting procedure, e.g., every 500ms. The reporting can be done for a time window when the UE context is still available at the source, e.g., for a given configured time or until signalling of UE context Release. As it can be seen, event-based reporting can be implicitly supported by a procedure reporting periodically. The figure below explains how event-based reporting can be achieved via reporting in a single procedure:

Figure 1: Example of event-based reporting via a periodic reporting procedure

In Figure 1, the AI/ML Assistance Data Reporting procedure is triggered to request reporting of UE Performance Measurements. The procedure can be triggered with a given periodicity, e.g., 500ms. However, given that UE Performance Measurements are available only upon the occurrence of an AI/ML triggered event, the reporting node will not report UE Performance Measurements until completion of an HO for AI/ML reasons, from gNB-1 Cell1 to gNB-2 Cell 2.

Upon completion of such handover, gNB-2 will be able to measure the performance of the UE in Cell 2. Hence, gNB-2 is able to generate AI/ML ASSISTANCE DATA UPDATE messages, where the UE performance is reported. Such messages can be generated with the periodicity configured via AI/ML Assistance Data Reporting procedures. The AI/ML ASSISTANCE DATA UPDATE messages can be signalled for a fixed time window, namely from the time when the HO is completed, and measurements are available till the time when gNB-2 signals the UE context Release message. As an alternative, a configured amount of time can be specified to define the time window during which the AI/ML ASSISTANCE DATA UPDATE messages will be sent, i.e., a timer similar or equal to the Tstore_UE_cntxt timer (see TS38.300). In this case the AI/ML ASSISTANCE DATA UPDATE messages can be signalled from the time when the HO is completed, and measurements are available till Tstore_UE_cntxt expires.

It is worth pointing out that the approach described above is much simpler than enabling event-based reporting. The reason is that event-based reporting would entail the definition of events.

In the context of AI/ML an “event” is very much connected to the implementation of the AI/ML algorithm. For example, some implementations may consider the lack of training samples in a specific measurement value region as an event to trigger AI/ML data reporting; some other implementation may consider as an event the occurrence of measurements that exceed a certain deviation level from the average.

In essence, the definition of events is complex, and it would reveal the implementation of the AI/ML algorithm.

For this reason, we believe that adapting the new procedure for AI/ML data reporting to the very few events (e.g., UE mobility) that we have identified for the current use cases, is the simplest and most effective way to support AI/ML in NG-RAN.

Proposal 4:  Event-based reporting is not needed.

There might be cases also where the reporting node is not able to continue with the reporting either completely or not according to the configuration. For example, the reporting node might not be able to report at the agreed periodicity, or it might not be able to report at all. The reporting node could indicate unavailability of measurements in the AI/ML ASSISTANCE DATA UPDATE message, to inform the requesting node that data will not be available.

Proposal 5:  The reporting node can use the AI/ML ASSISTANCE DATA UPDATE messages to indicate potential issues preventing the reporting.

## Possible implementation

In light of the conclusions and proposals derived above, and as an example, the AI-ML ASSISTANCE DATA REQUEST message, which is part of the AIML Assistance Data Reporting Initiation procedure, is shown here:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the assistance data is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement object the NG-RAN node 2 is requested to report. First Bit = Predicted Radio Resources Periodic, Second Bit = Current Energy Efficiency Periodic, Third Bit = Predicted Energy Efficiency Periodic (FFS), Fourth Bit = UE Performance Indicator. Other bits shall be ignored by the NG-RAN node 2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting. | YES | ignore |
| Cause Value List |  | 0..1 |  |  | YES | ignore |
| >Cause Value Item |  | 1..<MaxNumberofCause> |  |  | - | - |
| >>Cause | M |  | 9.2.3.2 | Cause value of handovers for which UE Performance Feedback is requested | - | - |

As a further example, the AI-ML ASSISTANCE DATA UPDATE message, which is part of the AIML Assistance Data Reporting procedure, is shown below. We note the proposed UE Assistant Identifier IE that can be used to signal UE-associated information in this non-UE-associated procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Energy Efficiency | O |  | INTEGER (0..100) | Value 0 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| Predicted Energy Efficiency (presence of this IE is FFS) | O |  | INTEGER (0..100) | Value 0 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.3.xx | Indicates the predicted values of reported resource measurements |  |  |
| >>UE Performance Indicator List | O | 1 .. < maxnoofFeedbackUEs > |  |  |  |  |
| >>>UE Performance Measurements |  |  | 9.2.3.yy |  |  |  |
| >>>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

One important detail is how the Predicted Radio Resources IE should be represented. The simplest way would be to define the Predicted Radio Resources IE as a sequence of predicted values for the IEs reported in the Xn: Resource Status Update message. According to the latest agreements on the information to consider for predicted resource status, one example of how the Predicted Radio Resources IE may look like is shown below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator (FFS) | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group (FFS) | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity (FFS) | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | –- |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

While the above tabulars are not definitive and need to be discussed in RAN3 and refined, they can be taken as a baseline for further work.

Proposal 6:  Take as baseline the above tabulars for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the XnAP Resource Status Reporting Initiation and Resource Status Reporting procedures.

# Conclusion

In this contribution, we have discussed the agreements and the open issues related to information exchange for AI/ML in the RAN. Corresponding observations and proposals are reported below.

Conclusion 1: there is no need to introduce a procedure purely dedicated to the reporting of Input/Training information

Conclusion 2: there is no need to introduce a procedure purely dedicated to the reporting of prediction information

Conclusion 3: there is no need to introduce a procedure purely dedicated to the reporting of feedback information. It is possible to facilitate association of feedback to AI/ML actions by signalling assistance information together with the feedback, e.g., by signalling the APIDs of handed over UEs together with the UE performance feedback

Proposal 1:  The new procedure for exchange of AI/ML related information should be information agnostic, i.e., it should be the same regardless of whether the type of information being transferred is an input, an output, or feedback.

Proposal 2:  Agree that the new procedure introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer any type of AI/ML data.

Proposal 3:  We propose there is no need to introduce any additional procedure to transfer different types of AI/ML data.

Proposal 4:  Event-based reporting is not needed.

Proposal 5:  The reporting node can use the AI/ML ASSISTANCE DATA UPDATE messages to indicate potential issues preventing the reporting.

Proposal 6:  Take as baseline the above tabulars for the definition of the AIML Assistance Data Reporting Initiation and AIML Assistance Data Reporting, resembling the XnAP Resource Status Reporting Initiation and Resource Status Reporting procedures.

A CR to TS 38.423, mirroring the proposals above as well as the proposals made in papers R3-226496, R3-226500, is available in the appendix.

# Appendix - CR for TS 38.423

Below, a draft CR is presented where the proposals made in this paper as well as in R3-226496, R3-226500 are reflected

3GPP TSG-RAN WG3 Meeting #118	R3-22xxxx

14 November – 18 November 2022

Toulouse, France

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to TS38.423: Support for AI/ML in NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to fulfil the objectives of the WID in RP-220635 (NR_AIML_NGRAN-Core) a number of changes are required. This CR implements such changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for AI/ML based mechanisms in support for Network Energy Saving, Load Balancing and Mobility Optimisation use cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML based procedures and functions cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.1, 8.4.xx, 8.4.yy, 9.1.3.xx, 9.1.3.ww, 9.1.3.yy, 9.1.3.zz, 9.2.3.2 and 9.2.3.xx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< Start of CHANGEs >>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe 	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

<<<<<< NEXT CHANGE >>>>>>

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI-ML Assistance Data Reporting Initiation | AI-ML ASSISTANCE DATA REQUEST | AI-ML ASSISTANCE DATA RESPONSE | AI-ML ASSISTANCE DATA FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI-ML Assistance Data Reporting | AI-ML ASSISTANCE DATA UPDATE |

<<<<<< NEXT CHANGE >>>>>>

### 8.4.xx	AI-ML Assistance Data Reporting Initiation

#### 8.4.xx.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML assistance data to another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.xx.2	Successful Operation

Figure 8.4.xx.2-1: AI-ML Assistance Data Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI-ML ASSISTANCE DATA REQUEST message to NG-RAN node2 to start AI/ML assistance data reporting, stop assistance data reporting, or add cells to report for assistance data. Upon receipt, NG-RAN node2:

-	shall initiate the requested assistance data reporting according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells assistance data reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the assistance data reporting initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If assistance data reporting is already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the AI-ML ASSISTANCE DATA REQUEST message and the Report Characteristics IE indicates cell specific assistance data, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "add" in the AI-ML ASSISTANCE DATA REQUEST message, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested AI/ML assistance data, it shall initiate the assistance data reporting as requested by NG-RAN node1 and respond with the AI-ML ASSISTANCE DATA RESPONSE message.

Interaction with other procedures

When starting AI/ML assistance data reporting, the Report Characteristics IE in the AI-ML ASSISTANCE DATA REQUEST indicates the type of objects NG-RAN node2 shall perform assistance data reporting on. For each cell, NG-RAN node2 shall include in the AI-ML ASSISTANCE DATA UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resources Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1";

- 	the UE Performance Indicator List IE for UEs handed over from NG-RAN node1 to the cells in the Cell To Report List IE with handover cause indicated in the Cause Value List IE, if the fourth bit, "UE Performance Indicator" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1". It is FFS for how long the periodic reporting of the UE Performance Indicator List IE shall last.

For the NG-RAN node2, NG-RAN node2 shall include in the AI-ML ASSISTANCE DATA UPDATE message:

-	the Energy Efficiency IE, if the second bit, "Current Energy Efficiency Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1";

-	the Predicted Energy Efficiency IE (presence of this IE is FFS), if the third bit, "Predicted Energy Efficiency Periodic" of the Report Characteristics IE included in the AI-ML ASSISTANCE DATA REQUEST message is set to "1".

If the Reporting Periodicity IE in the AI-ML ASSISTANCE DATA REQUEST is present, this indicates the periodicity for the reporting of AI/ML assistance data. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If NG-RAN node2 is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the AI-ML ASSISTANCE DATA REQUEST message and set to “partial success allowed”, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the AI-ML ASSISTANCE DATA RESPONSE message.

#### 8.4.xx.3	Unsuccessful Operation

Figure 8.4.xx.3-1: AI-ML Assistance Data Reporting Initiation, unsuccessful operation

If the Partial Success Indicator IE is present in the AI-ML ASSISTANCE DATA REQUEST message and set to “partial success not allowed”, and if any of the requested assistance data cannot be initiated, NG-RAN node2 shall send the AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the Partial Success Indicator IE is present in the AI-ML ASSISTANCE DATA REQUEST message and set to “partial success allowed”, and all of the requested assistance data cannot be initiated, NG-RAN node2 shall send the AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

#### 8.4.xx.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI-ML ASSISTANCE DATA RESPONSE message or the AI-ML ASSISTANCE DATA FAILURE message, the NG-RAN node1 may reinitiate the AI-ML Assistance Data Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI-ML ASSISTANCE DATA REQUEST message is identical to the content of the previously unacknowledged AI-ML ASSISTANCE DATA REQUEST message.

If the NG-RAN node2 receives an AI-ML ASSISTANCE DATA REQUEST message which includes the Registration Request IE set to "add" or "stop" and if the NG-RAN node2 Measurement ID value received in the AI-ML ASSISTANCE DATA REQUEST message is not used, the NG-RAN node2 shall initiate AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the AI-ML ASSISTANCE DATA REQUEST message, then NG-RAN node2 shall initiate an AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives an AI-ML ASSISTANCE DATA REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going assistance data reporting, then NG-RAN node2 shall initiate an AI-ML ASSISTANCE DATA FAILURE message with an appropriate cause value.

If the initiating NG-RAN node1 receives the AI-ML ASSISTANCE DATA RESPONSE message including the Measurement Initiation Result IE containing no admitted measurements, the NG-RAN node1 shall consider the procedure as failed.

### 8.4.yy	AI-ML Assistance Data Reporting

#### 8.4.yy.1	General

This procedure is initiated by an NG-RAN node to report the results of assistance data admitted by the NG-RAN node following a successful AI-ML Assistance Data Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.yy.2	Successful Operation

Figure 8.4.yy.2-1: AI-ML Assistance Data Reporting, successful operation

NG-RAN node2 shall report the results of the admitted assistance data in AI-ML ASSISTANCE DATA UPDATE message. The admitted assistance data were successfully initiated during the preceding AI-ML Assistance Data Reporting Initiation procedure.

#### 8.4.yy.3	Unsuccessful Operation

Not applicable.

#### 8.4.yy.4	Abnormal Conditions

Void

<<<<<< NEXT CHANGE >>>>>>

#### 9.1.3.xx	AI-ML ASSISTANCE DATA REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested assistance data reporting in support to AI/ML functions, according to the parameters given in the message.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the assistance data is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement object the NG-RAN node 2 is requested to report. First Bit = Predicted Radio Resources Periodic, Second Bit = Current Energy Efficiency Periodic, Third Bit = Predicted Energy Efficiency Periodic (FFS), Fourth Bit = UE Performance Indicator. Other bits shall be ignored by the NG-RAN node 2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting. | YES | ignore |
| Cause Value List |  | 0..1 |  |  | YES | ignore |
| >Cause Value Item |  | 1..<MaxNumberofCause> |  |  | - | - |
| >>Cause | M |  | 9.2.3.2 | Cause value of handovers for which UE Performance Feedback is requested | - | - |
| Partial Success Indicator | M |  | ENUMERATED(partial success allowed, partial success not allowed) | Indicates if partial success is allowed or not allowed | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.ww	AI-ML ASSISTANCE DATA RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested assistance data, for some or all of the measurement objects indicated in the bitmap included in the assistance data request, is successfully initiated.

Direction: NG-RAN node2 ® NG-RAN node1

| IE/Group Name | Presence | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Measurement Initiation Result |  |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Failure Cause List |  |  | 0..1 |  | Indicates that eNB2 could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>Measurement Failed Report Characteristics | M |  |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resources Periodic, Second Bit = Current Energy Efficiency Periodic, Third Bit = Predicted Energy Efficiency Periodic (FFS), Fourth Bit = UE Performance Indicator Periodic. Other bits shall be ignored by the NG-RAN node1. | – |  |
| >>>Cause | M |  |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |
| Criticality Diagnostics | O |  |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |

#### 9.1.3.yy	AI-ML ASSISTANCE DATA FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested assistance data the measurement cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.zz	AI-ML ASSISTANCE DATA UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested assistance data.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Energy Efficiency | O |  | INTEGER (0..100) | Value 1 indicates the minimum measured Energy Efficiency and 100 indicates the maximum measured Energy Efficiency. Energy Efficiency should be measured on a linear scale |  |  |
| Predicted Energy Efficiency (presence of this IE is FFS) | O |  | INTEGER (0..100) | Value 1 indicates the minimum Predicted Energy Efficiency and 100 indicates the maximum Predicted Energy Efficiency. Predicted Energy Efficiency should be measured on a linear scale |  |  |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.3.xx | Indicates the predicted values of reported resource measurements |  |  |
| >>UE Performance Indicator List | O | 1 .. < maxnoofFeedbackUEs > |  |  |  |  |
| >>>UE Performance Measurements | M |  | 9.2.3.yy |  |  |  |
| >>>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofFeedbackUEs | Maximum no. of UEs reporting feedback information. Value is FFS |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, AI/ML Triggered Handover) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

The meaning of the different cause values is specified in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the NG-RAN node2 Proposed Mobility Parameters IE is too low or too high. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| RSN not available for the UP | The redundant user plane resources are not available. |
| NPN Access denied | Access denied, or release is required, due to NPN reasons. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the S-NG-RAN node for UE power saving purpose. |
| Not existing NG-RAN node2 Measurement ID | The action failed because the NG-RAN node2 Measurement ID is not used. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| AI/ML Triggered Handover | The handover is triggered as a result of an AI/ML action |

| Transport Layer cause | Meaning |
| --- | --- |
| Transport resource unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | NG-RAN node control processing overload. |
| Hardware Failure | NG-RAN node hardware failure. |
| Not enough User Plane Processing Resources | NG-RAN node has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to NG-RAN node equipment. |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.3.xx	Predicted Radio Resources

This IE provides prediction values for Radio Resource parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | IE/Group Name | Presence |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE | – |  |
| Predicted TNL Capacity Indicator (FFS) | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE | – |  |
| Predicted Composite Available Capacity Group (FFS) | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE | – |  |
| Predicted Slice Available Capacity (FFS) | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE | – |  |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active UEs IE | – |  |
| Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE | – |  |

#### 9.2.3.yy	UE Performance Measurements

The UE Performance Measurements IE indicates performance measurements for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | 9.2.3.4 | Average overall user plane UE throughput in DL |
| Average UE Throughput UL | O |  | 9.2.3.4 | Average overall user plane UE throughput in UL |
| Average Packet Delay | O |  | FFS | Average value for the delay that a packet may experience. |
| Average Packet Error Rate | O |  | 9.2.3.13 | Average Packet Error Rate |
| Average RAN Visible QoE (FFS) | O |  | FFS | Average RAN Visible QoE experienced by the UE on all the active services for which QoE is collected at the time of reporting |