3GPP TSG-RAN WG3 #118	R3-226601

14th – 18th Nov 2022

Toulouse, France

Agenda item:		14.2

Source:	LG Electronics Inc.

Title:	(TP for NR_Mob_enh2 BL CR for TS 38.401) Open issues for L1/L2 based inter-cell mobility

Document for:		Other

# 1. Text Proposal for TS 38.401

Beginning of Text Proposal to TS 38.401

#### 8.2.1.X	Intra-gNB-DU L1/L2 based inter-cell Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 based inter-cell mobility. Figure 8.2.1.x-y shows the intra-gNB-DU L1/L2 based inter-cell mobility procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 based inter-cell Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 inter-cell mobility configuration.

- WA: The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells. FFS: whether parallel UE Context Modification procedure should be used rather than a candidate list in a single message.

- WA: If the request for configuring L1/L2 inter-cell mobility is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the L1/L2 inter-cell mobility configuration, which may contain a configuration per target candidate (e.g. including the lower layer RRC configurations of the candidate cells for L1/L2 inter-cell mobility. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the UE CONTEXT MODIFICATION REQUIRED message to the gNB-CU containing the suggested cells within the candidate cells provided by the gNB-CU.

- If the request for modifying the suggested cells is accepted, the gNB-CU responds with a UE CONTEXT MODIFICATION CONFIRM message including the modified candidate cell(s).

- The gNB-DU sends the L1/L2 inter-cell mobility command to the UE.

- FFS: How the gNB-DU detects the UE access to the target cell is up to RAN2.

- The gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message. Th target cell ID is included.

- FFS: The gNB-CU may send the UE Context Modification message to the gNB-DU to release the resources of prepared cells.

- FFS: The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

End of Text Proposal to TS 38.401