3GPP TSG-RAN WG3 Meeting #118	R3-226741

# Toulouse, 14 – 18 November 2022

Agenda item:		12.2.1

Source:	ZTE

Title:	TP to 38.300 for the stage2 impacts on AI/ML procedures

Document for:		Discussion and Approval

Introduction

In this contribution, we provide the TP to 38.300 on the stage2 descriptions on the AI/ML procedures.

- TP to 38.300

<Unchanged Text Omitted>

- X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

- X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

- X.X.3 Procedures for AI/ML NG-RAN

Following are the procedures to support AI/ML NG-RAN, including AI/ML DATA COLLECITON procdure used for AI/ML input information, AI/ML PREDICTED INFORMATION procedure used for AI/ML predicted information, and AI/ML FEEDBACK INFORMATION procedure used for AI/ML feedback information.

AI/ML DATA COLLECTION PROCEDURE:

NG-RAN node initiates the AI/ML DATA COLLECTION procedure to request the input information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

AI/ML PREDICTED INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML PREDICTED INFORMATION procedure to request the predicted information from other NG-RAN nodes to perform AI/ML function (e.g., AI/ML model training/inference).

AI/ML FEEDBACK INFORMATION PROCEDURE:

NG-RAN node initiates the AI/ML FEEDBACK INFORMATION procedure to request the feedback information from other NG-RAN nodes to evaluate or monitor the AI/ML model performance.