3GPP TSG-RAN WG3 Meeting #119	          R3-230887

Athens , GR，27th February –3rd March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Network controlled repeater |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, Samsung,CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-core |  |  |  |  |  | Date: |  |  | 2022-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Rel-18 work item on Network Controlled Repeater has been approved. Update the stage 2 description for Network Controlled Repeater. Capture the agreement on NCR made by RAN3: OAM-NCR connectivity can be provided via PDU session. Take Solution 3 as the basis for NCR management. Capture the following RAN1 agreement: The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link: Option-2: The information is informed to gNB and NCR via OAM - Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor). - Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation. - The beam index in SCI corresponds to the configured beam(s) sequentially. Capture the agreements made in RAN3#119: The NCR may be configured with a list of allowed and/or forbidden cells. RAN validation is not supported in R18. The NG-RAN node selects an appropriate AMF for a NCR accesses to the network with the assumption that the NCR indication is received from NCR in MSG5. gNB-CU shall indicate the NCR authorization info explicitly to gNB-DU once a NCR device is authorized by UE context setup request message and UE context modification request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following functionalities are introduced for Network Controlled Repeater: - Network Controlled Repeater OAM connectivity - Network Controlled Repeater management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement for Network Controlled Repeater can not be supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X, X.Y, X.Y1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0890 TS 38.401 CR 0274 TS 38.473 CR 1109 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Re-submission to RAN3#119. Rev 2: Capture agreements made in RAN3#119. |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGE >>>>>>

# X Network controlled repeater

## X.Y OAM aspects

The transport connection between the NCR-node and its OAM may be provided by the NCR-MT’s PDU session. NCR may be configured a list of allowed and/or forbidden cells.

- The physical beam(s) used by NCR-Fwd for access link may be provided by OAM to the gNB and NCR for its operation, which will be sequentially mapped to the beam index in side control information (SCI).The information to characterize the supported physical beam of NCR-Fwd is based on implementation (e.g., declaration from NCR vendor).

## X.Y1 Network controlled repeater manegement

NCR identification is performed at RAN side, and NCR authorization is performed at CN side.

The general procedure of the NCR management is illustrated in below figure:

Figure X.Y-1. NCR management

The gNB selects an appropriate AMF for NCR accessing upon the gNB receives the NCR indication from NCR.

CN performs NCR authorization, and provides NCR authorized information to the gNB.

<<<<<< END OF CHANGE >>>>>>