3GPP TSG-RAN WG3 #119bis-e																		R3-231320

17th – 26th Apr 2023

Online

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for RV-QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#118 meeting, the NR QoE supporting in NR-DC was discussed. The RVQoE related agreements and open issues are captured in chair Note[1] as below:

To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC:

For the first RVQoE configuration, it is blindly configured by MN or SN.

From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session.

After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified.

If a node receives an RVQoE report from a UE in NR-DC, and determines that the bearers for the application session are also or only provided by the peer node, this node can send the received RVQoE report to the peer node.

Turn WA to agreement: SN can send an RVQoE configuration to the UE.

This contribution will further discuss the supporting on the RV QoE in NR-DC based on the agreements.

2		Discussion

In last meeting, we turned the WA to agreement: SN can send an RVQoE configuration to the UE. But how to send is FFS if SN decides configuring RVQoE. There are two possible ways the SN sending its generated RVQoE configuration to UE. One is the SN send to UE directly via SRB3 and split SRB1. Another one is send to MN and MN forward to UE with or without modification. To have more flexible implementation, both ways should be supported. The SN should support sending the RVQoE configuration via SRB3

Proposal 1: SN can send RVQoE configuration directly to UE via SRB3 or via split SRB1

Proposal 2: SN can send RVQoE configuration to UE via MN

If the SN sends the generated RVQoE configuration to UE via MN, whether the MN can modify this configuration should be further analysed. We may have sight on the current R17 RVQoE configuration specified in 38.331[2]. We may find only three parameters including in R17 configuration. For these parameters, we don’t think it is necessary for MN to modify the reporting metrics based on SN generated configuration. But the report periodicity may be change or not to align the two nodes if MN also configures the RVQoE. Also the MN may encode the SN generated configuration in the same RRC message with MN generated configuration.

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17    ENUMERATED {ms120, ms240, ms480, ms640, ms1024}   OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17    INTEGER (1..8)            				    OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                         		    OPTIONAL, -- Need M

...

}

Proposal 3: MN can modify the SN generated RVQoE configuration

Proposal 4: MN may encode RVQoE configuration generated by MN and SN in same RRC message.

If both MN node and SN configure the RV-QoE for the same service type and QoE reference, we need more thinking. In R17, we just specified two parameters for RV-QoE. In the future release, there may be more parameters introduced. If both MN and SN configure the RV-QoE for same service type and QoE reference, the value of the parameters may be difference if there is no any coordination between two nodes. If like this, it introduces more complex in UE app layer and in report sending. So we propose the MN and SN may select same value for a parameter.

Proposal 5: MN and SN may aligned the value of parameters if both MN and SN configure same RV-QoE parameter

After the MN configures the legacy QoE (either receive from AMF or OAM), the MN may configure the RV-QoE for MN. The MN also need send the available metrics to SN for the SN RV-QoE configuration potential needed. Also if the SN receives the M-based QoE configuration from OAM, the SN should forward the available metrics to MN for the MN RV QoE configuration when SN pre-select this UE and sends the legacy QoE configuration to MN for the coordination.

Proposal 6: The node that received the QoE configuration from the AMF/OAM may send the available RVQoE metrics to other node.

As RAN2 agreement, SN will configure the RV-QoE with same RRC ID with MN for same QoE reference. How the SN sends the RV-QoE configuration to UE may have several options as we proposed in P1 and P2: option1, Via SRB3 directly to UE; option2, via MN to UE. If the option2 is used, the configuration from SN should carry the indication of SN node. So the UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.  In opposite direction, the SN also need provide the RRC ID of its legacy QoE to MN for MN RV-QoE configuration

Proposal 7: The MN sends the RRC ID of its legacy QoE to SN for SN RV QoE configuration

Proposal 8: The SN sends the RRC ID of its legacy QoE to MN for MN RV QoE configuration

Regarding the configuration and reporting node, in the past several meetings, We agree the RV-QoE report can be separately sent to the configuration initialled node directly or send to another node and forwarding to the configuration initialled node, we agreed that ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. But FFS on how to ensure.

In last meeting, we agree the RV QoE configuration can be configured blindly. We may understand the situation as below: the legacy QoE is configured by either MN or SN. The two nodes are aware of the legacy QoE configuration via coordination procedure. So then the node (MN or SN) wants to configure the RV-QoE for the service type or QoE reference even though it doesn’t know whether the application service data will go through it. The UE receive the configuration and perform the RV-QoE related measurement. And send the report to RAN node. So then we have some question on the reporting sending. Which node does the report send? We may have two options:

Option1: send the report to the RVQoE initial configured node.

Option2: send the report to the node which service associated QoS flow located.

For option1, the UE app layer and AS layer should be aware of the configuration initialled node ID. So the RV-QoE configuration and report should carry the indication of the node. And the UE app layer may be informed the configuration initialled node and the APP layer send the report to AS layer carrying the indication of the node. So then the report can be correctly sent to the configuration initialled node.  And follow our agreement made in last meeting, the initial node may modified the configuration and the report sent to the service located node in next time reporting.

Proposal 9: UE may differentiate the RV-QoE configuration and report according node indication.

For option2, the UE APP layer will send the RV QoE report with PDU session ID plus QoS flow ID to AS layer and then the AS layer send the report to the node which the QoS flow is located.

Proposal 10: UE may send the RV-QoE report to corresponding node according QoS flow information

In this option2, we have several scenarios.

- The service associated QoS flow is in the configuration initialled node

- The service associated QoS flow is not in the configuration initialled node

- The service associated QoS flow is split in the two nodes

For the scenario 1, it is more simple and straightforward. We don’t need do anything. For scenario 2, if the report receiving node doesn't configure the RV-QoE parameter within report, the report may be discarded or kept for its using. If the report receiving node has configured the same RV-QoE parameter, we proposed in Proposal 5, if agreed, the scenario equals scenario 1. But if the value is different, such as periodicity or event trigger, we should further study it.

Proposal 11: if the report receiving node doesn't configure the RV-QoE parameter within report, the report may be discarded or kept for its using

Proposal 12: if the report receiving node configures the RV-QoE parameter with different configured value from report, we should do more study.

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: SN can send RVQoE configuration directly to UE via SRB3 or via split SRB1

Proposal 2: SN can send RVQoE configuration to UE via MN

Proposal 3: MN can modify the SN generated RVQoE configuration

Proposal 4: MN may encode RVQoE configuration generated by MN and SN in same RRC message.

Proposal 5: MN and SN may aligned the value of parameters if both MN and SN configure same RV-QoE parameter

Proposal 6: The node that received the QoE configuration from the AMF/OAM may send the available RVQoE metrics to other node.

Proposal 7: The MN sends the RRC ID of its legacy QoE to SN for SN RV QoE configuration

Proposal 8: The SN sends the RRC ID of its legacy QoE to MN for MN RV QoE configuration

Proposal 9: UE may differentiate the RV-QoE configuration and report according node indication.

Proposal 10: UE may send the RV-QoE report to corresponding node according QoS flow information

Proposal 11: if the report receiving node doesn't configure the RV-QoE parameter within report, the report may be discarded or kept for its using

Proposal 12: if the report receiving node configures the RV-QoE parameter with different configured value from report, we should do more study.

4		Reference

- RAN3_119_agenda_20230303_EOM2

- TS38.331 NR; Radio Resource Control (RRC) protocol specification