3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231431

17th – 26th April 2023

Agenda item:	 11.3

Source:		Lenovo

Title:		Discussion on QoE measurement in NR-DC

Document for:		Discussion and Decision

- Introduction

This paper further discusses the following objectives in QoE enhancement WID RP-221803 [1]:

-	Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

-	Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

-	Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

-	Specify the QoE measurement continuity in mobility scenarios in NR-DC.

-	Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

- Discussion

- Basic Signalling for QoE configuration

- In RAN3#119 meeting, it was agreed that:

-	If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.

-	The MN should inform the SN that a UE is configured with an m-based QoE measurement.

- One straightforward way is that the SN sends m-based QoE measurement configuration request to the MN by the existing SN Modification Required procedure. We don’t see the necessity to define a new procedure. The QMC Configuration Information IE can be added in S-NODE MODIFICATION REQUIRED message to achieve this.

- In RAN3#119 meeting, it was also agreed that:

-	For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

-	The MN shall send the configuration information to the UE, or

-	The SN should send the configuration to the UE directly, or

-	The SN should send the configuration information to the UE via the MN (inside a container).

- In this case, the MN needs to add the following information in the S-NODE MODIFICATION CONFIRM message:

-	QoE reference;

-	when the QoE measurement is allowed, the MN needs to notify the SN whether:

-	The MN shall send the configuration information to the UE, or

-	The SN should send the configuration to the UE directly, or

-	The SN should send the configuration information to the UE via the MN (inside a container).

-	Information for QoE measurement is not allowed e.g., due to the QoE measurement has been configured to UE by the MN

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration:

- The SN sends the QMC Configuration Information IE can be added in S-NODE MODIFICATION REQUIRED message;

- The MN needs to add the following information in the S-NODE MODIFICATION CONFIRM message:

c1) QoE reference;

c2) when the QoE measurement is allowed, the MN needs to notify the SN whether:

-	The MN shall send the configuration information to the UE, or

-	The SN should send the configuration to the UE directly, or

-	The SN should send the configuration information to the UE via the MN (inside a container).

c3) Information for QoE measurement is not allowed e.g., due to the QoE measurement has been configured to UE by the MN;

- In RAN2#121 meeting, it was agreed that:

-	RRC configuration determines to which node UE sends the QoE report.  It is possible to change the reporting leg via RRC signalling after it has been configured.

-	Split SRB for QoE reporting is not supported (unless serious problems are identified).

-	Define new SRB (“SRB5”) for the QoE reporting to SN. SRB4 can only be configured for MCG (as in Rel-17). The priority of “SRB5” is lower than SRB1 or SRB3.

- For signalling based QoE activation, RAN3 has agreed that

-	MN is responsible to configure the s-based QoE to UE.

-	QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session.

When the MN receives the QoE measurement activation from CN for signalling based QoE activation, the MN decides which leg is used for the QoE reporting e.g., according to load situation in MN leg or SN leg.

- In NR-DC case, both MN leg and SN leg can be used for the QoE measurement reports. Since the data volume of QoE measurement report would be large in case of network congestion, it is better to have a flexible QoE reporting leg configuration, selection and switching e.g., based on the network overload status or due to fast MCG link recovery.  For example, MN may decide that the UE only sends the QoE measurement reports to MN directly. In this case, the UE uses SRB4 for the QoE measurement reporting as same with Rel-17. For another example, the MN may decide the UE only sends the QoE measurement reports to SN. In this case, MN needs to indicate requested SRB5 for QoE measurement reports to SN. And SN needs to acknowledge whether SRB5 is available or not to the MN. The requested SRB5 for QoE measurement reporting can be included in Xn-AP S-Node Addition Request message in case of SN addition or XnAP S-Node Modification Request message in case of SN modification. The SN may send the admitted SRB5 to the MN in Xn-AP S-Node Addition Acknowledge or S-Node Modification Request Acknowledge message.

For S-based and MN initialized m-based QoE measurement configuration, if the MN that  decides the UE sends the QoE measurement report to the SN:

- The MN indicates ‘requested SRB5 for QoE measurement reporting’ in Xn-AP S-Node Addition Request message or XnAP S-Node Modification Request message.

- The SN responds ‘admitted SRB5 for QoE measurement reporting’ to the MN in Xn-AP S-Node Addition Acknowledge or S-Node Modification Request Acknowledge message.

- For SN initialized m-based QoE measurement, it makes sense that the SN decides the QoE reporting leg. For example, the SN may decide that the UE only sends the QoE measurement reports to SN directly. In this case, the UE uses SRB5 for the QoE measurement reporting. For another example, the SN may decide the UE only sends the QoE measurement reports to the MN. In this case, the SN needs to indicate requested SRB4 for QoE measurement reports to MN. And MN needs to acknowledge whether SRB4 is available or not to the MN. The requested SRB4 for QoE measurement reporting can be included in Xn-AP S-Node Modification Required. The NN may send the ‘available SRB5’ to the SN in Xn-AP S-Node Modification Confirm message.

For SN initialized m-based QoE measurement configuration, if the SN decides that the UE sends the QoE measurement report to the MN:

- The SN indicates ‘requested SRB4 for QoE measurement reporting’ in Xn-AP S-Node Modification Required message.

- The MN responds ‘available SRB5 for QoE measurement reporting’ to the SN in Xn-AP S-Node Modification Confirm message.

- RRC ID allocation

- In RAN3#119 meeting, it was agreed that:

-	The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

-	FFS on whether a pool of RRC ID is splitted between MN and SN or whether it is per measurement.

- There are two solutions for RRC ID allocation:

- Solution 1: Value range-based solution

- In solution 1, the MN is responsible for RRC identifier (i.e., measConfigAppLayerId) value range allocation for QoE measurement. The MN allocates the value range of RRC identifier for QoE measurements used by SN. When SN configures a QoE measurement to UE, the SN selects one RRC identifier within the range for a QoE measurement triggered by SN.

- Solution 2: SN requests RRC identifier from MN

- In solution 2, if the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure. The MN should inform the SN that a UE is configured with an m-based QoE measurement. If the QoE measurement configuration is allowed by MN, the MN can allocate a RRC ID for the QoE measurement and send it to the SN.

- Since we already agreed that the SN should send request of an m-based QoE measurement configuration to the MN, it is straightforward that the MN allocates a RRC ID for the QoE measurement and sends it to the SN in the Xn-AP S-Node Modification Confirm message.

The MN allocates a RRC ID when receives the request of an m-based QoE measurement configuration from the SN, and send the RRC ID to the SN e.g. in the Xn-AP S-Node Modification Confirm message.

- RVQoE configuration coordination

- In RAN3#119 meeting, it was agreed that:

-	To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC:

-	For the first RVQoE configuration, it is blindly configured by MN or SN.

-	From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session.

-	After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified.

- But it is still not clear whether the node which sends the first RVQoE configuration to UE and the node which sends the legacy QoE configuration to UE should be the same for the first RVQoE configuration. Since the first RVQoE configuration is a blind configuration anywat, it would be better the node which sends the first RVQoE configuration to UE is the same node for the legacy QoE configuration.

For the first RVQoE configuration, the node which sends the first RVQoE configuration to UE is the same node which sends the legacy QoE configuration to UE.

- Upon receiving the first RVQoE report, if the node determines the bearers associated to the corresponding application session is carried by the peer node, it would be better that the peer node takes over the RVQoE configuration and reconfigure it if needed. In this case, the node needs to send the RRC RVQoE configuration e.g., as a RRC container to the peer node for potential delta configuration of the RVQoE configuration.

If the node determines the bearers associated to the corresponding application session are only carried by the peer node, it would be better that the peer node takes over the RVQoE configuration.

- QoE pause/resume in NR-DC

- In Rel-17, QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE in RAN overload situation. However, in NR-DC, since both MN/MCG leg and SN/SCG leg can be used for the QoE measurement reports, how to configure QoE measurement pause/resume function needs to be addressed.

- For example, the MN configures QMC pause/resume taking both MN and SN overload information into account. The UE then pauses or resumes the QoE measurement reporting on both DC legs. The MN can infer the SN overload status for QoE measurement reporting by implementation e.g. according to the flow control info.

- The MN decides to pause to the QoE measurement reporting if both MN and SN are overloaded for QoE measurement reporting. The MN sets pauseReporting to true and send it to UE.

When setting ‘pause or resume’, both MN and SN overload should be taken into account. The UE shall then pause or resume the QoE measurement reporting on both DC legs.

- QoE measurement continuity in mobility scenarios in NR-DC

- SN Change

- It is assumed that both MN and SN can configure QoE measurement independently. For example, the MN configures QoE measurement by SRB1 while SN can configure QoE measurement by SRB3. During SN change procedure, in order to support delta configuration in target SN, the source SN provides source SCG related QoE configuration information to the MN. One of solution is the source SCG related QoE configuration information is provided by scg-CellGroupConfig IE in the CG-Config IE that is an IE of inter-node RRC message as specified in TS 38.331:

:

scg-CellGroupConfig - Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig and/or otherConfig and/or conditionalReconfiguration and/or bap-Config and/or iab-IP-AddressConfigurationList and/or appLayerMeasConfig)

- The MN provides the source SCG related QoE configuration information to the target SN. The MN may also provide the MN related QoE configuration information. The MN includes the information in the SN Addition Request message.  Specifically, the MN includes the source SCG related QoE configuration information in the sourceConfigSCG IE in the CG-ConfigInfo IE that is an IE of inter-node RRC message:

sourceConfigSCG - Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig, and appLayerMeasConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

- The target SN uses the source SCG related QoE configuration information to generate the delta configuration of the QoE configuration.

In order to support delta configuration of a QoE measurement configuration in target SN during SN change procedure, source SN provides the source SCG related QoE configuration information to target SN, forwarded via MN.

- SCG Failure

- The following cases are considered as when SCG failure occurs:

-	A QoE Measurement is configured by MN, but the QoE measurement reporting is configured as SCG that means that the UE sends the QoE measurement report to network by SCG leg by SRB5.

-	A QoE Measurement is configured by SN, and the QoE measurement reporting is configured as MCG or SCG that means that the UE sends the QoE measurement report to network by MCG leg or SCG leg either by a SRB4 or SRB5.

- When SCG failure is detected, it is possible that the QoE measurement report is available at UE RRC layer (e.g. application layer measurement report container has been received from upper layers when SCG failure happens) since QoE measurement reporting has lower priority. When the reporting leg is configured as ‘SCG’, the UE is not able to send the QoE measurement report via SCG leg due to SCG failure. In this case, the following solutions can be considered:

-	Option 1: The UE switches the reporting leg to be MCG automatically upon detecting SCG failure;

-	Option 2: The UE suspends the QoE measurement reporting via SCG leg and buffers the QoE measurement report in RRC until instruction from MN is received. the UE may also send ‘QoE measurement result available’ indication to MN in the SCGFailureInformation message.

-	Option 3: The UE discards the QoE measurement report that reporting leg is configured as ‘SCG leg’.

RAN3 discusses how to handle the available QoE measurement report in SCG leg when SCG failure happens.

3	Conclusion

This contribution further discusses how to support QoE measurement in NR-DC from RAN3 perspective. And we propose:

Basic Signalling for QoE configuration:

- For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration:

- The SN sends the QMC Configuration Information IE can be added in S-NODE MODIFICATION REQUIRED message;

- The MN needs to add the following information in the S-NODE MODIFICATION CONFIRM message:

c1) QoE reference;

c2) when QoE measurement is allowed, the MN needs to notify the SN whether:

-	The MN shall send the configuration information to the UE, or

-	The SN should send the configuration to the UE directly, or

-	The SN should send the configuration information to the UE via the MN (inside a container).

c3) QoE measurement is not allowed e.g., due to the QoE measurement has been configured to UE by the MN;

For S-based and MN initialized m-based QoE measurement configuration, if the MN decides that the UE sends the QoE measurement report to the SN:

- The MN indicates ‘requested SRB5 for QoE measurement reporting’ in Xn-AP S-Node Addition Request message or XnAP S-Node Modification Request message.

- The SN responds ‘admitted SRB5 for QoE measurement reporting’ to the MN in Xn-AP S-Node Addition Acknowledge or S-Node Modification Request Acknowledge message.

For SN initialized m-based QoE measurement configuration, if the SN decides that the UE sends the QoE measurement report to the MN:

- The SN indicates ‘requested SRB4 for QoE measurement reporting’ in Xn-AP S-Node Modification Required message.

- The MN responds ‘available SRB5 for QoE measurement reporting’ to the SN in Xn-AP S-Node Modification Confirm message.

RRC ID allocation

The MN allocates a RRC ID when receives the request of an m-based QoE measurement configuration from the SN, and send the RRC ID to the SN e.g. in the Xn-AP S-Node Modification Confirm message.

RVQoE Configuration coordination:

For the first RVQoE configuration, the node which sends the first RVQoE configuration to UE is the same node which sends the legacy QoE configuration to UE.

If the node determines the bearers associated to the corresponding application session are only carried by the peer node, it would be better that the peer node takes over the RVQoE configuration.

QoE pause/resume in NR-DC:

When setting ‘pause or resume’, both MN and SN overload should be taken into account. The UE shall then pause or resume the QoE measurement reporting on both DC legs.

QoE measurement continuity in mobility scenarios in NR-DC:

In order to support delta configuration of a QoE measurement configuration in target SN during SN change procedure, source SN provides the source SCG related QoE configuration information to target SN, forwarded via MN.

RAN3 discusses how to handle the available QoE measurement report in SCG leg when SCG failure happens.

Reference

[1] RP-221803. WID update for Enhancement on NR QoE. China Unicom.