3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231442

Online, 17th – 26th April, 2023

Agenda Item:	13.3

Source: 	Lenovo

Title: 	Mobility enhancements for mobile IAB-node and its served UE

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, enhancement for mobile IAB-node mobility is discussed, including aspects related to mobile IAB-node indication, information sharing between two logical IAB-DUs and configuration for IAB-DU’s parameters.

- Discussion

Supporting mobile IAB-node indication

Because legacy R16/R17 IAB-donors or parent IAB-nodes may not support the functionalities and characters that will be introduced by the R18 mobile IAB, it’s not a good choice for the R18 mobile IAB-node to always camp on a legacy R16/R17 IAB-donor or parent IAB-node. Therefore, to assist the mobile IAB-node access to a suitable IAB-donor or parent IAB-node, a “mobile-IAB-Support” IE can be introduced in SIB1 just as the “iab-Support” IE in R16. And in RAN2 meeting, RAN2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell. Therefore, IAB-donor-CU needs to configure whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported or not.

Proposal 1: IAB-donor-CU configures whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported.

Fixed IAB-node handover

For the mobile IAB-node, it should have no descendent IAB-nodes and only serve for UEs as defined in the WID. In RAN2 119 e-meeting, RAN2 has agreed that the method of not broadcasting “iab-Support” indication is sufficient to prevent other IAB-node from accessing mobile IAB-node. However, the “iab-Support” indication is only applied for IAB-node integration procedure, it needs to avoid that a source IAB-donor-CU to configure measurement to an IAB-node with a potential cell belonging to a mobile IAB-node, and the source IAB-donor-CU to trigger handover of the IAB-node to the mobile IAB-node cell. Therefore, the source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

Proposal 2: Source IAB-donor-CU of a fixed IAB-node should be aware of whether potential target cell belongs to a mobile IAB-node.

Mobile IAB-node indication to AMF

As defined in SA2 and captured in the latest running TS 23. 501 as follow, mobile IAB-node needs to include a mobile IAB-indication in MSG5 to let IAB-donor-CU select an AMF which supports mobile IAB, and the IAB-donor-CU needs to includes a mobile IAB-indication in the NGAP INITIAL UE MESSAGE to AMF. In addition, as defined in R16 for fixed IAB-node, the similar procedure has been defined in MSG5 and NGAP INITIAL UE MESSAGE. We’d better to design similar procedure for mobile IAB in R18. Otherwise, an LS is needed to inform SA2 to update the procedure defined in TS 23.501.

Proposal 3: Mobile IAB-node includes a mobile IAB-indication in MSG5, and IAB-donor-CU includes a mobile IAB-indication in NGAP INITIAL UE MESSAGE to AMF.

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization as described in clause 5.35A.4.

BH transport in case two logical IAB-DUs

RAN2 121: For the upstream data handling at the BAP of mobile IAB MT, RAN2 assume that the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection (To be confirmed by RAN3). For the downstream data handling arriving at the mobile IAB node, RAN2 assume upper layers (e.g. IP layer) can differentiate the data to different logical DUs based on e.g. the IP address, i.e. no need to introduce logical-DU-specific BAP address. (To be confirmed by RAN3).

Figure 1: An example of IAB-DU migration with its served UE

For DL transmission of mobile IAB-node, upon receiving a DL BAP Data PDU from IAB-donor A or IAB-donor B by mobile IAB-MT, BAP entity of mobile IAB-MT shall remove the BAP header of this BAP Data PDU since the mobile IAB-node has no descendant IAB-nodes and it’s the destination node on the BH link, and the IAB-MT delivers the BAP SDU to upper layer. For the IAB-MT, it needs differentiate the DL traffic from IAB-donor A or IAB-donor B then it can deliver the DL traffic to the right logical IAB-DU. As agreed in RAN2, the DL traffic can be differentiated via the IP address. In addition, source IP address of the DL traffic is different between IAB-donor A and IAB-donor B, and separate IP addresses may be configured for two IAB-DUs for F1 traffic transport, Then the DL traffic differentiation for mobile IAB-node can be based on the IP address, e.g., source IP address or target IP address of the DL traffic.

Proposal 4: DL traffic differentiation for mobile IAB-node is based on the IP address, e.g., source IP address or target IP address of the DL traffic.

For UL transmission, both the source IAB-DU and target IAB-DU can receive the UL F1AP BAP configuration from its terminating IAB-donor-CU respectively. Then the mobile IAB-MT can perform the routing and bearer mapping for UL traffic from each IAB-DU based on the UL F1AP BAP configuration of the corresponding IAB-DU. Alternatively, the UL BAP configuration may be pre-defined to be configured by only one of the two IAB-donors, but it needs extra coordination between two IAB-donors.

Proposal 5: UL F1AP BAP configuration of each logical IAB-DU is configured by its own F1-terminating IAB-donor-CU via the corresponding F1AP connection.

Information sharing between logical IAB-DUs

RAN3 117-e: For mobile DU migration, RAN3 to discuss the benefit and whether to support the sharing info of configuration and/ or UE context between two logical DUs in the mobile IAB-node. RAN3 117bis-e: RAN3 to discuss whether and which information can be shared between two logical DUs in case of IAB-DU migration.

In case of inter-donor mobile IAB-DU migration, the mobile IAB-node will support two logical IAB-DUs which have their F1 connections to source and target IAB-donor-CU, respectively. And the served connected UEs perform handover from the cell of the source logical IAB-DU1 to the cell of the target logical IAB-DU2.

As for the served UEs of mobile IAB-node, since source logical IAB-DU1 and target logical IAB-DU2 are co-located physically and the serving mobile IAB-node doesn’t change for UE during the whole full migration procedure or IAB-DU migration procedure, which means the air interface between the UE and the mobile IAB-node may be maintained during the whole full migration procedure or IAB-DU migration procedure. Therefore, in this case, at least some lower layer configurations (RLC, MAC and PHY) can be reused before and after migration between source logical IAB-DU1 and target logical IAB-DU2.

Proposal 6: Lower layer configuration (RLC, MAC and PHY) can be shared between two logical IAB-DUs in case of full migration and IAB-DU migration.

Configuration for IAB-DU’s parameters

RAN3 117-e: RAN3 to discuss whether a mobile IAB node may be configured with multiple configurations, each corresponding to a different target donor, that can be activated upon fulfillment of certain condition(s). The details of the configurations are FFS. RAN3 117bis-e: RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed. RAN3 119: With respect to mIAB-DU migration and partial migration, RAN3 to discuss how the mobile IAB-DU’s parameters are (re-)configured.

Pre-configuration at the mobile IAB-node may have some advantages to reduce the latency during IAB-DU migration or full migration, especially for the case of regular trajectory or predicable trajectory of mobile IAB-node. In addition, only some relative-static paraments could be considered to be pre-configured at the mobile IAB-node, otherwise, it’s meaningless and the pre-configured parameters may be already outdated by the time they are used which led to extra overhead. As an example, at least the TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-stored at the mobile IAB-node to accelerate the F1 setup between second logical IAB-DU and target IAB-donor-CU.

Proposal 7: The TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-configured at the mobile IAB-node in case of determined trajectory.

However, for the mobile IAB-node, the target IAB-donor-CU may be unpredictable due to the random mobility of mobile IAB-node, and in this case, pre-configuration at the IAB-DU may not feasible.

For NCGI of IAB-DU, as legacy, it’s configured by OAM and is changed when the F1-terminating IAB-donor-CU of the IAB-DU is changed. OAM based reconfiguration may cause extra latency. To accommodate frequent NCGI change of IAB-DU due to IAB-node mobility, IAB-donor-CU based reconfiguration for NCGI is more flexible.

Proposal 8: F1-terminating IAB-donor-CU can reconfigure the NCGI of IAB-DU.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: IAB-donor-CU configures whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported.

Proposal 2: Source IAB-donor-CU of a fixed IAB-node should be aware of whether potential target cell belongs to a mobile IAB-node.

Proposal 3: Mobile IAB-node includes a mobile IAB-indication in MSG5, and IAB-donor-CU includes a mobile IAB-indication in NGAP INITIAL UE MESSAGE to AMF.

Proposal 4: DL traffic differentiation for mobile IAB-node is based on the IP address, e.g., source IP address or target IP address of the DL traffic.

Proposal 5: UL F1AP BAP configuration of each logical IAB-DU is configured by its own F1-terminating IAB-donor-CU via the corresponding F1AP connection.

Proposal 6: Lower layer configuration (RLC, MAC and PHY) can be shared between two logical IAB-DUs in case of full migration and IAB-DU migration.

Proposal 7: The TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-configured at the mobile IAB-node in case of determined trajectory.

Proposal 8: F1-terminating IAB-donor-CU can reconfigure the NCGI of IAB-DU.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 117-119 meetings.