3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231452

17th – 26th April 2023

Agenda item:	 8.1

Source:		Lenovo

Title:		Discussion on RAN information exposure for XRM

Document for:		Discussion and Decision

- Introduction

This paper discusses the issues questioned by SA2 in the incoming LS [1].

- Discussion

- SA2 has concluded for RAN information exposure for XRM:

-	For the congestion information, based on the PCC rule from PCF, the SMF requests the NG-RAN to report via GTP-U header to UPF and to PSA UPF to expose the information

- 	For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4 of TS 23.501, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF

- 	Based on SMF request over NGAP, data rate information on a per QoS Flow basis may be measured and exposed via NGAP to SMF or via GTP-U to UPF.

In the incoming LS, SA2 ask RAN3’s view on the last two information exposed from RAN.

- QoS Notification Control

- Currently, both notification control without alternative QoS profiles and with alternative QoS profiles are supported by control plane signalling as specified in section 5.7.2.4 in TS 23.501 [2].

- If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled, NG-RAN shall send a notification towards SMF that the "GFBR can no longer be guaranteed". Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not fulfilling the requested QoS profile for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to fulfil the GFBR, the PDB and the PER of the QoS profile again. - When the NG-RAN determines that the GFBR, the PDB and the PER of the QoS profile can be fulfilled again for a QoS Flow (for which a notification that the "GFBR can no longer be guaranteed" has been sent), the NG-RAN shall send a notification, informing the SMF that the "GFBR can be guaranteed" again and the SMF may forward the notification to the PCF, see TS 23.503 [45].

- If the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled, NG-RAN shall send a notification towards SMF that the "GFBR can no longer be guaranteed". Before sending a notification that the "GFBR can no longer be guaranteed" towards the SMF, the NG-RAN shall check whether the GFBR, the PDB and the PER that the NG-RAN currently fulfils match any of the Alternative QoS Profile(s) in the indicated priority order. If there is a match, the NG-RAN shall indicate the reference to the matching Alternative QoS Profile with the highest priority together with the notification to the SMF. - If there is no match, the NG-RAN shall send a notification that the "GFBR can no longer be guaranteed" towards the SMF indicating that the lowest Alternative QoS Profile cannot be fulfilled (unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion).

- Observation 1	Currently, the NG-RAN node has been able to determine ‘GFBR can be no longer guaranteed’ and ‘GFBR can be guaranteed again’ with/without alternative QoS profiles.

The current QoS Notification Control (QNC) mechanism is provided by control plane signalling. SA2 is discussing whether it is feasible QNC can be achieved by user plane protocol.

As specified in TS 38.415, NG-RAN node will send UL PDU SESSION INFORMATION in the "PDU Session Container" GTP-U Extension Header:

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

- As shown in the tabular, NG-RAN node has already been able to provide some QoS monitoring information such as DL delay result to the UPF directly.

- Observation 2	Currently, the NG-RAN node has been able to provide some QoS monitoring information such as DL delay result to the UPF directly in the "PDU Session Container" GTP-U Extension Header.

- According to the observation 1 and observation 2, we believe that there is no any technical issue that NG-RAN provides the QoS Notification Control information in the "PDU Session Container" GTP-U Extension Header to UPF.

Respond SA2 that RAN3 does not see any technical issue that the NG-RAN indicates that "GFBR can no longer (or can again) be guaranteed" via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF.

- Data Rate Information

- As described in solution 6 of TR 23.700-060 [3], the data rate information would be available mean bit rate for the non-GBR flow. The available mean bit rate is the bitrate with a time windows, e.g. averaging windows, i.e. the (default) averaging windows (e.g. 2000ms in the Table 5.7.4-1 of TS 23.501) is also applicable to the Non-GBR 5QI. The available mean bitrate is the bitrate that can be provided by the non-GBR QoS Flow based on the available scheduling radio resource to the non-GBR QoS Flow. The available mean bitrate can be treated as the near-future bitrate and is not the measured value of the current bitrate in the radio interface for the non-GBR QoS Flow.

From RAN3’s point of view, it is feasible that NG-RAN node provides available bit rate to CN. Some companies may do not to explore the detailed data rate information which relies on gNB’s implementation. But as the QNC, there are two ways to report the available bit rate for a QoS flow: First one is that the SMF can provide different bitrate span to the RAN, e.g. the three level bitrate span are 0~500kbps, 500~2500kbps, above 2500kpb ~ Ͼ. Only after the available mean bitrate is changed from one bitrate level to another bitrate level, the RAN can generate the non-GBR QoS Notification Control Report Message. The second on is that the SMF can provide different bitrate changes percentage to RAN, only after the available mean bitrate is changed UP/Down above the change percentage, the RAN will generate the non-GBR QoS Notification Control Report message. One example of 3 level bitrate change percentage can be 50%, 100%, 200%.

Respond SA2 that RAN3 thinks it is feasible to report the available bit rate for a QoS flow and prefers a similar mechanism as QNC for reporting the available bit rate for a QoS flow.

3	Conclusion

This paper discusses the issues questioned by SA2 in the incoming LS. And we propose:

- Observation 1	Currently, the NG-RAN node has been able to determine ‘GFBR can be no longer guaranteed’ and ‘GFBR can be guaranteed again’ with/without alternative QoS profiles.

- Observation 2	Currently, the NG-RAN node has been able to provide some QoS monitoring information such as DL delay result to the UPF directly in the "PDU Session Container" GTP-U Extension Header.

- Respond SA2 that RAN3 does not see any technical issue that the NG-RAN indicates that "GFBR can no longer (or can again) be guaranteed" via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF.

- Respond SA2 that RAN3 thinks it is feasible to report the available bit rate for a QoS flow and prefers a similar mechanism as QNC for reporting the available bit rate for a QoS flow.

A daft response to the LS is provided in the Annex

Reference

[1] R3-230791. LS on RAN Information exposure for XRM. (Source SA2)

[2] TS 23.501

[3] TR 23.700 - 060.

Annex

Text to respond SA2:

RAN3 thanks SA2 for the LS on RAN Information exposure for XRM.

For QoS Notification Control for GBR QoS Flow, RAN3 does not see any technical issue that the NG-RAN indicates that "GFBR can no longer (or can again) be guaranteed" via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF.

-For data rate information on a per QoS Flow basis, RAN3 thinks it is feasible to report the available bit rate for a QoS flow and prefers a similar mechanism as QNC for reporting the available bit rate for a QoS flow.

Actions to SA2

ACTION: 	SA3 kindly ask SA2 to take the above response into account.