3GPP TSG-RAN WG3 #119bis-e 	  								R3-231470

17th – 26th April 2023

Online

Agenda item:	13.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Support IAB-node mobility

Document for:	Discussion and Decision

# Introduction

This contribution discusses the issues on UE position with mobile IAB as required by SA2, consecutive partial migration and full migration.

# UE position with Mobile IAB

As agreed by the SA2, the CN may initiate UE position procedure for a UE connected with the mobile IAB. The mobile IAB’s location may be dynamic. So it is necessary for LMF to know the latest location information of the TRP. The location of the mobile IAB can be considered as the location of the TRP. The location of the mobile IAB can be determined by using current MO/MT position procedure.

In normal system, the TRP’s location is configured in the gNB. The LMF can initiate the TRP information Exchange procedure to retrieve the TRP information. The initial TRP Information Exchange procedure is usually triggered by OAM configuration, e.g. when OAM configure a new TRP ID in the LMF, then the LMF initiate the TRP Information Exchange procedure to get more information about the TRP. The information of the TRP can include the NR PCI, NG-RAN CGI, NR ARFCN, PRS config, SSB config, SFN init time, spatial direction info, geocoordinates, TRP type, on demand PRS, TRP TX TEG, beam antenna info, etc. Similar TRP information exchange procedure is also supported in F1 interface.

The SA2 requirement can be supported by enhance the F1/NRPPa procedure with following changes:

- A new codepoint in TRP Type IE to indicate this is a mobile TRP. By knowing it is a mobile TRP, LMF can request the updated location of the TRP, whenever LMF decide to use this TRP to position a UE.

- An ID of the mobile IAB-MT. SA2 has agreed to use GPSI, which is a public identifier already used both inside and outside of the 3GPP system.

- Other enhancements may be considered later. For example, whether velocity information need be considered, but this may need further study, e.g. how does the IAB determine its velocity.

Proposal 1: enhance F1/NRPPa procedure to provide the mobile TRP indication, and IAB-MT’s ID to the LMF.

# Consecutive partial migration

Last RAN3 meeting agreed following

The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the:

•	gNB ID of the target donor CU for the mIAB-MT HO.

•	ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed.

The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID.

Let’s assume the partial migration is performed from IAB-donor1 to IAB-donor2, then to IAB-donor3. The open issue is How the mIAB-MT ID is maintained across migrations.

- Step 1: IAB is partial migration from IAB-donor1 to IAB-donor2. (source UE XnAP ID: 100, target UE XnAP ID: 200)

Both IAB-donor1-CU and IAB-donor2-CU maintains following UE XnAP ID for the IAB-MT:

- UE XnAP ID (i.e. 100) assigned by IAB-donor1-CU

- UE XnAP ID (i.e. 200) assigned by IAB-donor2-CU

(Starts further partial migration to IAB-donor3)

- Step 2: IAB-donor2 send Xn HANDOVER REQUEST (source UE XnAP ID: 201) to IAB-donor3, i.e. start the further partial migration to IAB-donor3.

The Xn HANDOVER REQUEST message is enhanced to include the UE XnAP ID (i.e. 100) assigned by IAB-donor1-CU, and the ID of IAB-donor1.

Note: UE XnAP ID is unique within an IAB-donor. IAB-donor2-CU can use the same UE XnAP ID (i.e. 200) or a different UE XnAP ID over the Xn interface with IAB-donor3-CU. In this example, a different UE XnAP ID (i.e. 201) is used.

- Step 3: IAB-donor2 receive the Xn HANDOVER REQUEST Ack (target UE XnAP ID: 300) from IAB-donor3.

Up to now, both IAB-donor2-CU and IAB-donor3-CU maintains following UE XnAP ID for the IAB-MT:

- UE XnAP ID (i.e. 100) assigned by IAB-donor1-CU, and ID of IAB-donor1

- UE XnAP ID (i.e. 201) assigned by IAB-donor2-CU for Xn interface between IAB-donor2 and IAB-donor3

- UE XnAP ID (i.e. 300) assigned by IAB-donor3-CU for Xn interface between IAB-donor2 and IAB-donor3

- Step 4: IAB-donor2 informs IAB-donor1 for the HO of IAB-MT to IAB-donor3.

IAB-donor1-CU update its context for IAB-MT:

- UE XnAP ID (i.e. 100) assigned by IAB-donor1-CU

- UE XnAP ID (i.e. 200) assigned by IAB-donor2-CU

- UE XnAP ID (i.e. 300) assigned by IAB-donor3-CU for Xn between IAB-donor2 and IAB-donor3

- ID of IAB-donor3

- Step 5: IAB-donor2 send HOCommand to IAB-MT

- Step 6: IAB-MT detach from IAB-donor2 and connect with IAB-donor3.

IAB-donor2 can release the context for the IAB when no further data is routed via IAB-donor2’s topology.

- Step 7: IAB-DU inform IAB-donor1 for the new TNL address. F1-C is migrated to IAB-donor3’s topology.

- Step 8: IAB-donor1 initiate the Xn IAB Transport Migration Management Request procedure to move the offloaded traffic to IAB-donor3. The REQUEST message includes the UE XnAP ID (i.e. 100) assigned by IAB-donor1-CU and UE XnAP ID (i.e. 300) assigned by IAB-donor3-CU.

- Step 9: IAB-donor3 reply with the L2 transport info for the offloaded traffic.

- Step 10: the F1-U is migrated to IAB-donor3’s topology.

The consecutive partial migration can be supported with little enhancements to XnAP. The possible enhancement is Step 2 and Step 4.

Proposal 2-1: during the Xn handover preparation procedure, the source donor CU (e.g. IAB-donor2-CU) for the mIAB-MT HO provides to the target donor CU (e.g. IAB-donor3-CU) the following information:

- UE XnAP ID assign by the F1-terminating IAB-donor-CU for the IAB-DU,

- ID of the F1-terminating IAB-donor

Proposal 2-2: after the handover is completed, the source donor CU (e.g. IAB-donro2-CU) for the mIAB-MT HO provides to the donor CU serving the mIAB-DU (i.e. F1-terminating IAB-donor-CU) the following information:

- UE XnAP ID assign by the target donor CU,

- ID of the target donor CU.

# IAB-DU migration

Last RAN3 meeting agreed

In case the donor of the mIAB-DU decides the F1AP setup for DU migration, the donor of the mIAB-DU triggers via F1 signalling the IAB node to perform the F1 Setup procedure for the DU migration. An OAM based solution is not excluded.

For IAB-DU migration, it may be mostly up to the operator’s configuration. For example, operator may want the IAB-DU migration to be performed when the IAB moves at a specific location. The source CU of the IAB-DU may not be able to make decision for IAB-DU migration without operator’s input/configuration. In one example, operator may use dedicated gNB-CU for terminating IAB-DU’s F1. For example, operator may configure a few gNB-CUs to serve all UEs connected with mobile IABs. This may be obvious in a cloud implementation that gNB-CUs are located in a central place to server all UEs in a city, and some gNB-CUs are designated to server all UEs connected with mobile IAB(s).

The operator should configure the donor of the mIAB-DU on when/whether a full migration needs to be performed. Based on the configured information, the donor of the mIAB-DU can request the IAB-DUb to setup F1 with target IAB-donor. IAB-DUb also need to know the information of target IAB-donor (e.g. gNB ID of target IAB-donor). This serves following purpose:

- Before IAB-DUb initiate F1 setup with target IAB-donor, IAB-DUb need to be OAM configured with appropriate DU parameters, e.g. NCGI associated with the gNB ID of target IAB-donor, IP address of target IAB-donor-CU, SeGW information, etc. To assist the OAM server to make the decision, IAB-DUb should provide the information of target IAB-donor (e.g. NCGI of target IAB-donor, gNB ID) to OAM server, thus the OAM server can configure the appropriate DU parameters.

The IAB-DUb configuration is similar to Rel-16/17 IAB-DU configuration. In Rel-16/17 IAB integration procedure, the IAB need to be OAM configured with the DU parameters (e.g. NCGI) related to a specific IAB-donor that the IAB-DU will setup F1 with. In case an IAB is deployed in an area overlapped by cells from more than one IAB-donor (e.g. overlapped by IAB-donor1’s cell and IAB-donor2’s cell), the IAB need to be configured with the appropriate DU parameters (e.g. NCGI). For example, in case IAB need to setup F1 with IAB-donor1-CU, OAM should configure a NCGI corresponding to the gNB ID of IAB-donor1, rather a NCGI corresponding to the gNB ID of IAB-donor2. To assist the OAM server to make the right decision (e.g. to determine the right IAB-donor that IAB-DU connect with), there are various ways. For example, after IAB-MT is registered and setup OAM connection, IAB may report its serving/parent cell information (e.g. NCGI and TAC of the parent cell) to the OAM server. Thus OAM server can know the target IAB-donor for the co-located IAB-DU, and then configure IAB-DU the DU parameters related to the target IAB-donor.

The similar method can be reused for Rel-18 IAB-DUb configuration. IAB-donor1-CU know the target IAB-donor for IAB-DUb, so it informs IAB-Dub about the information of target IAB-donor. IAB-DUb report the information of target IAB-donor to OAM server, in order for the OAM server configure IAB-DUb the DU parameters related to target IAB-donor.

An example call flow is shown below: (for simplicity reason, IAB-donor-DU and UE are omitted).

Figure 1: Example for target F1 setup before HO connected UE(s)

Before the IAB-DU migration, IAB-DUa connects with IAB-donor1-CU.

- IAB-donor1-CU decides to perform DU migration to IAB-donor2. IAB-donor1-CU may be configured with the information on when/whether to perform IAB-DU migration.

- IAB-donor1-CU send a REQUEST to IAB node to setup F1 with IAB-donor2-CU. The REQUEST includes the information of target donor, e.g. gNB ID of IAB-donor2.

- IAB send a request to OAM server, requesting the DU parameters to be used for F1 with IAB-donor2-CU.

- OAM server configures IAB for the related DU parameters, e.g. NCGI(s) to be used by IAB-DUb’s cell(s), IP address of target IAB-donor CU, SeGW information, etc. This is similar to current OAM configuration to a gNB-DU before the gNB-DU initiate F1 with gNB-CU.

NOTE: An IAB-DU may have more than 1 cell.

- Based on the OAM configured DU parameters, IAB-DUb setup SCTP/F1 with IAB-donro2-CU. If needed, IAB-DUb may need to first setup secure IP connection with SeGW.

- IAB-DUa informs IAB-donor1-CU that target F1 is setup with IAB-donor2-CU. IAB also provides the cell ID of IAB-DUb.

- IAB-donor1-CU initiates the handover procedure for UE(s) connected with IAB-DUa to IAB-DUb.

In case an IAB-DU has more than 1 cell, IAB-donor1-CU need to know the cell ID mapping from a specific IAB-DUa’s cell to IAB-DUb’s cell. It is 1:1 cell mapping between IAB-DUa’s cell and IAB-DUb’s cell.

Figure 2: Example for IAB-DU with more than 1 cell

Observation 3-1: IAB-DUb need to know the information of target IAB-donor before it initiates F1 setup with target IAB-donor.

Observation 3-2: before IAB-DUb setup F1 with target IAB-donor, IAB-DUb need to be OAM configured the DU parameters and SeGW IP address, related to target IAB-donor.

Last RAN3 meeting agreed

When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID).

The target CU’s information is used by the IAB-DUb to setup F1 with target CU. Before the F1 setup, a secure connection needs to be established with target SeGW, e.g. in case source CU and target CU uses different SeGWs. The SeGW may be an external standalone SeGW, so the source CU may not know the SeGW’s IP address. This information is usually known in the OAM, e.g. OAM need to configure IAB with the SeGW IP address during initial integration,. Since source CU know the gNB ID of target CU, we prefer the source CU provide the gNB ID of target CU to the IAB, then IAB retrieve other information (e.g. IP address of SeGW, IP address of target CU) from OAM by providing the gNB ID of target CU.

Proposal 3-1: When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU provide the gNB ID of target logical mIAB-DU’s CU to IAB.

Proposal 3-2: Similar as normal gNB-DU, OAM server configures IAB-DUb the DU parameters and SeGW IP address related to target IAB-donor. The configured parameters include the NCGI(s) to be used by IAB-DUb’s cell(s), IP address of SeGW, IP address of target IAB-donor-CU, etc.

Proposal 3-3: after target F1 is setup, IAB informs CU of IAB-DUa (i.e. IAB-donor1-CU) about the cell ID of IAB-DUb.

- In case IAB-DU has more than 1 cell, IAB-donor1-CU need to know the cell ID of IAB-DUb corresponding to a specific IAB-DUa’s cell.

# IAB migration via NG-HO

Last RAN3 meeting agreed “RAN3 to discuss support of mIAB-node mobility over NGAP.” When the IAB node migrate to target IAB-donor, Rel-17 defines Xn-HO procedure is used to handover the IAB-MT to target IAB-donor. In case target IAB-donor connects to a different AMF, i.e. the target IAB-donor belongs to a different AMF area, NG-HO should be used to handover the IAB-MT to target IAB-donor. We think it is a valid scenario that the target IAB-donor belongs to a different AMF area, while Xn may still exist between IAB-donors from different AMF area.

Observation 4-1: NG-HO can be used to handover the IAB-MT to target IAB-donor.

The existing NG-HO procedure can be reused, with small enhancements. One aspect is the IAB-MT may not have a PDU session. However, the PDU Session Resource List IE is a Mandatory IE in the NGAP HANDOVER REQUIRED message, and the PDU Session Resource Setup List IE is a Mandatory IE in the NGAP HANDOVER REQUEST message. Similar issue was also discussed in the Xn-HO, and RAN3 agreed to add the No PDU Session Indication IE in the XnAP HANDOVER REQUEST message. Similar mechanism may be introduced for NG-HO.

There are following issues to be addressed:

- How does AMF handle the NGAP HANDOVER REQUIRED message including the PDU Session Resource List IE, but the IAB-MT actually does not have a PDU session?

Current TS38.413 defines

Upon reception of the HANDOVER REQUIRED message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Handover Required Transfer IE to the SMF associated with the concerned PDU session.

In case the IAB-MT does not have PDU session, this paragraph needs to be modified. The AMF can know the UE is an IAB-MT, and AMF also knows that there is no PDU session for the IAB-MT, so the AMF can handle the received PDU session information differently. For the Stage-3 implementation, there are two options:

- Option 1: not to introduce new IE, but just add text about the behaviour, e.g. “if the UE is an IAB-MT and does not have a PDU session, the AMF shall ignore the PDU session information.”

- Option 2: introduce No PDU Session Indication IE in the HANDOVER REQUIRED message and add following text about the behavior: “if the No PDU Session Indication IE is included in the HANDOVER REQUIRED message, the AMF shall ignore the PDU session information. ”

Option 1 may be enough.

- How does the target IAB-donor handle the NGAP HANDOVER REQUEST message including the PDU Session Resource Setup List IE, but the IAB-MT actually does not have a PDU session?

Current TS38.413 defines:

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure.

In case the IAB-MT does not have PDU session, this paragraph needs to be modified. The HANDOVER REQUEST message needs to have a new indication, e.g. No PDU Session Indication IE, to inform target IAB-donor that the IAB-MT has no PDU session.

This also affects the SA2 spec (TS23.502). For example, Nsmf operation (Step 4, 7, 11a, 11f in below figure) may not be needed when the IAB-MT does not have a PDU session and should be changed to conditional for IAB-MT. RAN3 should send a LS to SA2.

Proposal 4-1: Introduce the No PDU Session Indication IE in the HANDOVER REQUEST message

A draft TS38.413 CR can be found at ([2])

Proposal 4-2: RAN3 send LS to SA2 asking for the support for NG-HO without PDU session for the IAB-MT.

# Conclusions

For IAB-DU migration, our proposals are:

Proposal 1: enhance F1/NRPPa procedure to provide the mobile TRP indication, and IAB-MT’s ID to the LMF.

Proposal 2-1: during the Xn handover preparation procedure, the source donor CU (e.g. IAB-donor2-CU) for the mIAB-MT HO provides to the target donor CU (e.g. IAB-donor3-CU) the following information:

- UE XnAP ID assign by the F1-terminating IAB-donor-CU for the IAB-DU,

- ID of the F1-terminating IAB-donor

Proposal 2-2: after the handover is completed, the source donor CU (e.g. IAB-donro2-CU) for the mIAB-MT HO provides to the donor CU serving the mIAB-DU (i.e. F1-terminating IAB-donor-CU) the following information:

- UE XnAP ID assign by the target donor CU,

- ID of the target donor CU.

Proposal 3-1: When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU provide the gNB ID of target logical mIAB-DU’s CU to IAB.

Proposal 3-2: Similar as normal gNB-DU, OAM server configures IAB-DUb the DU parameters and SeGW IP address related to target IAB-donor. The configured parameters include the NCGI(s) to be used by IAB-DUb’s cell(s), IP address of SeGW, IP address of target IAB-donor-CU, etc.

Proposal 3-3: after target F1 is setup, IAB informs CU of IAB-DUa (i.e. IAB-donor1-CU) about the cell ID of IAB-DUb.

- In case IAB-DU has more than 1 cell, IAB-donor1-CU need to know the cell ID of IAB-DUb corresponding to a specific IAB-DUa’s cell.

Proposal 4-1: Introduce the No PDU Session Indication IE in the HANDOVER REQUEST message

A draft TS38.413 CR can be found at ([2])

Proposal 4-2: RAN3 send LS to SA2 asking for the support for NG-HO without PDU session for the IAB-MT.

# References

- RP-221815 New WID on Mobile IAB

- R3-231471, CR for TS38.413