3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231502

# e-meeting, 17th - 26th Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0975 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on multicast session distribution setup procedure in TS 38.413 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.247 "7.2.4.2 Support of location dependent multicast service", for location dependent multicast service the mapping of location information (e.g., cell ID or TAI ID list) and MBS Area Session ID is indicated to NG-RAN node in the DISTRIBUTION SETUP RESPONSE message. Upon UE's intra-gNB mobility, NG-RAN node uses above mapping information to determine whether the user plane for the local Multicast MBS session for the new cell UE moves to is established or not. If not, NG-RAN node shall establish the corresponding user plane according to the mapped MBS Session ID IE and the MBS Area Session ID IE based on the mapping relationship provided in previous DISTRIBUTION SETUP RESPONSE message. In current procedural description that part is missing and may cause confusion for the understanding of stage 3 IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the functional description of the MBS Service Area IE in the Distribution Setup procedure as below: "If MBS Service Area IE is included in the DISTRIBUTION SETUP RESPONSE message, NG-RAN node shall store and/or update the mapping of location information (e.g., NR CGI IE list or TAI IE list) and MBS Area Session ID IE for the corresponding multicast session, and use it to determine whether the user plane for the local Multicast MBS session is established or not upon UE's intra-gNB mobility as specified in TS 23.247 [44]. If not, NG-RAN node shall establish the corresponding user plane according to the MBS Session ID IE and the MBS Area Session ID IE in another DISTRIBUTION SETUP REQUEST message." Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause any non-backward compatible issues. This CR only have impacts on the MBS multicast context setup function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing essential function description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.18	Multicast Session Management Procedures

### 8.18.1	Distribution Setup

#### 8.18.1.1	General

The purpose of the Distribution Setup procedure is to assign NG-U resources for a multicast MBS session. The procedure uses non-UE-associated signalling.

#### 8.18.1.2	Successful Operation

Figure 8.18.1.2-1: Distribution Setup, successful operation.

The NG-RAN node initiates the procedure by sending a DISTRIBUTION SETUP REQUEST message to the AMF. The AMF responds with a DISTRIBUTION SETUP RESPONSE message.

For location dependent multicast sessions, the NG-RAN node shall include the MBS Area Session ID IE in the DISTRIBUTION SETUP REQUEST message, and the AMF shall provide the same value of the MBS Area Session ID IE in the DISTRIBUTION SETUP RESPONSE message. If MBS Service Area IE is included in the DISTRIBUTION SETUP RESPONSE message, NG-RAN node shall store and/or update the mapping of location information (e.g., NR CGI IE list or TAI IE list) and MBS Area Session ID IE for the corresponding multicast session, and use it to determine whether the user plane for the Multicast MBS session is established or not upon UE's intra-gNB mobility as specified in TS 23.247 [44]. If not, NG-RAN node shall establish the corresponding user plane according to the MBS Session ID IE and the MBS Area Session ID IE in another DISTRIBUTION SETUP REQUEST message.

If the Shared NG-U Unicast TNL Information IE is included in the MBS Distribution Setup Request Transfer IE in the DISTRIBUTION SETUP REQUEST message, the MB-SMF shall use the included information as the downlink termination point for the shared NG-U transport.

If the Shared NG-U Unicast TNL Information IE is not included in the MBS Distribution Setup Request Transfer IE in the DISTRIBUTION SETUP REQUEST message, the MB-SMF shall interpret that the IP multicast is used for this shared NG-U transport, and include the Shared NG-U Multicast TNL Information IE in the MBS Distribution Setup Response Transfer IE in the DISTRIBUTION SETUP RESPONSE message.