3GPP TSG-RAN3 Meeting #119bis-e	R3-231520

# Online, 17 April - 26 April 2023

Agenda item:	11.3

Source: 	Xiaomi

Title: 	Discussion on QoE configuration and reporting in NR-DC

Document for:	Discussion & decision

Introduction

In this contribution, we’d like to discuss the QoE measurement collection in NR-DC scenario based on the agreement and FFS in last RAN3 meeting.

RAN3#119:

If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.

The MN should inform the SN that a UE is configured with an m-based QoE measurement.

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

The MN shall send the configuration information to the UE, or

The SN should send the configuration to the UE directly, or

The SN should send the configuration information to the UE via the MN (inside a container).

In NR-DC, a node can configure the UE with an m-based QoE configuration only if it has received this configuration from the OAM, and if it serves the UE by a cell within the area scope.

The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

FFS on whether a pool of RRC ID is split between MN and SN or whether it is per measurement.

To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC:

- For the first RVQoE configuration, it is blindly configured by MN or SN.

- From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session.

- After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified.

If a node receives an RVQoE report from a UE in NR-DC, and determines that the bearers for the application session are also or only provided by the peer node, this node can send the received RVQoE report to the peer node.

Turn WA to agreement: SN can send an RVQoE configuration to the UE.

Discussion

In last RAN3 meeting, RAN3 had a good progress for the case that the SN is interested in configuring a UE with an m-based QoE. Based on the related agreement, the possible signalling can be shown as in Figure 1

Figure 1 stage 2 procedure

Step 1, MN inform SN that a UE is configured with an m-based QoE measurement. This is based on the agreement (The MN should inform the SN that a UE is configured with an m-based QoE measurement.). however, we didn’t discuss when to send this information and the purpose of this information, we think this information should be transferred before SN send the interest indication to MN, it can be used for SN to decide whether to send the interest indication.

Step 2, if SN is interested, the SN send the interest indication to MN. In addition, we think the SN should also notify whether it support the SRBs for sending QoE configuration and reporting, then MN can take that into account to decide how to send the QoE configuration and how to configure the reporting leg.

Step 3, if MN decides to configure the QoE indicate by SN, MN should response SN, including the assigned RRC ID for the requested QoE configuration from SN, information about how to send the configuration and the SRB for reporting.

Step 4a, if MN decide MN should send the QoE configuration, SN can include the QoE configuration in the RRC Transfer message, and then MN send it via RRC Reconfiguration message to UE.

Step 4b, if MN decide SN should send the QoE configuration, SN can send the QoE configuration directly to UE via SRB3.

With above understanding, we have the following proposals:

Proposal 1, MN should inform the SN that a UE is configured with m-based QoE measurement once it’s configured in the UE via SN Addition Request message, SN Modification Request message.

Proposal 2, SN can consider the already configured m-based QoE configuration in MN to decide whether to send the interest to MN.

Proposal 3, SN can indicate its interest in configuring m-based QoE via SN Modification Required message, MN can response whether it agreed to send the QoE configuration indicated by SN via SN Modification Confirm message.

Proposal 4, SN should also notify MN whether the SRBs for sending QoE configuration via SN and receiving QoE report via SN are available or not.

Proposal 5, MN is responsible to assign RRC ID per QoE configuration request from SN and response SN the assigned RRC ID.

Proposal 6, MN should also notify SN which node to send the QoE configuration and the reporting configuration (i.e. which leg for reporting) as well.

Proposal 7, if MN should send the QoE configuration, SN can include the QoE configuration including the reporting configuration from SN in RRC Transfer message.

Conclusion

The following are the proposals.

Proposal 1, MN should inform the SN that a UE is configured with m-based QoE measurement once it’s configured in the UE via SN Addition Request message, SN Modification Request message.

Proposal 2, SN can consider the already configured m-based QoE configuration in MN to decide whether to send the interest to MN.

Proposal 3, SN can indicate its interest in configuring m-based QoE via SN Modification Required message, MN can response whether it agreed to send the QoE configuration indicated by SN via SN Modification Confirm message.

Proposal 4, SN should also notify MN whether the SRBs for sending QoE configuration via SN and receiving QoE report via SN are available or not.

Proposal 5, MN is responsible to assign RRC ID per QoE configuration request from SN and response SN the assigned RRC ID.

Proposal 6, MN should also notify SN which node to send the QoE configuration and the reporting configuration (i.e. which leg for reporting) as well.

Proposal 7, if MN should send the QoE configuration, SN can include the QoE configuration including the reporting configuration from SN in RRC Transfer message.

Reference