3GPP TSG-RAN WG3 #119bis-e                                               R3-231556

Online, 17th – 26th April 2023

Agenda Item:	10.2.4

Source:	CATT

Title:	Discussion on SONMDT enhancement for NPN

Document for:	Discussion

Introduction

In last RAN3 meeting, we made general discuss about NPN, but did not achieve some consensus.

In the document, we provide some analysis on SONMDT enhancements for NPN according to the new split topic.

Discussion

  - MRO for NPN

In RAN3 meeting, the open issues on SON for NPN are listed as below:

Which NPN related information should be included in RLF report can be further discussed:

1) The accessing NPN

2) Include UE NPN capability

Here we focus on MRO for NPN. Due to UE NPN capability and network NPN configuration, UE may select different RRC reestablishment cell after handover failure. When performing MRO analysis, network shall be informed about UE NPN related information.

Let’s consider the following cases first as in the figure 1:

Cell0, cell1, cell2 and cell3 support public network. At the same time, NPN is deployed on the basis of PLMN network, i.e. cell0, cell1 and cell3 also support NPN, while cell 2 only support PN.

Figure 1

Case 1: UE0 with NPN and PN capability, i.e. UE0 is allowed to access cell0, cell1, cell2 and cell3.

Case 2: UE1 with only NPN capability, i.e. UE1 is allowed to access cell0, cell1, and cell3.

When UE moves from cell0 to cell1, if handover failure occurs, a RLF Report is generated.

For UE0, after handover failure, UE may initiate RRC reestablishment to cell 2 because the signal quality of cell 2 is better than the others.

For UE1, after handover failure, UE may initiate RRC reestablishment to cell 3 for the reason that cell 2 is not allowed to access due to UE NPN capability and only the cell 3 with second best signal quality can be selected.

When analyzing RLF Report for UE0 and UE1, without UE context, network cannot identify the difference between UE0 and UE1 according to current specification. Network shall analyze that cell 3 is the suitable handover target cell for NPN UE, while cell 2 is the suitable handover target cell for PLMN UE based on the UE RLF report.

The main difference for UE0 and UE1 is UE NPN capability which leads to the different RRC reestablishment cell. Therefore, we propose to include UE NPN capability in RLF Report.

Observation 1: Different UE NPN capability may result in different RRC reestablishment cell. UE NPN capability is needed for network optimization.

In previous RAN3 meeting, some companies believe 1-bit indicator (whether it is only NPN capable UE) is sufficient. We think more information is needed. Let’s consider the similar scenarios as in figure 2:

Figure 2

Cell0 and cell1 supports CAG0/1;

Cell2 supports CAG0;

Cell3 supports CAG1;

UE0 and UE1 are both PNI-NPN capable UE, UE0 supports CAG0 while UE1 supports CAG1. After handover from cell 0 to cell 1 failure, different reestablishment cell is selected.

UE0 selects cell2 as RRC reestablishment cell while UE1 selects cell3.

The main difference for UE0 and UE1 is UE NPN capability, i.e. CAG0 for UE0 and CAG1 for UE1. So, we propose to include complete UE NPN capability. The same example can be applied in SNPN.

As for the detail information, we propose to include NPN Mobility Information IE in Mobility Restriction List IE in RLF report.

Proposal 1: It is proposed to include complete UE NPN capability i.e., NPN Mobility Information IE in Mobility Restriction List IE in RLF Report.

As for the accessing NPN, we think it is also needed.

Let’s also consider the above scenario as in figure 1.

Cell 0, cell1 cell 2 and cell 3 support public network. At the same time, cell0, cell1 and cell3 also support SNPN i.e., cell0, cell1 and cell3 is shared by SNPN and PN, while cell 2 only supports PN.

Case 3: UE2 with NPN and PN capability, i.e. UE2 is allowed to access cell0, cell1, cell2 and cell3. UE2 first stay in cell 0 in PN, after handover failure, UE may initiate RRC reestablishment to cell 2 because cell 2 is a PN cell.

Case 4: UE3 with NPN and PN capability, i.e. UE3 is allowed to access cell0, cell1, cell2 and cell3. UE3 first stay in cell 0 in SNPN, after handover failure, UE may initiate RRC reestablishment to cell 3 because cell 3 is a SNPN cell. Note that UE3 can only select SNPN cell for RRC reestablishment because the network type should not be changed during handover.

The main difference for UE2 and UE3 is the first accessing network. When analyzing RLF Report for UE2 and UE3, without UE context, network cannot identify the difference between UE2 and UE3. Therefore, we propose to include accessing NPN in RLF Report.

Proposal 2: It is proposed to include accessing NPN in RLF Report.

  - MDT for NPN

Area scope for PNI-NPN

For the area scope of PNI-NPN, we reach a general solution in last meeting, but there still left some details and need further check, e.g., how to adjust the Semantics description to fully reflect our intention[1].

For the This IE is ignored if the Area Scope of MDT IE is set to “PNI-NPN based”, in last meeting, Samsung indicate this sentence may have conflict with the text procedure due to it is a “shall” behaviour. We all know the intention of introducing this semantics description is to reflect an abnormal case when the “PNI-NPN based” and “PNI-NPN Area Scope of MDT” present together. But seems in the spec., usually the abnormal case is not reflected in the tabular. So, we suggest just back to the previous version, that to say, delete this Semantics description, and add text procedure in the abnormal condition section.

If the target NG-RAN node receives a TRACE START message where the choice for the Area Scope of MDT IE is equal to PNI-NPN based, and if the PNI-NPN Area Scope of MDT IE is present, the the NG-RAN node shall ignore the PNI-NPN Area Scope of MDT IE.

Proposal 3: Delete the Semantics description for PNI-NPN Area Scope of MDT IE and add new text procedure in the abnormal case.

For the “If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only”, we think no need to add such limitation in the original CHOICE Area Scope of MDT. It is a little strange for us, when we only what to measurement PN cell and select the PLMN wide, all the cells in the PLMNs will be measurement, including the PNI-NPN cells. But when we want to measurement both PN and PNI-NPN, we give a further limitation. In our understanding, the PNI-NPN is based on the PN network, and MDT’s measurement result is almost the cell quality instead of some private information, no need to add this Semantics description. One acceptable way is that once PLMN wide/cell based/TA based/TAI based and PNI-NPN Area Scope for MDT appears together, UE will measure the union set of both two IE.

What’s more, we afraid this limitation may introduce extra work for RAN2, and we need to check with RAN2 about the feasibility before we decided to introduce such limitation.

Proposal 4: Delete the Semantics description and measure the union set when PNI-NPN Area Scope for MDT and CHOICE Area Scope of MDT for PN appeared together.

Proposal 5: If we want to introduce limitation for CHOICE Area Scope of MDT, we need a further check with RAN2.

Area scope for SNPN

For SNPN, it is operated by an NPN operator and not relying on network functions provided by a PLMN. The SNPN can be considered as a kind of special PLMN which cannot be equivalent with other PLMNs, so, the Area Scope of MDT for SNPN can follow the design of publish network, including cell-based, TA based, and SNPN wide.

Proposal 6: Support cell-based, TA based, and SNPN wide level area scope of MDT in SNPN.

In our understanding, there is no need to point out the NID in the area scope of SNPN. The propose of introduce NID is to identify a specific SNPN network, but at least in REL-17, SNPN doesn’t have the equivalent SNPNs, the UE can only connected in the register SNPN.

Observation 2: No need to introduce NID in area scope of MDT in SNPN as UE only can collection MDT measurement in the registered SNPN.

The next issue is how to introduce the area scope for SNPN: introduce a separately IE or extended the current IE. We notice in RAN2 REL-16 NPN discussion, to supporting SNPN, they only change the field description from “PLMN” to “PLMN/SNPN”. The same change can be reused in RAN3 for area scope for SNPN and PN. The simplest modification can be shown as below.

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN/SNPN wide |  |  | NULL | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only |  |  |

As a UE will not connect to the SNPN and PN at the same time, and according to the current NPN, when the UE move out of the SNPN, it will trigger de-register procedure.

When the UE in SNPN, the CHOICE Area Scope of MDT IE set to PLMN/SNPN wide will means SNPN wide, and UE can measurement the cells in the whole SNPN; When the UE in PN, the area scope IE set to PLMN/SNPN wide can be understood as PLMN wide. And even in the further discussion, area scope is still no need to identify SNPN, enhancement can be done in user consent. Similar understanding can be shared with cell based and TA based level. When the UE in SNPN, the CHOICE Area Scope of MDT IE set to Cell based, then the UE will measurement the cells in SNPN according to the provided Cell ID List for MDT IE. When the UE in SNPN, the CHOICE Area Scope of MDT IE set to TA based, then the UE will measurement the TA in SNPN according to the provided TAC, and vice versa.

What’s more, even supporting SNPN in further discussion, we still think the SNPN list is not needed to introduce in the area scope. The reason is that in PN, we only have PLMN wide. Following the same logic, SNPN wide is enough. And other limitation should be introduced in user consent, which will be checked early when execute MDT.

Proposal 7：No need to introduce a separately IE for supporting Area Scope of MDT in SNPN, which may make the structure looks redundancy.

Proposal 8: Extend the current CHOICE Area Scope of MDT IE by changing “PLMN wide” to “PLMN/SNPN wide” for supporting Area Scope of MDT in SNPN, discuss and agree the TP in attachment.

What’s more, this modification for SNPN has no impact on Xn as PLMN wide is not appeared in the CHOICE Area Scope of MDT IE on XnAP. And we checked the RAN2 Spec.[3] as below, if the area scope is not present (NULL), it will set as PLMN wide, and UE will measurement all the cells in the EPLMN.

3> if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport: 4> if areaConfiguration is not included in VarLogMeasConfig; or 4> if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig: 5> perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

Proposal 9: No spec. impact on XnAP for supporting SNPN in MDT.

user consent for NPN

In last meeting, we received the LS S3-231399[2] from SA3 about user consent for NPN. For PNI-NPN, SA3 suggested that using public network identifiers (PLMN ID) is sufficient. We think it can also make sense from RAN3 perspective, whether UE can configure MDT is according to the PLMN list in a PLMN bias, and which further limitation for the CAG is already provided in the area scope.

Proposal 10: For user consent of PNI-NPN, the MDT PLMN list IE is sufficient, no need to introduce CAG information.

For the SNPN case, SA3 assumes that the SNPN operator itself will develop operational functions that are specific to their use case. As such, SA3 has not identified any requirements for user consent in this case. But even it is decided by the SNPN operator, it still need an IE for indicating to the RAN whether MDT is allowed to be configured to a UE. The SNPN can be considered as a kind of special PLMNs, so it is reasonable to introduce paralleled user consent for SNPN, e.g., MDT SNPN ID. For further support ESNPN in further, we suggest to introduce a MDT SNPN list and the max number of SNPN can be set as 1 in this release.

Proposal 11: It is proposed to introduce user consent for SNPN, i.e. MDT SNPN list IE include the PLMN identify and NID.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Observation 1: Different UE NPN capability may result in different RRC reestablishment cell. UE NPN capability is needed for network optimization.

Proposal 1: It is proposed to include complete UE NPN capability i.e., NPN Mobility Information IE in Mobility Restriction List IE in RLF Report.

Proposal 2: It is proposed to include accessing NPN in RLF Report.

Proposal 3: Delete the Semantics description for PNI-NPN Area Scope of MDT IE and add new text procedure in the abnormal case.

Proposal 4: Delete the Semantics description and measure the union set when PNI-NPN Area Scope for MDT and CHOICE Area Scope of MDT for PN appeared together.

Proposal 5: If we want to introduce limitation for CHOICE Area Scope of MDT, we need a further check with RAN2.

Proposal 6: Support cell-based, TA based, and SNPN wide level area scope of MDT in SNPN.

Observation 2: No need to introduce NID in area scope of MDT in SNPN as UE only can collection MDT measurement in the registered SNPN.

Proposal 7：No need to introduce a separately IE for supporting Area Scope of MDT in SNPN, which may make the structure looks redundancy.

Proposal 8: Extend the current CHOICE Area Scope of MDT IE by changing “PLMN wide” to “PLMN/SNPN wide” for supporting Area Scope of MDT in SNPN, discuss and agree the TP in attachment.

Proposal 9: No spec. impact on XnAP for supporting SNPN in MDT.

Proposal 10: For user consent of PNI-NPN, the MDT PLMN list IE is sufficient, no need to introduce CAG information.

Proposal 11: It is proposed to introduce user consent for SNPN, i.e. MDT SNPN list IE include the PLMN identify and NID.

# 4. Reference

[1].