3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231632

E-meeting, 17 – 26 April, 2023

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on LS on RAN UE Id optionality

Document for:	Discussion and Decision

# 1	Introduction

We here provide elements for reply LS to SA5 following their LS to RAN3 in [1].

# 2	Discussion

SA5 indicates in their LS: “SA5 has continued to deliver enhancements to Trace/MDT.  All currently defined trace formats [TS 32.423 Rel-17] have used existing UE identifiers including those specified by RAN3.”

As per our understanding, the UE identifiers currently defined in TS 32.423 (Rel-17) are the ones specified in Annex A1 of this specification:

| ue | This element gives the ue identifier provided in trace activation messages. It includes: - the ue identifier type (attribute specification "idType") - the ue identifier value (attribute specification "idValue") This element shall not be present in the Trace record of E-UTRAN. |
| --- | --- |
| ue idType | Attribute specification that provides the ue identifier type (IMSI, IMEI (SV), TAC, or Public User Identity). For management based MDT, IMSI or IMEI(SV) can not be selected as ue idType. |
| ue idValue | Attribute specification that provides the ue identifier value, represented in decimal. This attribute is optional for management based MDT. |

The ue identifier values provided to the TCE are provided by the CN, including for management-based trace and MDT using the Cell Traffic Trace mechanism. RAN3 enhanced the Cell Traffic Trace mechanism in Rel-16 to support trace for Dual Connectivity, allowing management-based trace data to be correlated for a session originating from the secondary node of a Dual Connectivity deployment or from the gNB-CU-UP and gNB-DU in split RAN architecture deployments.

We therefore suggest to reply back to SA5 to take into account the Cell Traffic Trace mechanism enhanced in Rel-16 for support of trace/MDT sessions originating from the secondary node of a Dual Connectivity deployment or from the gNB-CU-UP and gNB-DU in split RAN architecture deployments.

A corresponding draft LS has been submitted to this meeting in [2].

Proposal: Reply back to SA5 as proposed in [2].

# 3	Conclusion

Proposal: Reply back to SA5 as proposed in [2].

# References

[1]		S5-232761 - R3-231121, LS on RAN UE Id optionality, SA5

[2]	R3-231633, [Draft] Reply LS on RAN UE Id optionality, Nokia, Nokia Shanghai Bell