3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231656

E-meeting, 17 – 26 April, 2023

Agenda item:	12.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) LB and AI/ML Information Exchange over Xn

Document for:	Text Proposal

# 1 	Introduction

In RAN3 #117bis-e the following agreements and FFS points were captured:

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

It is FFS whether a node requesting a prediction includes timing information in order to indicate for which time a prediction is requested.

Whether there is a need for prediction accuracy at a receiving node is FFS.

How to indicate validity time (e.g., implicitly with a new prediction when the previous prediction becomes invalid, explicitly with every prediction in the AI/ML output or by the request to the prediction) shall be discussed on a case by case basis.

In RAN3 #118 some open aspects were captured:

Event-based triggers can be used as one of the reporting options. FFS on the event-based reporting format.

Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can include predicted TNL capacity indicator, predicted slice available capacity, and predicted composite available capacity group.

In RAN3 #119 we made the following agreements:

The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting.

Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed.

Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed.

The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

Partial reporting mechanism is supported in the agreed AI/ML information procedures (AI/ML INFORMATION REQUEST/RESPONSE/UPDATE, which name is FFS). The solutions need to be discussed.

In this contribution, we provide our views on the open points and provide a Text Proposal for BL CR for TS 38.423 based on [1] in the Annex.

# 2	UE Performance Feedback

In the last meeting, we agreed that UE Performance feedback can be configured in the Class 1 AI/ML procedure and that the actual performance measurement is triggered in a Handover message. Even though we reached an agreement on this topic, it remains still an open issue how to indicate that UE performance is requested.

UE Performance feedback is sent after an AI/ML action for all the Rel.18 use cases, i.e., for AI/ML Energy Saving, AI/ML Load Balancing and for AI/ML Mobility Enhancements. Even though for the AI/ML Mobility use case for a single Handover scenario feedback could be requested per UE, for the AI/ML Load Balancing and AI/ML Energy Saving use cases UE performance may be needed based on a group of UEs that have been impacted by an AI/ML action and e.g., have been offloaded by a source node to a target node.

Proposal 1: It should be possible to collect UE performance of a single UE or of all UEs that were impacted by the same AI/ML Inference Output.

UE performance information calculation at the target node involves Average UE performance. This means that for the average to converge, the target node needs to receive enough samples of UE performance to produce the average measurement. However, during a Handover the UE context is typically more short-lived compared to the time needed to calculate UE performance at the target node. After UE context is released a source node is unable to correlate the performance information of the handed over UEs provided by the target to the UEs that were handed over.

Observation 1: After UE context is released a source node is unable to correlate UE performance feedback from the target node to the UEs that were handed over.

The designed procedures should be such that if a source node requests a target node to calculate UE performance information, the source should be able to correlate feedback information to the corresponding action. This requirement could also prevent that NG-RAN nodes request UE performance feedback from their respective target nodes in vain. This would create extra requirements on UE performance measurements from the target nodes, without clear use by the source nodes.

Proposal 2: If a source node requests a target node for UE performance feedback information it should be capable to correlate a UE or a number of UEs to the UE performance feedback information provided by the target node.

In our view, the source node placing a request for UE performance can be assumed to store information about UE context until receives the requested UE Performance Feedback from the target node. In particular, the source does not need to store all the information stored in UE context, but only a subset to correlate a UE to the UE performance Feedback corresponding to it.

Proposal 3: A source node shall store information to correlate the received UE performance from a Target node to a UE or a group of UEs for which UE performance is requested.

# 3	Partial Success

In the last meeting we agreed that partial reporting mechanism is supported in the new AI/ML related procedures, but the exact solutions remain open. The discussion around partial success in the reporting in the last meeting was motivated as a simplified method to exchange capability between neighbours. According to this proposal a node requesting measurements indicates in the request message a binary indication, whether it supports to receive successfully only some of the requested measurements or not. The motivation behind this is that if the measurements are needed for example to execute model inference a node may wish to receive all the requested measurements in full. In this case, partial success is not supported. Otherwise, if partial success is allowed, the responding node may provide only some of the requested measurements without leading to an error message. In the response message, the node may additionally provide a list of measurements that it is capable to provide and a list of measurements that it is not capable to provide.

Introducing such detailed mechanism for partial success doesn’t solve the initial intention to indicate a node’s capability to provide certain measurements.

If in the response message a neighbour indicates that some measurements are not supported, due to different reasons, this doesn’t provide information to the requesting node regarding when those measurements will be supported. For example, if a measurement is temporarily not available, this indication doesn’t tell the requesting node about when the measurement will become available. If a measurement is not available with the requested periodicity or with current combination of requested information as an example, the requesting node cannot know which are the correct parameters to request those measurements, e.g., in terms of a required periodicity, correct input combination, etc. So, this information provided by the responding node does not help the requesting node to place in a more efficient way measurement requests in the future.

Observation 2: If a node indicates it does not temporarily support a measurement or that it does not support a measurement with a certain combination of parameters, a neighbouring node requesting this information does not know when the measurement will be supported or with which parameters the measurement shall be requested again in the future.

Therefore, the indication by the responding node does not provide a node capability, but rather an incapability to provide certain information.

In addition, in practice it is possible that a node places a measurement request, out of which some measurements are mandatory while the rest may be only optional. The binary partial success flag does not have sufficient granularity to indicate to a neighbour “which” are the mandatory measurements from the requesting node’s point of view and which measurements could be optionally received. Also, since it is binary it does not enable a node that may be receiving multiple measurement requests to prioritize provision of some of those, in the absence of enough resources to provide them all.

Observation 3: Partial success flag is not granular enough to identify subsets of the requested measurements that may be mandatory, as opposed to optional.

At the same time, Resource Status Reporting over Xn supports partial reporting in the sense that if a node cannot provide all the requested measurements, this does not need to lead to a failure. As mentioned in TS 38.423: “ If some results of the admitted measurements in RESOURCE STATUS UPDATE message are missing, NG-RAN node1 shall consider that these results were not available at NG-RAN node2.”.

Observation 4: A limited form of partial reporting is supported in XnAP.

Furthermore, we think that simple methods, as already supported, over Xn are sufficient for AI/ML purposes, while at the same time re-use existing methods. When AI/ML is deployed in the network, nodes are expected to have an understanding on each other’s support of AI/ML functionality to a certain extent. OAM may configure the nodes so that they have a general understanding of each other’s capability, so that requests for measurements are not completely bound to errors.

Proposal 4: Reuse existing Xn principles to support partial success over XnAP for NG-RAN AI/ML use cases.

# 4	Event-based Reporting

Resource status procedure allows a NG-RAN node to report load measurements to a neighbouring NG-RAN node in a periodic way. During the resource status procedure, a measurement request from a neighbour is started and continues periodically until it is stopped by the requesting node.

Event-based reporting may be useful to enable a node receive predictions more infrequently from neighbouring NG-RAN nodes, for example, related to monitoring. In the current Resource Status Procedure, while the procedure is active it is not possible to control and limit the number of measurements received since the only reporting method is periodic. If a measurement is stopped between two nodes, then there are no more associations between them with respect to this measurement so no additional measurement information can be reported. On the other hand, monitoring only certain events capturing the updates in model inference from network nodes may allow to keep the reporting procedure active to collect Training Data and monitor impacting events to model inference function, while at the same time keep the amount of reporting measurements limited. For example, when it comes to predicted resource status information, it may be requested from a node to report predicted resource status information only when a predicted load exceeds a given threshold or when it drops below a threshold. Such would allow the association between the two nodes to inform each other about inference information, but at the same time to control the amount of received measurements.

Observation 5: Monitoring only certain events capturing updates in model inference from network nodes allows a network node to keep the associations of the reporting procedure active for collection of Training Data and for monitoring of impacting events, while at the same time keep the amount of reported measurements limited.

Such monitoring would be useful also when a node is only interested to train an AI/ML model satisfying a certain event, e.g., in this case satisfying conditions on the load values. In this case, periodic reporting would waste network resources. In fact, periodic reporting may not be suitable to train a certain event, since the defined period may not necessarily coincide with the time the event takes place.

Observation 6: Periodic reporting may not be suitable for monitoring certain events or updates in model inference, when a prediction exceeds or drops below a certain value.

Proposal 5: Introduce event-based reporting in the new procedure used for reporting of AI/ML related to detecting if a predicted load exceeds or is below a certain threshold.

One topic that has been discussed extensively in previous meetings is the validity time of prediction. In our view validity time, is a model related parameter which is not expected to change during the time a model is in use. An issue that may arise though with predictions during the reporting procedure is that the AI/ML Model providing those predictions may be updated by the node based on implementation. For example, this can happen because a node may have different models in use for different times in the day or load input scenarios. This may also happen if network conditions change considerably due to an error, e.g., a cell failure forcing UEs to be offloaded to a different cell and thus affecting the provided load prediction. Such issues may affect the validity time of a prediction. In our view a node should be allowed to switch transparently between different models providing a certain output based on its own implementation without the need to inform its neighbours about this event. However, usage of different models within the reporting time may change model related information during the reporting.

Even if a model is updated, providing new output characteristics the inference output may have no impacts on the receiving node, if the reporting procedure is flexible to compensate for those updates. For example, if the time interval during which a prediction is valid decreases slightly, a receiving node may still be able to use successfully this information. In some cases, it could even be possible that the node whose model inference characteristics have been updated tries itself to compensate for this change in the characteristics of model inference. Consider an example where a node produces model inference that has a certain validity time but in the course of time the node determines that the validity time of the prediction is halved. The node could try to compensate for this by adjusting the reporting period of the model inference and report twice as often. As another option, the node could return an error if it cannot provide inference with identical characteristics during the reporting period.

Proposal 6: Event-based reporting  may indicate changes or updates in the inference characteristics of a node (e.g., related to ML Model validity).

# 5	Predicted Load Reporting

One open aspect from earlier meetings is whether a node requesting a prediction indicates in the request the time that the prediction needs to be calculated. In our view, the time a prediction is calculated is an important input to the ML Model producing the predictions. Requesting calculation of a prediction at different times in the future may have different outputs, e.g., predicted load at rush hours is expected to be higher than predicted load during evenings.

Observation 7: The time when a prediction needs to be calculated is important input to the ML Model calculating predictions.

Furthermore, the time a prediction needs to be calculated is different from the time of reporting which can start when the node has the prediction available. For instance, a node may request a neighbour for predicted load after 15 minutes. The neighbour will need to execute a model and start the reporting as soon as the prediction is available.

Observation 8: The time when a prediction needs to be calculated is different from the starting of the reporting.

Proposal 7: The time of a prediction shall be indicated in the message requesting the prediction.

Each prediction requested by a node and produced by a neighbour has a certain validity time. The time of validity of a prediction depends on its accuracy and how far into the future the prediction is obtained. A prediction pertains to a random variable and is based on a probability and not on a certainty, which means that a prediction is not perfect. When a prediction is being made the more we try to predict into the future the more unreliable the results are going to be. For instance, if the variable to be predicted is the load of a neighbouring NG-RAN node, then the prediction will be more reliable if load is predicted 1 minute into the future as opposed to 15 minutes or a day into the future. The reason is that when the number of observations is small (predicting a value in the near future) then models can work reasonably well as an approximation to the process that generated the data. When the number of observations increases (predicting a value far in the future) then the number of different options increases making the prediction more unreliable (while at the same time optimization becomes more time consuming since the number of the observations involved is also higher).

Observation 9: A prediction is more accurate when it is predicted in the near future as opposed to later into the future.

The accuracy of the requested prediction is also an important factor in the usability of the received prediction. A node may only be interested in predictions that meet a certain accuracy requirement since otherwise using information of lower accuracy will impact its performance. Using inaccurate data may lead to bad decisions at the requesting node. Furthermore, utilizing low confidence predictions as an input to a ML Model will further lead to low confidence since the quality of input data influences the model output.

One possible option to determine whether a prediction is accurate is to compare the prediction with the corresponding (actual) measured values. However, this comparison can only happen later in time, meaning that a node needs to use a prediction for a while without knowing its accuracy. If for example predicted load information after 1 hour is requested and it happens to be inaccurate, the receiving node of this prediction may use inaccurate predicted load information for 1 hour before it determines through a comparison with the ground truth that it was inaccurate in reality. Learning the accuracy of a neighbour through comparisons with ground truth information may take several rounds of comparisons before a node can really determine how accurate information a neighbour is providing.

Observation 10: Determining accuracy of a prediction through comparisons with the ground truth means that a NG-RAN node has to use a prediction for a possibly long period of time without having any information on the prediction quality.

Furthermore, a node may update its deployed model to provide a certain prediction. Even further, a new model providing the same prediction may be deployed at a node. Currently, this is done transparently to the neighbours and there is no signaling in place to indicate that a new model is deployed or updated at a neighbouring node. Therefore, it is practically impossible for a neighbouring node to determine accuracy information from neighbouring nodes’ predictions through comparison with ground truth if the ML Models at a neighbouring node are updated often.

Observation 11: In certain cases, it may be impossible to determine accuracy of a prediction through comparison with ground truth information since an NG-RAN node may be using different ML Models for its predictions.

On the other hand, ML Model accuracy can be part of the ML Model input information. A node requesting predictions may only be interested in predictions of high confidence/accuracy, and no other information about predictions accuracy is relevant.  Naturally, accuracy and validity time are inter-related since a prediction of high confidence will likely have a shorter validity time as opposed to a prediction of low confidence. A node requesting predictions should place minimum requirements that the received predictions should satisfy in order to be useful to it. For example, if a node requests predicted load information to subsequently use it as an input to another local ML Model, the received predicted load information has to be valid sufficiently long and has to further have good enough confidence to eventually provide a reliable output. Therefore, a node requesting predictions must place minimum requirements that the requested predictions must satisfy to be useful to it.

Proposal 8: Trigger unsuccessful outcome if the NG-RAN node providing predictions can’t satisfy the minimum requirements placed by the requesting node.

To indicate inability of a node to meet the minimum requirements of the requested predictions a new cause value can be introduced.

Proposal 9: Introduce a new cause value corresponding to proposal 8.

A node may also be unable to provide the requested predictions. In this case, a different cause value could be introduced to indicate that the requested prediction is not supported.

Proposal 10: Introduce a new cause value to indicate whether a requested prediction is supported by a node.

Finally, we propose not to consider Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity at this point, until a clear need for those is identified.

Proposal 11: Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity can be introduced later if there is a need identified by a use case.

Finally, we provide a TP for BL CR for TS 38.423 in the Annex.

Proposal 12: Accept the TP for BL CR for TS 38.423 in the Annex.

# 6 			Conclusion

In this contribution we made the following observations and proposals:

Proposal 1: It should be possible to collect UE performance of a single UE or of all UEs that were impacted by the same AI/ML Inference Output.

Observation 1: After UE context is released a source node is unable to correlate UE performance feedback from the target node to the UEs that were handed over.

Proposal 2: If a source node requests a target node for UE performance feedback information it should be capable to correlate a UE or a number of UEs to the UE performance feedback information provided by the target node.

Proposal 3: A source node shall store information to correlate the received UE performance from a Target node to a UE or a group of UEs for which UE performance is requested.

Observation 2: If a node indicates it does not temporarily support a measurement or that it does not support a measurement with a certain combination of parameters, a neighbouring node requesting this information does not know when the measurement will be supported or with which parameters the measurement shall be requested again in the future.

Observation 3: Partial success flag is not granular enough to identify subsets of the requested measurements that may be mandatory, as opposed to optional.

Observation 4: A limited form of partial reporting is supported in XnAP.

Proposal 4: Reuse existing Xn principles to support partial success over XnAP for NG-RAN AI/ML use cases.

Observation 5: Monitoring only certain events capturing updates in model inference from network nodes allows a network node to keep the associations of the reporting procedure active for collection of Training Data and for monitoring of impacting events, while at the same time keep the amount of reported measurements limited.

Observation 6: Periodic reporting may not be suitable for monitoring certain events or updates in model inference, when a prediction exceeds or drops below a certain value.

Proposal 5: Introduce event-based reporting in the new procedure used for reporting of AI/ML related to detecting if a predicted load exceeds or is below a certain threshold.

Proposal 6: Event-based reporting  may indicate changes or updates in the inference characteristics of a node (e.g., related to ML Model validity).

Observation 7: The time when a prediction needs to be calculated is important input to the ML Model calculating predictions.

Observation 8: The time when a prediction needs to be calculated is different from the starting of the reporting.

Proposal 7: The time of a prediction shall be indicated in the message requesting the prediction.

Observation 9: A prediction is more accurate when it is predicted in the near future as opposed to later into the future.

Observation 10: Determining accuracy of a prediction through comparisons with the ground truth means that a NG-RAN node has to use a prediction for a possibly long period of time without having any information on the prediction quality.

Observation 11: In certain cases, it may be impossible to determine accuracy of a prediction through comparison with ground truth information since an NG-RAN node may be using different ML Models for its predictions.

Proposal 8: Trigger unsuccessful outcome if the NG-RAN node providing predictions can’t satisfy the minimum requirements placed by the requesting node.

Proposal 9: Introduce a new cause value corresponding to proposal 8.

Proposal 10: Introduce a new cause value to indicate whether a requested prediction is supported by a node.

Proposal 11: Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity can be introduced later if there is a need identified by a use case.

Proposal 12: Accept the TP for BL CR for TS 38.423 in the Annex.

# References

[1]		R3-231042, (BLCR to 38.423) for AI/ML for NG-RAN

# Annex – TP for BL CR for TS 38.423

This TP is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<<< skip unchanged procedural text >>>

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node considers the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with AI/ML Information Reporting procedure:

If the UE Performance Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, trigger the AI/ML Information Reporting procedure using the NG-RAN node2 Measurement ID contained in the UE Performance Request IE.

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

<<< skip unchanged procedural text >>>

#### 8.2.1.4	Abnormal Conditions

<<< skip unchanged procedural text >>>

If the HANDOVER REQUEST message includes information for a PLMN not serving the UE in the target NG-RAN node in the Management Based MDT PLMN List IE, the target NG-RAN node shall ignore information for that PLMN within the Management Based MDT PLMN List.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	FFS

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If Predicted Radio Resource Status Info IE is included in the AI/ML INFORMATION REQUEST then NG-RAN node1 requests predicted radio resource information from NG-RAN node2. Furthermore, additional input information may be included in the request:

- If time of prediction is included in the request, then NG-RAN node2 shall calculate predicted radio resources at the time indicated.

- If a minimum confidence level is included in the request, then NG-RAN node2 shall provide NG-RAN node1 with predicted radio resources that has at least an accuracy indicated by the minimum confidence level.

- If a minimum validity time is included in the request, then the predicted radio resources provided by  NG-RAN node2 shall be valid for at least a period of time indicated by the minimum validity time.

If Predicted Number of Active UEs Info IE is included in the AI/ML INFORMATION REQUEST then NG-RAN node1 requests a predicted number of active UEs from NG-RAN node2. Furthermore, additional input information may be included in the request:

- If time of prediction is included in the request, then NG-RAN node2 shall calculate a predicted number of active UEs at the time indicated.

- If a minimum confidence level is included in the request, then NG-RAN node2 shall provide NG-RAN node1 with a predicted number of active UEs that has at least an accuracy indicated by the minimum confidence level.

- If a minimum validity time is included in the request, then the predicted number of active UEs provided by  NG-RAN node2 shall be valid for at least a period of time indicated by the minimum validity time.

If Predicted RRC Connections Info IE is included in the AI/ML INFORMATION REQUEST then NG-RAN node1 requests a predicted number of RRC Connections from NG-RAN node2. Furthermore, additional input information may be included in the request:

- If time of prediction is included in the request, then NG-RAN node2 shall calculate a predicted number of RRC Connections at the time indicated.

- If a minimum confidence level is included in the request, then NG-RAN node2 shall provide NG-RAN node1 with a predicted number of RRC Connections that has at least an accuracy indicated by the minimum confidence level.

- If a minimum validity time is included in the request, then the predicted number of RRC Connections provided by  NG-RAN node2 shall be valid for at least a period of time indicated by the minimum validity time.

Interaction with AI/ML Information Reporting procedure and the Handover Preparation procedure:

If the fourth bit of the Report Characteristics IE is set to 1, the AI/ML Information Reporting procedure is triggered according to information in the UE Performance Request IE included in the HANDOVER REQUEST message.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If any of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AI/ML INFORMATION REQUEST message or the AI/ML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AI/ML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AI/ML INFORMATION REQUEST message is identical to the content of the previously unacknowledged AI/ML DATA REQUEST message.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the AI/ML INFORMATION REQUEST message then NG-RAN node2 shall initiate an AI/ML INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives an AI/ML INFORMATION REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1Measurement ID IE corresponding to an existing on-going AI/ML information reporting, then NG-RAN node2 shall initiate an AI/ML INFORMATION FAILURE message with an appropriate cause value.

FFS

### 8.4.BB	AI/ML Information Reporting (FFS on the name)

#### 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information accepted by the NG-RAN node following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: AI/ML Information Reporting, successful operation

NG-RAN node2 shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

If some results of the admitted measurements in AI/ML INFORMATION UPDATE message are missing, NG-RAN node1 shall consider that these results were not available at NG-RAN node2.

#### 8.4.BB.3	Unsuccessful Operation

Not applicable.

#### 8.4.BB.4	Abnormal Conditions

Void

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |
| UE Performance Request | O |  | AIML NG-RAN Node2 Measurement ID 9.2.3.Y2 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| AIML NG-RAN node1 Measurement ID | M |  | 9.2.3.Y1 |  | YES | reject |
| AIML NG-RAN node2 Measurement ID | C-ifRegistrationRequestStop |  | 9.2.3.Y2 |  | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = UE Performance FFS on the coding | YES | reject |
| Predicted Radio Resources Reporting Configuration | O |  |  |  | YES | ignore |
| > Time of prediction | O |  | INTEGER (0..4095) | Corresponds to the time of prediction. Unit: [second]. |  |  |
| >Minimum confidence level | O |  | INTEGER (1..100) | This IE indicates a minimum confidence level for the prediction. |  |  |
| >Minimum validity time | O |  | INTEGER (0..4095) | Corresponds to the time a prediction is valid. Unit: [second]. |  |  |
| Predicted Number of Active UEs Reporting Configuration | O |  |  |  | YES | ignore |
| > Time of prediction | O |  | INTEGER (0..4095) | Corresponds to the time of prediction. Unit: [second]. |  |  |
| >Minimum confidence level | O |  | INTEGER (1..100) | This IE indicates a minimum confidence level for the prediction. |  |  |
| >Minimum validity time | O |  | INTEGER (0..4095) | Corresponds to the time a prediction is valid. Unit: [second]. |  |  |
| Predicted Number of RRC Connections Reporting Configuration |  |  |  |  | YES | ignore |
| > Time of prediction | O |  | INTEGER (0..4095) | Corresponds to the time of prediction. Unit: [second]. |  |  |
| >Minimum confidence level | O |  | INTEGER (1..100) | This IE indicates a minimum confidence level for the prediction. |  |  |
| >Minimum validity time | O |  | INTEGER (0..4095) | Corresponds to the time a prediction is valid. Unit: [second]. |  |  |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Predicted Radio Resource Status Event Configuration Information | ifReportCharacteristicsFirstBitIsSet |  |  | Triggering conditions for reporting based on Predicted Radio Resource Status |  |  |
| > Threshold min | M |  | FFS | Event triggered if Predicted Radio Resource Status is below the threshold |  |  |
| > Threshold max | M |  | FFS | Event triggered if Predicted Radio Resource Status is above the threshold |  |  |
| > Threshold Condition | M |  | ENUMERATED (outside, inside, ...) | Code-point ‘outside’ indicates that the measured value is reported if lower than Threshold min or higher than Threshold max. Code-point ‘inside’ indicates that the measured value is reported if higher than Threshold min and lower than Threshold max. |  |  |
| Predicted Number of Active UEs Event Configuration Information | ifReportCharacteristicsSecondBitIsSet |  |  | Triggering conditions for reporting based on Predicted Number of Active UEs |  |  |
| > Threshold min | M |  | FFS | Event triggered if Predicted Number of Active UEs is below the threshold |  |  |
| > Threshold max | M |  | FFS | Event triggered if Predicted Number of Active UEs is above the threshold |  |  |
| > Threshold Condition | M |  | ENUMERATED (outside, inside, ...) | Code-point ‘outside’ indicates that the measured value is reported if lower than Threshold min or higher than Threshold max. Code-point ‘inside’ indicates that the measured value is reported if higher than Threshold min and lower than Threshold max. |  |  |
| Predicted RRC Connections Event Configuration Information | ifReportCharacteristicsThirdBitIsSet |  |  | Triggering conditions for reporting based on Predicted RRC Connections |  |  |
| >> Threshold min | M |  | FFS | Event triggered if Predicted RRC Connections is below the threshold |  |  |
| >> Threshold max | M |  | FFS | Event triggered if Predicted RRC Connections is above the threshold |  |  |
| > Threshold Condition | M |  | ENUMERATED (outside, inside, ...) | Code-point ‘outside’ indicates that the measured value is reported if lower than Threshold min or higher than Threshold max. Code-point ‘inside’ indicates that the measured value is reported if higher than Threshold min and lower than Threshold max. |  |  |
| UE Performance Reporting Configuration | ifReportCharacteristicsFourthBitIsSet |  |  |  | YES | ignore |
| >Starting time | O |  | INTEGER (0..4095) | Corresponds to the starting time of the UE performance measurement after successful handover completion. Value 0 indicates that the measurement is started immediately upon successful handover completion. Unit: [second]. |  |  |
| >Averaging window | O |  | INTEGER (0..60) | Corresponds to the measurement averaging window. Value 0 indicates single measurement. Unit: [second]. |  |  |
| >>Direction | M |  | ENUMERATED (ul, dl, both, ...) |  |  |  |
| >>UE Performance Metric | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Averaged Throughput, Second Bit = Averaged Packet Delay, Third Bit = Averaged Packet Loss |  |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifReportCharacteristicsFirstBitIsSet | This IE shall be present if the first bit of the Report Characteristics IE is set to 1 |
| ifReportCharacteristicsSecondBitIsSet | This IE shall be present if the second bit of the Report Characteristics IE is set to 1 |
| ifReportCharacteristicsThirdBitIsSet | This IE shall be present if the third bit of the Report Characteristics IE is set to 1 |
| ifReportCharacteristicsFourthBitIsSet | This IE shall be present if the fourth bit of the Report Characteristics IE is set to 1 |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all of the measurement objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| AIML NG-RAN node1 Measurement ID | M |  | 9.2.3.Y1 |  | YES | reject |
| AIML NG-RAN node2 Measurement ID | M |  | 9.2.3.Y2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of  the requested measurement objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| AIML NG-RAN node1 Measurement ID | M |  | 9.2.3.Y1 |  | YES | reject |
| AIML NG-RAN node2 Measurement ID | M |  | 9.2.3.Y2 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| AIML NG-RAN node1 Measurement ID | M |  | 9.2.3.Y1 |  | YES | reject |
| AIML NG-RAN node2 Measurement ID | M |  | 9.2.3.Y2 |  | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item (FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resources Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>FFS | O |  |  |  | - | - |
| UE Performance Result | O |  | 9.2.3.Y |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.Y	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | 9.2.3.4 |  |
| Average UE Throughput UL | O |  | 9.2.3.4 |  |
| Average Packet Delay | O |  | FFS |  |
| Average Packet Loss | O |  | FFS |  |

#### 9.2.3.x	Cell Based UE Trajectory Prediction

Editors Note: Actual name of this IE is FFS.

The Cell Based UE Trajectory Prediction IE contains the list of NG-RAN cells where the UE is predicted to connect.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Predicted Trajectory Cell List |  | 1..<maxnoofCellsTrajectoryPredict> |  | List of cells where the UE is predicted to connect, in chronological order. The first predicted cell added to the top of this list, is the cell where the UE will move to after the serving cell. |
| >Predicted Trajectory Cell Information | M |  | 9.2.3.Z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectoryPredict | Maximum number of cells that can be predicted for UE trajectory. Value is FFS. |

#### 9.2.3.Z	Predicted Trajectory Cell Information

The Predicted Trajectory Cell Information contains the cell ID of the predicted NG-RAN cell for cell based UE trajectory prediction.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Predicted Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Predicted Time UE Stays in Cell | (FFS) |  | INTEGER (0..4095) | The duration of time the UE is expected to stay in the cell, or set of NG-RAN cells (FFS on the meaning of the set of NG-RAN cells), in seconds. If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.Y1	AIML NG-RAN Node1 Measurement ID

This IE indicates the AIML NG-RAN Node1 Measurement ID.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| AIML NG-RAN Node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated at the NG-RAN Node1 |

#### 9.2.3.Y2	AIML NG-RAN Node2 Measurement ID

This IE indicates the AIML NG-RAN Node2 Measurement ID, NG-RAN Node1.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| AIML NG-RAN Node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated at the NG-RAN Node2 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

The meaning of the different cause values is specified in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the NG-RAN node2 Proposed Mobility Parameters IE is too low or too high. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| RSN not available for the UP | The redundant user plane resources are not available. |
| NPN Access denied | Access denied, or release is required, due to NPN reasons. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the S-NG-RAN node for UE power saving purpose. |
| Not existing NG-RAN node2 Measurement ID | The action failed because the NG-RAN node2 Measurement ID is not used. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |

| Transport Layer cause | Meaning |
| --- | --- |
| Transport resource unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | NG-RAN node control processing overload. |
| Hardware Failure | NG-RAN node hardware failure. |
| Not enough User Plane Processing Resources | NG-RAN node has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to NG-RAN node equipment. |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

| AI/ML cause | Meaning |
| --- | --- |
| AI/ML Input Requirements Not Satisfied | Requested AI/ML model input requirements, e.g., accuracy or validity, of prediction are not satisfied |
| AI/ML Prediction Not Supported | Requested AI/ML prediction is not supported by NG-RAN node |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>