3GPP TSG-RAN WG3 #119bis-e                                                                            R3-231669

Online, 17th – 26th Apr, 2023

Agenda Item:	17.2

Source:	CATT

Title:	Discussion on Mobility and Service Continuity enhancements for NTN

Document for:	Decision

1Introduction

In the last RAN3 meeting, we further discussed the mobility and service enhancements for NTN. According to the discussion, some agreements are achieved [1], as below:

Agreements:

- Turn WA to agreement: The Uu cell ID is used as target Cell ID in both NG and Xn handover signaling.

- Introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact.

However, there’re still some open issues, listed in below:

Whether to use Uu cell ID or mapped cell ID over Xn configuration update procedure?

The idea of the solution needs to be further checked.

In this contribution, we will further discuss the open issues on mobility enhancements, and provide corresponding observations and proposals.

2. Discussion

2.1 Xn Setup/Configuration Update

FFS on whether to use Uu cell ID or mapped cell ID over Xn configuration update procedure.

The last RAN3 meeting has discussed what’s the TAC in Xn Setup/Configuration Update, but don’t make significant progress. Then the last meeting suggested to discussing whether to use Uu cell ID or mapped cell ID over Xn configuration update procedure firstly. In the BL CR [2], the following changes towards TS38.300 have been agreed.

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

Editor’s Note: Whether the Xn procedures could follow legacy behavior by using Uu Cell ID is FFS

In Rel-17, the introduction of the mapped Cell ID is to satisfy the requirement of SA2, and RAN3 discussed and agreed it could be used in the above procedures. Following the agreement of Rel-17, the mapped Cell ID is not used in Xn Setup and Configuration Update procedures. Now, we do not see any clear benefits to exchange mapped Cell ID(s) in Xn Setup and Configuration Update procedures. Therefore, we should not extend the using of mapped Cell ID without clear motivation. Therefore, Uu Cell ID is suggested to be used in Xn Setup and Configuration Update procedures.

Observation 1: There’s no clear requirement/benefit to exchange mapped Cell ID(s) in Xn Setup and Configuration Update procedures.

Proposal 1: Uu Cell ID should be used in Xn Setup and Configuration Update procedures.

What’s the TAC in Xn Setup/Configuration Update

For quasi earth fixed cell case, the TAC(s) of each cell is static. In normal cases, one earth fixed cell should broadcast one TAC per PLMN, even for cross country scenario. Thus, there’s no need to support a TAC list per PLMN in the Xn interface and no enhancement is required in this case.

Observation 2: For quasi earth fixed cell case, one earth fixed cell should broadcast one TAC per PLMN and no enhancement is required in this case.

For earth moving cell case, one cell may broadcast one or more TACs in a PLMN, and the broadcast TACs of a cell may change with time as the coverage of the cell is changed with time. Including the multiple TACs per PLMN in the Served Cell Information may require NG-RAN Configuration Update frequently, and we do not see any real benefit to do such kind of things.

In the Xn Setup procedure, the List of Served Cells NR to be exchanged between the gNBs is optional, the tabular specified in TS 38.423 is shown as below:

| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |

From implementation point of view, it’s possible to exchange the served cells and neighbour relations in case of earth fixed cells are deployed, like the way in TN. It may not need to exchange the served cells and neighbour relations in case of earth moving cells are deployed. The served cell information and its neighbour relations could be left to OAM.

Observation 3: For EMC case, it’s not necessary to exchange the List of Served Cells NR with the neighbour gNBs.

Above all, it’s un-necessary to exchange multiple TACs per PLMN for NTN cells in XN Setup procedure and Configuration Update procedure. It could be configured by OAM, if needed. Base on observation 1 and observation 2, both quasi earth fixed cell case and earth moving cell case could follow legacy behaviour. Therefore, using Uu Cell ID in Xn procedures could follow legacy behaviour and no issue is identified to Uu Cell ID in Xn procedures.

Proposal 2: It’s un-necessary to exchange multiple TACs per PLMN for NTN cells in XN Setup procedure and Configuration Update procedure.

Proposal 3: The Xn procedures could follow legacy behaviours by using Uu Cell ID and no issue is identified to Uu Cell ID in Xn procedures.

According to the discussion above, the corresponding stage 2 TP could be found in [3].

2.2 NGAP enhancements

The idea of the solution needs to be further checked.

The last RAN3 meeting has agreed that introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact. NG-based HO introduced the Time Based Handover Information in the Source to Target Container which is forwarded transparently to the target NG-RAN node. NGAP signalling works in the same way as the agreed Xn CHO which means that it’s beneficial to support also in NGAP, and the detailed analysis can be found in [4]. There is no doubt that introducing the Time Based Handover Information in the Source to Target Container is a necessary enhancement.

Observation 4: There is no doubt that introducing the Time Based Handover Information in the Source to Target Container for NGAP is an essential enhancement.

Data forwarding which is one potential issue for NGAP enhancements

From the previous RAN3 meetings, some companies raised the issue about data forwarding, and considered the impact on NGAP. After NG-based HO introduced the Time Based Handover Information, source gNB may start the data forwarding after the source gNB send the HO Command to the UE, but the UE will not immediately execute the handover. The UE may synchronize to target cell at any time during the handover window. In the duration time between handover window start time (i.e. T1) and the time UE successfully connected to target gNB, some DL packets have been successfully delivered to the UE from the source gNB, the packets are also forwarded from the source gNB to the target gNB.

If the duration time of the handover window is short (e.g. less than 1 second), not too much data are forwarded from source gNB to target gNB during the handover. Therefore, this problem is not essential and the existing handover mechanism should be work.

If the duration time of the handover window is a bit long, the target gNB may need to buffer and resend large amounts of data, even if the data has been successfully transferred to the UE by source gNB, more buffer is required for the gNB, and the fresh data will be delayed due to re-transmission of the forwarded data, even though the data may already be delivered to the UE from the source gNB.

The CR [4] was provided in the last meeting, and the key information of the handover time window is as below:

| Time Based Handover Information |  | 0..1 |  | This IE only applies to NTN. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to t1-Threshold defined in TS 38.331 [10] | - |  |
| >Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to duration defined in TS 38.331 [10] | - |  |

Refer to the above design of handover time window information, it’s possible that the handover window duration time is very long, which is possible theoretically. Although the implementation is generally not so long, based on different deployment and different base station implementation, there may be appear the possibility with a long handover window duration time. Therefore, to avoid the unnecessary data buffer in target gNB, it’s proposed to consider the issue of data forwarding when the handover window duration time is very long. There may be some benefits to enhancing data forwarding (i.e. indicate target gNB the successfully delivered PDU SDU), and it’s proposed RAN3 further discuss this issue in the future meetings.

Proposal 4:  To support time based CHO in Uu, there may be some benefits to enhance data forwarding for NG handover, and its proposed RAN3 to further discuss this issue in the future meetings.

3. Conclusion

In this contribution, we discussed the open issues to support Mobility enhancement in Rel-18. Based on the discussion above, we provided the following observations and proposals:

Observation 1: There’s no clear requirement/benefit to exchange mapped Cell ID(s) in Xn Setup and Configuration Update procedures.

Proposal 1: Uu Cell ID should be used in Xn Setup and Configuration Update procedures.

Observation 2: For quasi earth fixed cell case, one earth fixed cell should broadcast one TAC per PLMN and no enhancement is required in this case.

Observation 3: For EMC case, it’s not necessary to exchange the List of Served Cells NR with the neighbour gNBs.

Proposal 2: It’s un-necessary to exchange multiple TACs per PLMN for NTN cells in XN Setup procedure and Configuration Update procedure.

Proposal 3: The Xn procedures could follow legacy behaviours by using Uu Cell ID and no issue is identified to Uu Cell ID in Xn procedures.

Observation 4: There is no doubt that introducing the Time Based Handover Information in the Source to Target Container for NGAP is an essential enhancement.

Proposal 4:  To support time based CHO in Uu, there may be some benefits to enhance data forwarding for NG handover, and its proposed RAN3 to further discuss this issue in the future meetings.

4. References

- RAN3#119 Chair Notes

- R3-231033 Clarification on Cell Identifier used for handover signalling and XN Interface

- R3-231255 Cell ID over Xn for NTN

- R3-230438 Time-Based HO for NTN - NGAP Impacts