3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231677

# 17th Apr – 26th Apr 2023	 Online

Agenda Item:	18.2

Source:	CATT

Title:	Discussion on supporting discontinuous in IoT NTN

Document for:	agreement

1. Introduction

In TR23.700-28[1], SA2 has conclusion their study about discontinuous network coverage for both EPC and 5GS. And they also agreed the corresponding CR for TS 32.501 to support discontinuous network coverage in NR. But in RAN3, we only support discontinuous network coverage in E-UTRA.

2. Discussion

2.1 Whether supporting connected to AMF in IoT NTN

In SA2 studies, they provide the solutions for both EPS and 5GS in order to deal the discontinuous satellite coverage. The relative assumptions and principles are excerpt as below.

Solutions should be applicable to both EPS and 5GS. Discontinuous satellite coverage under EPC and 5GC interworking is not considered. Solutions shall minimize the impact on 5GS and EPS system protocols.

Based on the above description, SA2’s solution also include the case that UE connected to AMF, which is different with current RAN3 understanding.

Observation 1:SA2’s solution for discontinuous coverage is applicable to both 5GC and EPC.

Considered that the discontinuous coverage is only suitable for IoT NTN in RAN, in previous RAN3 discussion, we only focus on the case that UE is connected to EPC by eNB and just analysis the interface impact on S1AP and X2AP. For connecting to 5GC in IoT NTN, one valid case is that UE connected to ng-eNB, then to a satellite access to 5GC.

As RAN3 is also follow the discussion of SA2, in order to keep consists with SA2, it is natural to also support UE connecting to ng-eNB case. Thus, the enhancement in S1AP should always be introduced NGAP, such as introducing new cause value and adding new procedure text in stage 2. And also, an LS should be sent to SA2 for align our understanding. The draft LS is provided in appendix B.

Proposal 1: Supporting UE connects to ng-eNB then to the 5GC in discontinues coverage, and introduces new cause value for discontinues coverage in both stage 2 and stage 3.

Proposal 2: Send LS to SA2 to align our understanding, draft LS is presents in appendix A.

2.2 Time based CHO supporting

In last meeting, we have decided to support time-based CHO in IoT NTN and we added a new IE in the HANDOVER REQUEST message including Handover Window Start IE and Handover Window Duration IE.

And we also updated the procedural text as below

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use this information to allocate necessary resources for the incoming CHO. If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message but the UE does not appear in the candidate cell before the handover window duration indicated, the target eNB shall consider the conditional handover as canceled.

We mentioned that when UE is not appeared in the schedule time, the target eNB shall consider the conditional handover as cancelled, and we define the schedule time as the handover window duration. Of course, this mechanism can help eNB schedule effectively, but there was an unclearly point that what will happen if UE just arrived nearly the end of handover window duration. For simply the discussion, we call the start time as T1, and the end time of the window duration is T2.

In this case, UE arrived the new cell and starts RACH procedure at T (very nearly T2) and should have finished it at (T+T304). Once T+T304>T2, flowing the specified in above procedural text, the target eNB shall considered the conditional handover as cancelled at T2 and release the prepared resources right away. So, even UE appeared before T2, the time-based CHO will still failed, which is not align with our intention.

This issue is easy to solve, eNB should keep the allocated resources a little longer than T2, e.g., release the resources at (T2+T304). Thus, even UE just nearly T2, it can still finish the time-based CHO and connected successfully. But in our understanding, the T304 is totally belonging to RAN2, so we don’t want to mention it in our RAN3 spec. So we suggest to just delete the limitation about when to cancel the allocated resource and give more flexible to eNB implementation.

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use this information to allocate necessary resources for the incoming CHO.

Proposal 3: Delete the limitation about when to cancel the allocated resource and give more flexible to eNB implementation.

Proposal 4: Agree the TP in Appendix B and introduce the same modification in NR NTN.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1:SA2’s solution for discontinuous coverage is applicable to both 5GC and EPC.

Proposal 1: Supporting UE connected to ng-eNB then to the 5GC in discontinues coverage, and introduces new cause value for discontinues coverage in both stage 2 and stage 3.

Proposal 2: Send LS to SA2 to align our understanding, draft LS is presents in appendix A.

Proposal 3: Delete the limitation about when to cancel the allocated resource and give more flexible to eNB implement.

Proposal 4: Agree the TP in Appendix B and introduce the same modification in NR NTN.

4. Reference

[1]	TR23.700-28, Study on Integration of satellite components in the 5G architecture

5. Appendix A LS to SA2

3GPP TSG-RAN Meeting #119bis	R3-23xxxx

17th Apr – 26th Apr 2023 Online

Title:	[Draft] LS on UE connected to 5GC in IoT NTN discontinuous coverage

Release:	Rel-18

Work Item:	IOT_NTN_enh

Source:	CATT (to be RAN3)

To:	SA2

Cc:

Contact Person:

      - Name:	Jiaying Sun

      - E-mail Address:	sunjiaying@CATT.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 noticed that SA2 solutions for discontinues coverage are applicable to both EPS and 5GS, but in RAN, we only considered discontinues coverage for IoT NTN connected to EPS.

RAN3 kindly ask SA2 whether should we consider the case that IoT NTN UE connected to AMF and supporting corresponding enhancement.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take the above information into account.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #120      22th – 26th, May 2023 		 Incheon , KR

TSG RAN WG3 Meeting #121      21th – 25th, Aug. 2023 		 Toulouse , FR

Appendix B TP for IoT NTN for TS 36.413

****************** START CHANGE ***************

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source eNB initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. When the source eNB sends the HANDOVER REQUEST message, it shall start the timer TRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target eNB shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

UNCHANGED PART OMITTED

If the Maximum Number of CHO Preparations IE is included in Conditional Handover Information Acknowledge IE contained in the the HANDOVER REQUEST ACKNOWLEDGE message, then the source eNB should not prepare more candidate target cells for a CHO for the same UE towards the target eNB than the number indicated in the Maximum Number of CHO Preparations IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use the information to allocate necessary resources for the incoming CHO.

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use this information to T2+ for the incoming CHO.

If the EPC Handover Restriction List Container IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and shall use it as specified in TS 36.300 [15].

UNCHANGED PART OMITTED

****************** NEXT CHANGE ***************