3GPP TSG-RAN WG3 Meeting #120	R3-233375

Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	24.2

Source:	Huawei

Title:	Summary of offline discussions on CB: # 36_R18ES

Document for:	Discussion and approval

# 1	Introduction

CB: # 36_R18ES

- Discuss the possibility to introduce the new cause value as “SSB not Available”?

(moderator - HW)

Summary of offline disc R3-233375.

# 2	For Chairman’s note

Proposal A:  Note the TP for F1AP in R3-233465 (revision of R3-232902).

Mark the following to be continued in the chairman notes:

- Resolve the FFS(es) in the BLCR including the criticality of the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, the maximum number of the SSB areas etc.

To be continued at the next meeting (by contribution driven):

Inter-node Beam Activation

- Cause value for SSB activation failure

- Beams deactivation: Whether to introduce a cause for beams deactivation (e.g. ‘energy saving’) over Xn/F1

- Timer for SSB activation

# 3 Discussion

## 3.1 Inter-node Beam Activation

- Cause value for SSB activation failure

- No need to introduce a new cause value (e.g., “SSB not Available” or “SSB activation not Allowed”) to indicate the SSB activation failure???

- Update the BLCR for F1

Criticality of the new IE in GNB-CU CONFIGURATION UPDATE message

- Change the criticality of the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message to “ignore”, and remove the editor’s note “Editor’s Note: The SSBs within the cell to be Activated List IE may be further refined (e.g., the assigned criticality).”

The number of activated/disactivated SSBs

- Remove the FFS on the maximum number of the SSB areas.

- Beam activation: Whether the DU sends its own preferred beam activation to the CU

- No need for the DU to send its own preferred beam activation to CU for final decision???

- Beams deactivation: Whether to introduce a cause for beams deactivation (e.g. ‘energy saving’) over Xn/F1

- Introduce a new cause in the Coverage Modification Notification for F1AP and in the Coverage Modification List for XnAP to indicate “network energy saving” for SSB beam deactivation???

- Timer for SSB activation

- No need to introduce an explicit timer for SSB activation???

## 3.2 Cell DTX/DRX parameters

WA: Support the exchange of the Cell DTX/DRX configuration over Xn.

The detail infor of the Cell DTX/DRX configuration is pending to RAN1 and RAN2.

- in CU-DU split architecture, the Cell DTX/DRX parameters are finally decided by the gNB-DU

## 3.3 Paging on restricted area

To be continued at the next meeting based on reply LS.

## 3.4 Others

NES mode

Pending at this meeting.

# 3	Comments if any

| Company | Comment |
| --- | --- |

# 4	References

| R3-232577 | LS on Cell DTX/DRX activation/deactivation (RAN2(Huawei)) | LS in noted |
| --- | --- | --- |
| R3-232589 | (TP to TS 38.423) Support of network energy saving techniques (Nokia, Nokia Shanghai Bell) | other |
| R3-232902 | (TP to Netw_Energy_NR BLCR for TS 38.473, 38.470 and 38.300) Network energy saving techniques (Huawei) | other |
| R3-233285 | TPs to BL CRs for network energy saving (ZTE) | other |
| R3-232594 | (TP to TS 38.473) Beam deactivation decision and signalling for energy saving (Nokia, Nokia Shanghai Bell) | other |
| R3-232813 | Discussion on network energy saving (Samsung) | discussion |
| R3-232814 | (CR to 38.413) Introduction of Network Energy Saving for Paging IDLE UE (Samsung) | CR0993r, TS 38.413 v17.4.0, Rel-18, Cat. B |
| R3-232853 | Paging enhancements, Inter-node beam activation and Cell DTX/DRX (Qualcomm Incorporated) | discussion |
| R3-232890 | (TP to Netw_Energy_NR BLCR for TS 38.423) Network energy saving techniques (Huawei) | other |
| R3-232968 | Introduction of Network Energy Saving (Ericsson) | discussion |
| R3-232969 | Text Proposal on F1AP: Introduction of Network Energy Saving (Ericsson) | other |
| R3-233045 | Discussion on NES related issues (Lenovo) | discussion |
| R3-233246 | Discussion on Paging Enhancement (TP for 38.423) (CATT) | other |
| R3-233284 | Discussion on network energy saving (ZTE) | discussion |