3GPP TSG-RAN WG3 #120																			         R3-232687

22th – 26th May 2023

Incheon, Korea

Title: 	(TP to 38.401, 37.480) Introduction on MT-SDT

Source: 	ZTE

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

For E1AP, we have agreed as below.

Include an MT-SDT Information Request IE as optional IE in the E1AP: BEARER CONTEXT SETUP REQUEST/ MODIFICATION message to request the report of MT-SDT Information for bearers configured as SDT bearers.

The encoding and the name of MT-SDT information IE in E1AP DL DATA NOTIFICATION message include MT-SDT Data Size IE (Mandatory, INTEGER (1…96000, …). FFS on whether MT-SDT indicator IE is needed.

Proposal 1: Add MT-SDT procedure description (e.g., MT-SDT Data Size) into TS37.480.

In BLCR to 38.300, after receiving Uu MT-SDT paging, the UE may initiate MT-SDT procedure with non-SDT RACH resource, which is different from MO-SDT.

Proposal 2: Add MT-SDT special steps into TS38.401.

The encoding and the name of MT-SDT information IE in F1AP: Paging message include MT-SDT indicator IE (Mandatory). FFS on MT-SDT Data Size IE.

In our view, either gNB-CU or gNB-DU can generate and store the the MT-SDT Data Size threshold, and compare the total data size for all SDT bearers with MT-SDT Data Size threshold, and make the decision on MT-SDT procedure. Considering the above previous agreement, we can have the following proposal.

Proposal 3: Indicate that either gNB-CU or gNB-DU can decide to trigger MT-SDT procedure.

- Text proposal

  - TP to TS 38.401

===================Start Change=============================

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1a-0.	The gNB-CU-UP receives only DL SDT data for the UE in RRC Inactive on NG-U interface.

1a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If supported, the gNB-CU-UP include the MT-SDT information in the DL DATA NOTIFICATION message.

NOTE: The gNB-CU-CP may request the gNB-CU-UP to provide such MT-SDT information before step 1a-0, via BEARER CONTEXT SETUP REQUEST message or BEARER CONTEXT MODIFICATION REQUEST message.

1b. The gNB-CU-CP receives DL signalling over NGAP.

2.	After 1a or 1b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

3.	The gNB-DU sends the Paging message to the UE, the MT-SDT indicator may be included in the Paging message.

Editor’s Note: whether gNB-DU can make further decision on MT-SDT paging is FFS.

4. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- the UE indicates the RRC Resume Request is for MT-SDT, which may be without UL data, which may use non-SDT RACH resource.

===================End of Change=============================

  - TP to TS 37.480

===================Start Change=============================

5.1.2	E1 bearer context management function

The establishment of the E1 bearer context is initiated by the gNB-CU-CP and accepted or rejected by the gNB-CU-UP based on admission control criteria (e.g., resource not available).

The modification of the E1 bearer context can be initiated by either gNB-CU-CP or gNB-CU-UP. The receiving node can accept or indicate failure to carry out the modification request. The E1 bearer context management function also supports the release of the bearer context previously established in the gNB-CU-UP. The release of the bearer context is triggered by the gNB-CU-CP either directly or following a request received from the gNB-CU-UP.

This function is used to setup and modify the QoS-flow to DRB mapping configuration. The gNB-CU-CP decides flow-to-DRB mapping and provides the generated SDAP and PDCP configuration to the gNB-CU-UP. The gNB-CU-CP also decides the Reflective QoS flow to DRB mapping. The function is also used to send to the gNB-CU-UP the alternative QoS Parameters Sets when available for a QoS flow. For each PDU Session Resource to be setup or modified, the S-NSSAI, shall be provided in the E1 bearer context setup procedure and may be provided in the E1 bearer context modification procedure by gNB-CU-CP to the gNB-CU-UP.

This function is also used to setup and modify the EPS bearer/E-RAB to DRB mapping configuration for the case of eNB-CP and eNB-UP separation. The eNB-CP decides EPS bearer/E-RAB-to-DRB mapping and provides the E-UTRAN/NR PDCP configuration to the eNB-UP.

This function is also used for the gNB-CU-UP to report the MT-SDT data size to the gNB-CU-CP. With this function, the gNB-CU-UP requests gNB-CU-CP to trigger paging procedure over F1 or Xn to trigger the mobile terminated-small data transmission as described in TS 38.300 [6].

This function is used for the gNB-CU-CP to send the security information to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send to the gNB-CU-UP transport layer information to be used for data forwarding e.g., during handovers.

This function is used for the gNB-CU-CP to send the parameters for header compression for certain traffic types e.g., IP, Ethernet to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send the uplink data compression parameters to the gNB-CU-UP for certain data radio bearer(s).

This function is used for the gNB-CU-UP to notify the event of DL data arrival detection to the gNB-CU-CP. With this function, the gNB-CU-UP requests gNB-CU-CP to trigger paging procedure over F1 or Xn to support RRC Inactive state. RRC Inactive state is not supported when this function is used between an eNB-CP and an eNB-UP.

This function is used for the gNB-CU-UP to notify the gNB-CU-CP that a DL packet including a QFI value not configured by the gNB-CU-CP or an UL packet including a QFI value in the SDAP header of the default DRB not configured by the gNB-CU-CP is received for the first time. The gNB-CU-CP can take further action if needed.

This function is used for the gNB-CU-UP to notify the event of user inactivity to the gNB-CU-CP. With this function, the gNB-CU-UP indicates that the inactivity timer associated with a bearer, a PDU session or a UE expires, or that user data is received for the bearer, the PDU session or the UE whose inactivity timer has expired. The gNB-CU-CP consolidates all the serving gNB-CU-UPs for the UE and takes further action.

This function is used for the gNB-CU-UP to report data volume to the gNB-CU-CP.

This function is used for the gNB-CU-CP to notify the suspension and resumption of bearer contexts to the gNB-CU-UP. Suspension and resumption of bearer contexts are not applicable to eNB-CP/eNB-UP and ng-eNB-CU-CP/ng-eNB-CU-UP.

This function also allows to support CA based packet duplication as described in TS 38.300 [6], i.e. one data radio bearer should be configured with at least two GTP-U tunnels between gNB-CU-UP and a gNB-DU.

This function is used to support the enhanced mobility operations as described in TS 38.300 [6] in the gNB-CU-UP.

===================End of Change=============================