3GPP TSG-RAN WG3 Meeting #120     	R3-232739

Incheon, Korea, May 22-26, 2023

Agenda Item:	16.4

Source:	NEC

Title:	Discussion on the enhancement for multi-path relay

Document for:	Discussion and agreement

1	Introduction

During the previous RAN3 meetings, some details for the support of multi-path relay had been discussed and the following agreements have been made:

From RAN3 perspective, multi-path scenario should be supported in Rel-18. Both intra-DU and inter-DU cases will be supported under the same gNB. RAN3 waits for the RAN2 progress on how to define control plane and user plane scenarios for multi-path support. RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support. Addition of direct/indirect path are supported as follows: - Add direct path, after the establishment of the indirect path. - Add indirect path, after the establishment of the direct path. - This does not imply the exclusion of any other path addition possibility. RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision. The following use cases are not supported in Rel-18. - Configure two indirect paths - More than two paths - Inter-gNB multi-path support For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2. For the multi-path support, the gNB-CU takes the responsibility to decide the addition/modification/release of the path. For intra-DU and inter-DU cases, the UE Context Setup / Modification procedure can be reused to configure the 2nd path with possible enhancements. The details will be discussed based on RAN2 progress. The RAN3 waits for the RAN2 progress on whether the gNB-DU knows the path information of each configured path. WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision. The gNB-CU is responsible to determine the data split among two paths for a DRB for both intra-DU and inter-DU cases. For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB. WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition. Previous RAN3 agreement is updated as follows: For Scenario 1, addition of direct/indirect path are supported as follows: Add direct path, after the establishment of the indirect path. Add indirect path, after the establishment of the direct path. For Scenario 2, addition of direct/indirect path are supported as follows: Add indirect path, after the establishment of the direct path. Whether to add direct path, after the establishment of the indirect path is pending to RAN2 decision. For Scenario 2, interface between UEs are non-3GPP defined. Therefore, in the UE context setup/modification procedure, the PC5 Relay RLC channel configurations are not needed for remote UE and relay UE. RAN3#118: From RAN3 point of view, the Multi-path study phase is completed, and the Multi-path can move to normative work phase. RAN3 #119 Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401. FFS on other procedures. Turn the following WAs into agreement: WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision. WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB. WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition. Agreements: RAN3 further wait for RAN2 progress before removing two Editor’s notes in BL CR to TS 38.401. For intra-DU case, the gNB-CU should inform the gNB-DU about the path to be added, released or modified. The Relay UE L2 ID for indirect path addition and the PCell ID for direct path addition should be provided to the gNB-DU. Whether to inform the gNB-DU of other information needs to be further discussed. The radio bearer type and channels mapping to be added can be provided to the gNB-DU by Rel-17 U2N relay signaling design. Agreements: Add the intra-DU path addition procedure in the BL CR to TS 38.401. Agreements: For Scenario 2, the responsibility for gNB-CU and gNB-DU are defined as follows: - gNB-CU’s responsibility: - Remote UE and relay UE context maintenance - Remote UE bearer mapping - Relaying Uu RLC channel management - gNB-DU’s responsibility: - Determine the RLC/MAC/PHY Configuration for the Uu Relay RLC channels of relay UE - The responsibility for gNB-CU and gNB-DU can be further enhanced based on RAN2 progress. FFS: whether to explicitly or implicitly inform the gNB-CU that the old path should be kept depends on stage 3 design. Agreements: WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

In this contribution, we will continue to address the remaining issues in this topic.

2	Discussion

2.1 The responsibility of gNB-CU and gNB-DU

During RAN2 #120 meeting, it has been agreed that mode 1 resource allocation scheme can be configured towards U2N Remote UE under multi-path scenario, without further details. However, it can observe that both intra-DU and inter-DU cases are supported during RAN3 discussion. During last meeting, one WA has been made for intra-DU case, which is listed in the following:

WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

For intra-DU case, things can be relative simpler for both U2N Remote UE and U2N Relay UE because the only one DU will take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE if it is configured. Thus, based on Rel-17 principle of responsibility of CU/DU, resource scheduling function can be added to the DU of intra-DU case. Therefore, from our observation, there is no blocking issue for this WA, and also it is just a high level principle. Thus RAN3 can turn this WA into agreement during this meeting.

RAN3 turns the following WA into agreement:

- WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

For inter-DU case, it could be deducted that RAN2’s intention is to allow U2N remote UE to connect to DU1 of direct link can be naturally configured with mode 1 resource allocation scheme.

However, for the U2N remote UE connected to DU2 of indirect link, whether it could also be configured with mode 1 resource allocation scheme, is not determined by RAN2 yet, which will impact the discussion of the responsibility of gNB-CU and gNB-DU under multi-path scenario. Since whether the U2N Remote UE can be configured with mode 1 resource allocation scheme will be correlated with whether the corresponding DU needs to support resource scheduling function for U2N Remote UE.

As a conclusion, the responsibility of gNB-CU and gNB-DU for both U2N Relay UE and U2N Remote UE can be referred to the following table:

|  | gNB-CU | gNB-DU1 of direct link | gNB-DU2 of indirect link |
| --- | --- | --- | --- |
| U2N Relay UE | PC5-SRAP configuration Uu-SRAP configuration PDCP configuration RRC signalling procedure Uu measurement report | N.A. | RLC/MAC/PHY layer configuration Resource scheduling |
| U2N Remote UE | PC5-SRAP configuration PDCP configuration RRC signalling procedure Uu measurement report Relay measurement report | RLC/MAC/PHY layer configuration Resource scheduling | RLC/MAC/PHY layer configuration [Resource scheduling?] |

As shown in the above table, whether the DU2 needs to support resource scheduling function is correlated with whether U2N remote UE can be configured with mode 1 resource allocation scheme, which needs to be consulted by RAN2.

The responsibility of gNB-CU and gNB-DU for U2N Relay UE and U2N Remote UE can be separately analyzed under inter-DU scenario.

Whether the DU2 of indirect relay link can have the resource scheduling function for U2N remote UE is correlated with whether the U2N Remote UE can be configured with mode 1 resource allocation scheme under inter-DU case, RAN3 should wait for RAN2 decision at current stage.

3	Conclusion and Proposals

In this contribution, we have discussed the remaining issues which may need potential collaboration with RAN2, and the following observation is given out

Observation 1.	The responsibility of gNB-CU and gNB-DU for U2N Relay UE and U2N Remote UE can be separately analyzed under inter-DU scenario.

and the following proposals are given out:

Proposal1.	RAN3 turns the following WA into agreement:

WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

Proposal2.	Whether the DU2 of indirect relay link can have the resource scheduling function for U2N remote UE is correlated with whether the U2N Remote UE can be configured with mode 1 resource allocation scheme under inter-DU case, RAN3 should wait for RAN2 decision at current stage.

4	References